
	[image: image1.wmf]
	INTERNATIONAL TELECOMMUNICATION UNION

	
	

	ITU-T
	Q.816

	TELECOMMUNICATION
STANDARDIZATION SECTOR
OF ITU
	Amendment 1

(08/2001)  

	
	SERIES Q: SWITCHING AND SIGNALLING

Q3 interface



	
	COBRA-based TMN services

Amendment 1: OMG services profile

	
	ITU‑T Recommendation Q.816 – Amendment 1

(Formerly CCITT Recommendation)


ITU-T Q-SERIES RECOMMENDATIONS

SWITCHING AND SIGNALLING

	
	

	SIGNALLING IN THE INTERNATIONAL MANUAL SERVICE
	Q.1–Q.3

	INTERNATIONAL AUTOMATIC AND SEMI-AUTOMATIC WORKING
	Q.4–Q.59

	FUNCTIONS AND INFORMATION FLOWS FOR SERVICES IN THE ISDN
	Q.60–Q.99

	CLAUSES APPLICABLE TO ITU-T STANDARD SYSTEMS
	Q.100–Q.119

	SPECIFICATIONS OF SIGNALLING SYSTEMS No. 4 AND No. 5
	Q.120–Q.249

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 6
	Q.250–Q.309

	SPECIFICATIONS OF SIGNALLING SYSTEM R1
	Q.310–Q.399

	SPECIFICATIONS OF SIGNALLING SYSTEM R2
	Q.400–Q.499

	DIGITAL EXCHANGES
	Q.500–Q.599

	INTERWORKING OF SIGNALLING SYSTEMS
	Q.600–Q.699

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 7
	Q.700–Q.799

	Q3 INTERFACE
	Q.800–Q.849

	DIGITAL SUBSCRIBER SIGNALLING SYSTEM No. 1
	Q.850–Q.999

	PUBLIC LAND MOBILE NETWORK
	Q.1000–Q.1099

	INTERWORKING WITH SATELLITE MOBILE SYSTEMS
	Q.1100–Q.1199

	SIGNALLING REQUIREMENTS AND PROTOCOLS FOR IMT-2000
	Q.1700–Q.1799

	SPECIFICATIONS OF SIGNALLING RELATED TO BEARER INDEPENDENT CALL CONTROL (BICC)
	Q.1900–Q.1999

	BROADBAND ISDN
	Q.2000–Q.2999

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation Q.816

COBRA-based TMN services

Amendment 1

OMG services profile




	Summary

This Amendment contains corrections to B.1.1 of ITU-T Q.816 (2001) as well as a new Annex C.


	Source

Amendment 1 to ITU‑T Recommendation Q.816 was prepared by ITU‑T Study Group 4 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 13 August 2001.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

( ITU 2002

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS


Page

Annex C – OMG Naming, Notification, and Telecom Log Service profiles

1
C.1
OMG Naming Service

1
C.2
OMG Notification Service

2
C.3
OMG Event Service

6
C.4
OMG Telecom Log Service

7
ITU-T Recommendation Q.816

CORBA-based TMN services
AMENDMENT 1

OMG services profile

1)
Clause 8.1.1, Conformance points

Add the following bullet item to the bullet list under list item 1:
•
support the Naming Service capabilities identified in detail in C.1.

Add the following bullet item to the bullet list under list item 2:

•
If the OMG Notification Service is used, support the Notification Service capabilities identified in detail in C.2.

Also under list item 2, delete the note:
NOTE ( Further study is required to identify a minimum subset of Notification Service capabilities that must be supported for compliance to the framework.

Add the following bullet item to the bullet list under list item 3:
•
support the Telecom Log capabilities identified in detail in C.4.

2)
New Annex C

Add the following new normative annex:

Annex C

OMG Naming, Notification, and Telecom Log Service profiles 

The ITU-T TMN CORBA framework builds upon CORBA by not only using the basic communication capabilities of an ORB, but also by relying upon several of the Common Object Services defined by the OMG. In particular, support for three services is required: the Naming Service, the Notification Service, and the Telecom Log Service.

Only those interface types which need (or optionally need) to be used directly by the managing system to meet the requirements of this profile are presented in this annex. Interface types not discussed in this annex are not directly used by the managing system.

C.1
OMG Naming Service

Table C.1 lists the required capabilities that must be supported on the management interface by the Naming Service. Note that other capabilities may be required internally by the managed system, but only these capabilities need be used by managing systems.

Table C.1/Q.816 ( OMG Naming Service required capabilities

	CORBA object interface
	Object interface element
	Required

	NamingContext
	Operations:


bind


rebind


bind_context


rebind_context


resolve


unbind


new_context


bind_new_context


destroy


list
	X

X


C.2
OMG Notification Service

Table C.2 lists the required capabilities that must be supported on the management interface by the Notification Service. Note that other capabilities may be required internally by the managed system, but only these capabilities shall be used by managing systems. Clause 6.2 specifies that the use of typed notifications is optional, but they are an intended direction. The 'O's in Table C.2 identify the capabilities required to support typed notifications and are currently optional.

	Table C.2/Q.816 ( OMG Notification Service Required Capabilities

	No.
	CORBA object interface
	Object interface element
	Required

	1
	CosNotifyChannelAdmin::

EventChannel
Inherits from

CosNotification::

QoSAdmin
,

CosNotification::

AdminPropertiesAdmin
,

CosEventChannelAdmin::

EventChannel

	Attributes:



MyFactory



default_consumer_admin



default_supplier_admin



default_filter_factory

Operations:



new_for_consumers



new_for_suppliers



get_consumeradmin



get_supplieradmin



get_all_consumeradmins



get_all_supplieradmins
	X

X

X

X

X

	2
	CosNotification::

QoSAdmin
	Operations:



get_qos



set_qos



validate_qos

QoS Values:



EventReliability:




BestEffort




Persistent



ConnectionReliability:




BestEffort




Persistent



OrderPolicy:




AnyOrder




FifoOrder




PriorityOrder




DeadlineOrder



DiscardPolicy:




AnyOrder




FifoOrder




PriorityOrder




DeadlineOrder




LifoOrder




RejectNewEvents
	X

X

X

c.1

c.1

X

X

o.2



	3
	CosNotification::

AdminPropertiesAdmin
	Operations:


get_admin


set_admin
	X

X

	4
	CosNotifyChannelAdmin::

ConsumerAdmin

Inherits from

CosNotification::

QoSAdmin
,

CosNotifyComm::

NotifySubscribe
,

CosNotifyFilter::

FilterAdmin
,

CosEventChannelAdmin::

ConsumerAdmin

	Attributes:


MyID


MyChannel


MyOperator


priority_filter


lifetime_filter


pull_suppliers


push_suppliers

Operations:


get_proxy_supplier


obtain_notification_pull_supplier


obtain_notification_push_supplier


destroy
	X

X

X

X

X

	5
	CosNotifyChannelAdmin::

StructuredProxyPushSupplier

Inherits from

ProxySupplier,

CosNotifyComm::

StructuredPushSupplier

	Operations:


connect_structured_push_consumer


suspend_connection


resume_connection
	X

X

X

	6
	CosNotifyChannelAdmin::

SequenceProxyPushSupplier

Inherits from

ProxySupplier,

CosNotifyComm::

SequencePushSupplier

	Operations:


connect_sequence_push_consumer


suspend_connection


resume_connection
	

	7
	CosNotifyChannelAdmin::

ProxySupplier

Inherits from

CosNotification::QoSAdmin,

CosNotifyFilter::FilterAdmin
	Attributes:


MyType


MyAdmin


priority_filter


lifetime_filter

Operations:


obtain_offered_types


validate_event_qos
	X

X

X

X

	8
	CosNotifyFilter::

FilterAdmin
	Operations:


add_filter


remove_filter


get_filter


get_all_filters


remove_all_filters
	X

X

X

X

X

	9
	CosNotifyFilter::

Filter
	Attributes:

constraint_grammar

Operations:

add_constraints 


modify_constraints 


get_constraints


get_all_constraints


remove_all_constraint


destroy


match 


match_structured 


match_typed 


attach_callback 


detach_callback


get_callbacks
	X

X

X

X

X

X

X

X

X

X

	10
	CosNotifyFilter::

FilterFactory
	Operations:

create_filter


create_mapping_filter
	X

	11
	CosNotifyComm::

NotifySubscribe
	Operations:

subscription_change
	X

	12
	CosNotifyComm::

StructuredPushSupplier

Inherits from

NotifySubscribe
	Operations:


disconnect_structured_push_supplier
	X

	13
	CosNotifyComm::

SequencePushSupplier
Inherits from

NotifySubscribe
	Operations:

disconnect_sequence_push_supplier
	X

	14
	CosTypedNotifyChannelAdmin::

TypedEventChannel

Inherits from

CosNotification::QoSAdmin,
CosNotification::

AdminPropertiesAdmin
,
CosTypedEventChannelAdmin::

TypedEventChannel

	Attributes:


MyFactory


default_consumer_admin


default_supplier_admin


default_filter_factory

Operations:


new_for_typed_notification_consumers


new_for_typed_notification_suppliers 


get_consumeradmin


get_supplieradmin


get_all_consumeradmins


get_all_supplieradmins
	O

O

O

O

O

	15
	CosTypedNotifyChannelAdmin::

TypedConsumerAdmin

Inherits from

CosNotifyChannelAdmin::

ConsumerAdmin
,
CosTypedEventChannelAdmin::

TypedConsumerAdmin

	Operations:

obtain_typed_notification_pull_supplier


obtain_typed_notification_push_supplier
	O

	16
	CosTypedNotifyChannelAdmin::

TypedProxyPushSupplier

Inherits from

CosNotifyChannelAdmin::

ProxySupplier
,
CosNotifyComm::

PushSupplier

	Operations:


connect_typed_push_consumer


suspend_connection


resume_connection
	O

O

O

	17
	CosNotifyComm::

PushSupplier

Inherits from

NotifySubscribe,
CosEventComm::

PushSupplier

	Operations:
	

	c.1 – Either BestEffort or Persistent is to be supported. If only BestEffort is supported, then Q.821 CORBA Enhanced Current Alarm Summary shall also be supported.

o.2 – Priority Order may also be supported. If it is supported, then it should not be selected when Correlated Notifications are used. Also, it should only be selected if message priority is used on a managed object instance basis (see NOTIF 10).


C.3
OMG Event Service

The OMG Notification Service inherits interface capabilities from the OMG Event Service. Table C.3 lists the required capabilities that must be supported on the management interface that are inherited from the Event Service. Note that other capabilities may be required internally by the managed system, but only these capabilities shall be used by managing systems. Clause 6.2 specifies that the use of typed notifications is optional, but they are an intended direction. The 'O's in Table C.3 identify the capabilities required to support typed notifications and are currently optional.

	Table C.3/Q.816 ( OMG Event Service required capabilities

	No.
	CORBA object interface
	Object interface element
	Required

	1
	CosEventChannelAdmin::

EventChannel


	Operations:


for_consumers


for_suppliers


destroy
	

	2
	CosEventChannelAdmin::

ConsumerAdmin

	Operations:


obtain_push_supplier


obtain_pull_supplier
	

	3
	CosTypedEventChannelAdmin::

TypedEventChannel

	Operations:


for_consumers


for_suppliers


destroy
	O



	4
	CosTypedEventChannelAdmin::

TypedConsumerAdmin
Inherits from

CosEventChannelAdmin::

ConsumerAdmin

	Operations:


obtain_typed_pull_supplier


obtain_typed_push_supplier
	

	5
	CosEventComm::

PushSupplier
	Operations:

disconnect_push_supplier
	O

	c.1 – Either BestEffort or Persistent is to be supported. If only BestEffort is supported then Q.821 Corba Enhanced Current Alarm Summary shall also be supported.

o.2 – Priority Order may also be supported. If it is supported, then it should not be selected when Correlated Notifications are used. Also, it should only be selected if message priority is used on a managed object instance basis (see NOTIF 10).


C.4
OMG Telecom Log Service

Table C.4 lists the required capabilities that must be supported on the management interface by the Telecom Log Service. Note that other capabilities may be required internally by the managed system, but only these capabilities shall be used by managing systems.

	Table C.4/Q.816 ( OMG Telecom Log Service required capabilities

	No.
	CORBA Object Interface
	Object Interface Element
	Required

	1
	Log


	Operations:


my_factory


id


get_qos


set_qos


get_max_record_life


set_max_record_life


get_max_size


set_max_size


get_current_size


get_n_records


get_log_full_action


set_log_full_action


get_administrative_state


set_administrative_state


get_forwarding_state


set_forwarding_state


get_operational_state


get_interval


set_interval


get_availability_status


get_capacity_alarm_thresholds


set_capacity_alarm_thresholds


get_week_mask


set_week_mask


query


retrieve


match


delete_records


delete_records_by_id


write_records


write_record_list


set_record_attribute


set_records_attribute


get_record_attribute


copy


copy_with_id


flush
	X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

X

	2
	EventLog
Inherits from

Log,
CosEventChannelAdmin::

EventChannel

	Operations:


destroy
	

	3
	NotifyLog

Inherits from

EventLog,
CosNotifyChannelAdmin::

EventChannel

	Operations:


get_filter


set_filter
	X

X

	4
	TypedEventLog

Inherits from

Log,
CosTypedEventChannelAdmin::

TypedEventChannel

	Operations:


typed_query


typed_retrieve
	O

O

	5
	TypedNotifyLog

Inherits from

TypedEventLog,
CosTypedNotifyChannelAdmin::

TypedEventChannel

	Operations:


get_filter


set_filter
	O

O


	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	


