
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	Q.816

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Amendment 2

(05/2002)   

	
	SERIES Q: SWITCHING AND SIGNALLING

Q3 interface



	
	CORBA-based TMN services

Amendment 2: User guide for local name resolution



	
	ITU‑T Recommendation Q.816 (2001)  –  Amendment 2




ITU-T Q-SERIES  RECOMMENDATIONS

SWITCHING AND SIGNALLING

	
	

	SIGNALLING IN THE INTERNATIONAL MANUAL SERVICE
	Q.1–Q.3

	INTERNATIONAL AUTOMATIC AND SEMI-AUTOMATIC WORKING
	Q.4–Q.59

	FUNCTIONS AND INFORMATION FLOWS FOR SERVICES IN THE ISDN
	Q.60–Q.99

	CLAUSES APPLICABLE TO ITU-T STANDARD SYSTEMS
	Q.100–Q.119

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 4
	Q.120–Q.139

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 5
	Q.140–Q.199

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 6
	Q.250–Q.309

	SPECIFICATIONS OF SIGNALLING SYSTEM R1
	Q.310–Q.399

	SPECIFICATIONS OF SIGNALLING SYSTEM R2
	Q.400–Q.499

	DIGITAL EXCHANGES
	Q.500–Q.599

	INTERWORKING OF SIGNALLING SYSTEMS
	Q.600–Q.699

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 7
	Q.700–Q.799

	Q3 INTERFACE
	Q.800–Q.849

	DIGITAL SUBSCRIBER SIGNALLING SYSTEM No. 1
	Q.850–Q.999

	PUBLIC LAND MOBILE NETWORK
	Q.1000–Q.1099

	INTERWORKING WITH SATELLITE MOBILE SYSTEMS
	Q.1100–Q.1199

	INTELLIGENT NETWORK
	Q.1200–Q.1699

	SIGNALLING REQUIREMENTS AND PROTOCOLS FOR IMT-2000
	Q.1700–Q.1799

	SPECIFICATIONS OF SIGNALLING RELATED TO BEARER INDEPENDENT CALL CONTROL (BICC)
	Q.1900–Q.1999

	BROADBAND ISDN
	Q.2000–Q.2999

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation Q.816

CORBA-based TMN services

Amendment 2

User Guide for local name resolution



	Summary

This amendment adds a new non-normative appendix.


	Source

Amendment 2 to ITU‑T Recommendation Q.816 (2001) was prepared by ITU‑T Study Group 4 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 29 May 2002.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS


Page
Amendment 2 ( User Guide for local name resolution

1
1)
Appendix IV

1
ITU-T Recommendation Q.816

CORBA-based TMN services

Amendment 2

User Guide for local name resolution
1)
Appendix IV

Add the following new non-normative appendix:
Appendix IV

User Guide for local name resolution

This appendix provides additional information about the TMN CORBA framework intended to help those implementing systems that conform to the framework recommendations.

Requirement Name-6 specifies that a managed system must provide a procedure for assigning a name to each local root naming context. Since each managed object on a managed system must have a unique name, by assigning a globally unique name to the local root naming context, an administration can ensure that each managed object will have a globally unique name. Because these globally unique names may then be used as attribute values to represent associations between objects, there may at times be a need for a managed system to resolve the globally unique name of a local object. This can be eased by constructing on the managed system a naming context hierarchy represented by the globally unique name assigned to the local root naming context. See Figure 1 for a graphical representation.

[image: image2.wmf]T0417280-02

A

Object

Managed System

Managing System

A

Local resolutions 

start here

Element1:

Manag

edElement

Local Root


Figure 1/Q.816 – Local name resolution

In Figure 1, the managed system provides the local root naming context represented by the folder labelled "Local Root". This is referenced by the managing system. When the globally unique name "A" is assigned to the local root naming context, the managing system may create a naming context for its own local name resolutions, and bind the local root to the name "A" in that context. This naming context is represented in the figure by the top-most folder in the managed system box. Then, whenever the managed system needs to resolve a globally unique name for a local object (a name beginning with "A"), it can do so by starting the resolution on this naming context. Note that, had the globally unique name assigned to the local root naming context had multiple components, the managed system would have to create multiple naming contexts to perform the name resolution. So, for example, had the name "A.B/C.D" been assigned to the local root, the top-most naming context would have had the name "A.B" bound to a second naming context, which then would have had the name "C.D" bound to the local root.

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	








_1094455762.unknown

