��

INTERNATIONAL  TELECOMMUNICATION  UNION��





ITU-T�Q.825��TELECOMMUNICATION�STANDARDIZATION  SECTOR�OF  ITU�(06/98)   ��









SERIES Q: SWITCHING AND SIGNALLING

Specifications of Signalling System No. 7 – Q3 interface





Specification of TMN applications at the Q3 interface: Call detail recording



ITU-T  Recommendation  Q.825

(Previously  CCITT  Recommendation)



�ITU-T  Q-SERIES  RECOMMENDATIONS

SWITCHING AND SIGNALLING

���SIGNALLING IN THE INTERNATIONAL MANUAL SERVICE�Q.1–Q.3��INTERNATIONAL AUTOMATIC AND SEMI-AUTOMATIC WORKING�Q.4–Q.59��FUNCTIONS AND INFORMATION FLOWS FOR SERVICES IN THE ISDN�Q.60–Q.99��CLAUSES APPLICABLE TO ITU-T STANDARD SYSTEMS�Q.100–Q.119��SPECIFICATIONS OF SIGNALLING SYSTEMS No. 4 AND No. 5�Q.120–Q.249��SPECIFICATIONS OF SIGNALLING SYSTEM No. 6�Q.250–Q.309��SPECIFICATIONS OF SIGNALLING SYSTEM R1�Q.310–Q.399��SPECIFICATIONS OF SIGNALLING SYSTEM R2�Q.400–Q.499��DIGITAL EXCHANGES�Q.500–Q.599��INTERWORKING OF SIGNALLING SYSTEMS�Q.600–Q.699��SPECIFICATIONS OF SIGNALLING SYSTEM No. 7�Q.700–Q.849��General�Q.700��Message transfer part (MTP)�Q.701–Q.709��Signalling connection control part (SCCP)�Q.711–Q.719��Telephone user part (TUP)�Q.720–Q.729��ISDN supplementary services�Q.730–Q.739��Data user part�Q.740–Q.749��Signalling System No. 7 management�Q.750–Q.759��ISDN user part�Q.760–Q.769��Transaction capabilities application part�Q.770–Q.779��Test specification�Q.780–Q.799��Q3 interface�Q.800–Q.849��DIGITAL SUBSCRIBER SIGNALLING SYSTEM No. 1�Q.850–Q.999��General�Q.850–Q.919��Data link layer�Q.920–Q.929��Network layer�Q.930–Q.939��User-network management�Q.940–Q.949��Stage 3 description for supplementary services using DSS 1�Q.950–Q.999��PUBLIC LAND MOBILE NETWORK�Q.1000–Q.1099��INTERWORKING WITH SATELLITE MOBILE SYSTEMS�Q.1100–Q.1199��INTELLIGENT NETWORK�Q.1200–Q.1999��BROADBAND ISDN�Q.2000–Q.2999�����For further details, please refer to ITU-T List of Recommendations.

�ITU-T  RECOMMENDATION  Q.825



specification of tmn applications at the q3 interface:�call detail recording��



Summary

This Recommendation specifies the management functionality and the management information model required to support the management aspects of the "call detail recording" function.��



Source

ITU-T Recommendation Q.825 was prepared by ITU-T Study Group 4 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 26th of June 1998.��



��

�FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU�T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU�T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.











NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.







INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.











ã  ITU  1998

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

�CONTENTS

	Page

1	Scope		1

2	Normative references		2

3	Definitions, symbols and abbreviations		3

3.1	Definitions		3

3.2	Symbols		4

3.3	Abbreviations		4

4	General requirements		5

5	TMN management services		6

5.1	Subscriber billing		7

5.2	Account settlement management		7

5.3	Service provision		7

5.4	Customer administration		8

5.5	Analysis application		8

6	TMN management service components		8

6.1	CDR generation control		9

6.2	CDR transfer control		9

7	Conceptual model		9

8	Management functions and Services		12

8.1	Call detail recording control function		12

8.1.1	Initiate call detail recording		12

8.1.2	Terminate call detail recording for usage		13

8.1.3	Terminate call detail recording for analysis		13

8.1.4	Get call detail control data		13

8.1.5	Modify call detail control data		14

8.2	Call detail data function		14

8.3	CDR transfer		14

8.3.1	Real-time CDR reporting function		14

8.3.2	Near Real-time CDR reporting function		16

8.3.3	CDR transfer via file Generating Log		18

9	Functional units		21

9.1	Functional Units (FUs)		23

9.2	Functional units from other Recommendations		23

9.3	Negotiation of functional units		23

�	Page

10	Conformance		24

10.1	Static conformance		24

10.2	Dynamic conformance		24

Annex A – Information model		24

A.1	Overview		24

A.2	Naming hierarchy		25

A.3	Inheritance		26

A.4	Managed object classes		27

A.4.1	Block Generating Log		27

A.4.2	Call detail data		27

A.4.3	Call Detail Log Record		28

A.4.4	Configurable Simple Usage Metering Control Object		30

A.4.5	File Generating Log		31

A.4.6	Simple Usage Metering Control Object		31

A.5	Packages		32

A.5.1	Access Delivery Package		32

A.5.2	Account Code Input Package		32

A.5.3	b-Party Category Package		32

A.5.4	Bearer Service Package		32

A.5.5	CDR Purpose Package		32

A.5.6	Call Status Package		32

A.5.7	Called Party Number Package		33

A.5.8	Calling Party Category Package		33

A.5.9	Calling Party Number Not Screened Package		33

A.5.10	Calling Party Number Package		33

A.5.11	Calling Party Type Package		33

A.5.12	Carrier Id Package		33

A.5.13	Cause Package		33

A.5.14	Charged Directory Number Package		33

A.5.15	Charged Participant Package		34

A.5.16	Charging Information Package		34

A.5.17	Conversation Time Package		34

A.5.18	DPC Package		34

A.5.19	Daily Triggering Package		34

A.5.20	Data Validity Package		34

A.5.21	Duration Time  ACM Package		34

A.5.22	Duration Time B-answer Package		34

�	Page

A.5.23	Duration Time No B-answer Package		35

A.5.24	Exchange Info Package		35

A.5.25	Fallback Bearer Service Package		35

A.5.26	File Creation Notification Package		35

A.5.27	Glare Package		35

A.5.28	IN Package		35

A.5.29	ISUPPreferred Package		35

A.5.30	Immediate Notification for Usage MeteringPackage		35

A.5.31	Network Management Controls Package		36

A.5.32	Network Provider Id Package		36

A.5.33	OPC Package		36

A.5.34	Operator Specific1 Additional Number Package		36

A.5.35	Operator Specific1 Number Package		36

A.5.36	Operator Specific2 Additional Number Package		36

A.5.37	Operator Specific2 Number Package		36

A.5.38	Operator Specific3 Additional Number Package		36

A.5.39	Operator Specific3 Number Package		37

A.5.40	Original Called Number Package		37

A.5.41	Partial Generation Package		37

A.5.42	Percentage To Be Billed Package		37

A.5.43	Periodic Triggering Package		37

A.5.44	Personal User Id Package		37

A.5.45	Physical Line Code Package		37

A.5.46	Progress Package		37

A.5.47	Queue Info Package		38

A.5.48	Received Digits Package		38

A.5.49	Record Extensions Package		38

A.5.50	Record Id Package		38

A.5.51	Redirecting Number Package		38

A.5.52	Redirection Number Package		38

A.5.53	Related Call Number Package		38

A.5.54	Service specific IN information Package		38

A.5.55	Standard Extensions Package		39

A.5.56	Supplementary Service Package		39

A.5.57	Teleservice Package		39

A.5.58	Trunk Group Incoming Package		39

A.5.59	Trunk Group Outgoing Package		39

A.5.60	User-to-User Info Package		39

�	Page

A.6	Attributes		39

A.6.1	Access Delivery		39

A.6.2	Account Code Input		40

A.6.3	Additional Participant Info		40

A.6.4	B-party Category		40

A.6.5	Bearer Service		40

A.6.6	CDR Purpose		40

A.6.7	Call Detail Data Identifier		40

A.6.8	Call Duration		41

A.6.9	Call Identification Number		41

A.6.10	Call Status		41

A.6.11	Called Party Number		41

A.6.12	Calling Party Category		41

A.6.13	Calling Party Number		42

A.6.14	Calling Party Number Not Screened		42

A.6.15	Calling Party Type		42

A.6.16	Carrier Id		42

A.6.17	Cause		43

A.6.18	Charged Directory Number		43

A.6.19	Charged Participant		43

A.6.20	Charging Information		43

A.6.21	Configuration Mask		44

A.6.22	Conversation Time		44

A.6.23	Creation Trigger List		44

A.6.24	DPC		44

A.6.25	Data Validity		44

A.6.26	Duration Time Until ACM		45

A.6.27	Duration Time B-Answer		45

A.6.28	Duration Time No B-Answer		45

A.6.29	Exchange Info		45

A.6.30	Fallback Bearer Service		45

A.6.31	Glare		46

A.6.32	IN Service Information List		46

A.6.33	IN Specific Information		46

A.6.34	ISUP Preferred		46

A.6.35	Immediate Notification for Usage Metering		46

A.6.36	Max Block Size		47

A.6.37	Max Time Interval		47

A.6.38	Network Management Controls		47

�	Page

A.6.39	Network Provider ID		47

A.6.40	OPC		47

A.6.41	Operator Specific1 Additional Number		48

A.6.42	Operator Specific1 Number		48

A.6.43	Operator Specific2 Additional Number		48

A.6.44	Operator Specific2 Number		48

A.6.45	Operator Specific3 Additional Number		48

A.6.46	Operator Specific3 Number		49

A.6.47	Original Called Number		49

A.6.48	Partial Generation		49

A.6.49	Participant Info		49

A.6.50	Percentage To Be Billed		49

A.6.51	Periodic Trigger		50

A.6.52	Personal User Identification		50

A.6.53	Physical Line Code		50

A.6.54	Progress		50

A.6.55	Queue Info		50

A.6.56	Received Digits		51

A.6.57	Record Extensions		51

A.6.58	Record Id		51

A.6.59	Record Type		51

A.6.60	Redirecting Number		52

A.6.61	Redirection Number		52

A.6.62	Related Call Number		52

A.6.63	Sampling Rate		52

A.6.64	Service Specific IN Information		52

A.6.65	Service User		53

A.6.66	Standard Extensions		53

A.6.67	Start Time Stamp		53

A.6.68	Supplementary services		53

A.6.69	Teleservice		54

A.6.70	Times of Day		54

A.6.71	Trunk Group Incoming		54

A.6.72	Trunk Group Outgoing		55

A.6.73	User-to-User Info		55

A.7	Actions		55

A.7.1	Create File		55

�	Page

A.8	Notifications		55

A.8.1	Block Record Notification		55

A.8.2	Call Detail Record Notification		56

A.8.3	File Creation Notification		57

A.9	Name bindings		57

A.10	ASN.1 defined types module		58

Annex B – Call Detail  Records		69

B.1	General		69

B.1.1	Use of record types		69

B.1.2	Partial records		70

B.1.3	Use of supplementary services		70

B.2	Record contents		70

Appendix I – Alternative ASN.1 definitions		72

I.1	Definition of ASN.1 modules for use of 1994 version of ASN.1		72

I.2	Rules of extensibility		74

I.3	Use of Management Extension		74

Appendix II – Application of the model		75

Appendix III – Examples of the use of Call Detail Records		76

Appendix IV – Application of CDR (for information only)		77



�Recommendation Q.825

specification of tmn applications at the q3 interface:�call detail recording

(Geneva, 1998)

1	Scope

This Recommendation specifies the management functionality and the management information model required to support the management aspects of the "Call Detail Recording" function. 

The model  applies to the Operations System to Network Element (OS/NE) interface.

The scope of this Recommendation is to support call detail recording for the following services and technologies:

•	data collection requirements for the analogue, digital and Integrated Services Digital Network (ISDN) subscribers;

•	data collection requirements in connection with Intelligent Networks (IN).

Call detail information may be gathered for two primary purposes:

•	Accounting Management: This includes the collection of usage data for calls and call attempts that consume accountable resources. This data are then used for bill rendering and may also be used for market and customer behaviour analysis. Only data collection, data transfer and the control of these processes is in the scope of this Recommendation.

•	Analysis Administration: This includes the collection of usage data for calls and call attempts for Quality of Service (QoS), Fraud Investigation or Network Traffic Management (NTM) applications as defined below:

–	Quality of Service: This includes the collection of usage data for calls and call attempts that utilize scarce resources that impact the quality of service perceived by the subscriber, regardless of whether the resource is subject to billing. For this purpose, data may be collected on a sampled basis and information for both successful and failed call attempts is required. 

–	Fraud investigation: This includes collection of data on successful and denied service requests in order to determine irregularities in calling patterns to allow both real-time and off-line detection or reconstruction of fraudulent use of network resources.

–	Network traffic management: This includes collection of usage data calls and call attempts and their holding times to allow traffic studies for the purpose of network administration and capacity planning.

The data collection needs for all the above functions are supported by a single record type referred to as a Call Detail Record (CDR). The content of the CDR notification to be sent by the NE can be configured to support the data needs of any of the above applications. In the context of billing and accounting, CDRs will occasionally, for purposes of clarity, be referred to as Usage Metering Records (UMRs).

This Recommendation also defines a set of data elements needed to provide a complete usage information record to be utilized for charging and itemized billing and accounting. Collected data will be formatted and sent to the specialized centre.

�NOTE – The modelling of the charging and the billing processes is outside the scope of this Recommendation. Nevertheless the call detail record defined in this Recommendation may also contain the charge information if the calculation of charge is performed by the NE itself. This calculation may be performed by the NE autonomously or based on information provided by an SCP in an IN environment.

2	Normative references

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

–	ITU-T Recommendation M.1400 (1997), Designations for international networks.

–	ITU-T Recommendation M.3100 (1995), Generic network information model.

–	ITU-T Recommendation M.3200 (1997), TMN management service and telecommuni�cations managed areas: Overview.

–	ITU-T Recommendation Q.811 (1997), Lower layer protocol profiles for the Q3 and X interfaces.

–	ITU-T Recommendation Q.812 (1997), Upper layer protocol profiles for the Q3 and X interfaces.

–	ITU-T Recommendation Q.850 (1998), Usage of cause and location in the Digital subscriber Signalling System No. 1 and the Signalling System No. 7 ISDN user part.

–	CCITT Recommendation X.209 (1988), Specification of basic encoding rules for Abstract Syntax Notation One (ASN.1).

–	ITU-T Recommendation X.680 (1997) | ISO/IEC 8824-1:1998, Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation.

–	ITU-T Recommendation X.681 (1997) | ISO/IEC 8824-2-2:1998, Information technology – Abstract Syntax Notation One (ASN.1): Information object specification.

–	ITU-T Recommendation X.682 (1997) | ISO/IEC 8824-3:1998, Information technology – Abstract Syntax Notation One (ASN.1): Constraint specification.

–	ITU-T Recommendation X.683 (1997) | ISO/IEC 8824-4:1998, Information technology – Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 specifications.

–	ITU-T Recommendation X.701 (1997) | ISO/IEC 10040:1998, Information technology – Open Systems Interconnection – Systems management overview.

–	ITU-T Recommendation X.710 (1997) | ISO/IEC 9595:1998, Information technology – Open Systems Interconnection – Common Management Information Service.

–	ITU-T Recommendation X.711 (1997) | ISO/IEC 9596-1:1998, Information technology – Open Systems Interconnection – Common Management Information Protocol: Specification.

–	CCITT Recommendation X.720 (1992) | ISO/IEC 10165-1:1993, Information technology –Open Systems Interconnection – Structure of management information: Management information model.

�–	CCITT Recommendation X.721 (1992) | ISO/IEC 10165-2:1992, Information technology –Open Systems Interconnection – Structure of management information: Definition of management information.

–	CCITT Recommendation X.722 (1992) | ISO/IEC 10165-4:1992, Information technology –Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed objects.

–	CCITT Recommendation X.730 (1992) | ISO/IEC 10164-1:1993, Information technology – Open Systems Interconnection – Systems management: Object management function.

–	CCITT Recommendation X.734 (1992) | ISO/IEC 10164-5:1993, Information technology –Open Systems Interconnection – Systems management: Event report management function.

–	CCITT Recommendation X.735 (1992) | ISO/IEC 10164-6:1993, Information technology –Open Systems Interconnection – Systems management: Log control function.

–	ITU-T Recommendation X.742 (1995) | ISO/IEC 10164-10:1995, Information technology –Open Systems Interconnection – Systems management – Usage metering function for accounting purposes.

–	ITU-T Recommendation X.746 (1995) | ISO/IEC 10164-15:1995, Information technology –Open Systems Interconnection – Systems management: Scheduling function.

3	Definitions, symbols and abbreviations 

NOTE – When an event is described in terms of an ISUP message, it should be understood that if ISUP is not used an equivalent message or event should be used.

3.1	Definitions

This Recommendation defines the following terms.

3.1.1	accounting: Set of functions required for Usage Metering, Charging and Billing.

3.1.2	answered call: A call is considered an answered call when ANM or its equivalent is received.

3.1.3	call detail data: Data collected about significant events of a call or call attempt.

3.1.4	call detail record: A data record containing call detail information relating to a specific call or call attempt instance.

3.1.5	counter: A register whose value is increased in real time for each unit to be accounted for.

3.1.6	billing: Administrative function to prepare bills to service customers, to prompt payments, to obtain revenues and to take care of customer reclaims.

3.1.7	charging administration: Management function to manage charging functionality, to collect variable data out of NEs and provide information for the billing, accounting and service provisioning.

3.1.8	charging: The set of functions needed to determine the price assigned to the service utilization.

3.1.9	call failures: A call fails when some network conditions do not allow the connection between calling and called parties to be set up or properly maintained. See Recommendation Q.850 for the causes.

�3.1.10	service subscriber: It is the legal entity, which has subscribed to a certain service type. It is not necessarily a different party from the service user. (The calling and called parties of a service transaction are service users.)

3.1.11	tariff administration: Management function to set and manipulate tariff.

3.1.12	tariff: A set of data used for the determination of the utilization charges for the used services.

3.1.13	usage metering: The abstraction of activities that monitor the utilization of resources, for the purpose of accounting and controlling the recording of usage data.

3.1.14	usage metering data: Data which represent usage from which usage metering records may be derived.

3.1.15	usage metering record: A data item containing usage information relating to a specific period of resource utilization by a specific user.

3.1.16	usage: Quantification of the utilization of a resource from which information may be derived for the purpose of accounting.

3.1.17	user: An identifiable entity whose use of resources must be accounted.

3.2	Symbols

3.3	Abbreviations 

This Recommendation uses the following abbreviations.

ACM	Address Complete Message

ANM	Answer Message

AOC	Advice of Charge

ASN.1	Abstract Syntax Notation One

CCO	Configurable Simple Usage Metering Control Object

CDD	Call Detail Data Object

CDR	Call Detail Record

CEP 	Circuit End Point

CEPSG	Circuit End Point Sub-group (as defined in Recommendation M.3100)

CLIP	Calling Line Identification Presentation

DDI	Direct-Dialling-In

DN	Directory Number

DPC	Destination Point Code

EFD	Event Forwarding Discriminator

ER	Entity-Relationship

GDMO	Generic Definition for Managed Object Class

IN	Intelligent Network

ISDN	Integrated Services Digital Network

MF	Management Function

�MML	Man Machine Language

MSC	Management Service Component

MSN	Multiple Subscriber Number

NE	Network Element

NEF	Network Element Function block

NPI	Numbering Plan Identification

NTM	Network Traffic Management

OPC	Originating Point Code

OS	Operations System

OSF	Operations System Function

OSI	Open System Interface

QMR	Quality Metering Record

QoS	Quality of Service

PSTN	Public Switched Telephone Network

SCO	Simple Usage Metering Control Object

SCP	Service Control Point

SMF	Service Management Function

TMN	Telecommunications Management Network

TON	Type of Number

UMR	Usage Metering Record

UPT	Universal Personal Telecommunication

VAS	Value-Added Service

VPN	Virtual Private Network

In tables, the following abbreviations will be used:

M = Mandatory, P = Pass-through, U = User Option, C = Conditional

4	General requirements

The call detail recording function shall fulfil the requirements for the measurement of resource utilization, so that the data gathered may be used for the process of accounting management, the generation of bills, and analysis such as QoS, fraud detection, and network traffic management. The following requirements should be met:

–	The function must allow sufficient control over the collection of call detail data so that the relevant information can be made available when required. A management system should have the capability to access individual call detail records almost instantaneously. This feature is required, for example, for the support of real-time (or near real time) cost calculation as, for example, in hotel applications.

–	A management system shall have a standardized way of obtaining and representing call detail information to advise subscribers of their usage and to facilitate exchange of call detail information with other suppliers.

�–	The call detail record shall be self-contained, i.e. the interpretation of the record shall not be dependent on the system where the call detail record was created.

–	Several resources may be utilized to provide a service. The function should make it possible to relate call detail-records to the resources that are actually used or were required (but unavailable) for providing the requested service.

–	For quality of service determination and network management (traffic studies), the application need not collect data on all calls and call attempts but the analysis needs may be met by sampling. The function shall make it possible to specify a flexible sampling rate.

–	The function should support a number of trigger conditions for reporting of a call detail record. 

	Conditions that will cause the creation of a call detail record for measurement of usage and billing are:

•	termination of a service;

•	change of service, e.g. due to change of charging conditions;

•	reaching a volume threshold;

•	at regular intervals during a practical service transaction;

•	supplementary service input.

	Additional conditions that cause creation of a call detail record for analysis purposes such as quality of service measurement, fraud detection or traffic management are:

•	denial of a service;

•	change of observed service quality;

•	reaching a quality threshold;

•	call failures.

5	TMN management services

The TMN Management Service, "Tariff and Charging Administration", covers those management activities related to the charging of service usage including both the data collection process and the administration of charging data within the network elements. The relationship between this and other management services and activities is illustrated in Figure 1.

�� EMBED Designer.Drawing.7  ���

Figure 1/Q.825 – The tariff and charging administration

5.1	Subscriber billing

The call detail data collected from the network elements is employed to determine the network resources utilization charges for the basic and supplementary services utilized by the subscriber. The charges calculated may be further processed (e.g. to apply any special discounts) and then combined with the network access (subscription) charges and billed to the customer.

5.2	Account settlement management

The settlement of accounts with the operators of other networks for traffic carried is generally performed on a bulk basis. Accounting information may also be used for settlement of accounts with services provided by service centres and other Value-Added Service (VAS) providers. The charges for the various traffic shares may be determined on the basis of the call records generated by the network elements or on the basis of bulk counters (accounting metre records) in the gateway exchanges. For the purpose of this Recommendation, the management information required is assumed to be derived from call detail records. The management of bulk meters is outside the scope of this Recommendation.

5.3	Service provision

The call detail data collected from the network elements may be used to provide statistical information concerning the use of services within the network. In addition, the introduction of new services and/or modifications to the tariffs of existing services may also require the distribution of the appropriate tariff information to the network elements for Advice of Charge (AOC) purposes. The management of tariff information is outside the scope of this Recommendation.

�5.4	Customer administration

The call data collected from the NEs provide a historic record of subscriber activity and may be used for the handling of customer care inquiries such as billing complaints, statistic analysis, customer behaviour, detection of abnormal use, etc.

5.5	Analysis application

In addition, this Recommendation supports other applications requiring access to call detail records as shown in Figure 2 below. It should be noted that these applications are not intended to be limited and Administrations may use this data for any suitable purpose. 

� EMBED Designer.Drawing.7  ���

Figure 2/Q.825 – Supported call detail data analysis applications

6	TMN management service components

The TMN management Service "Tariff and charging administration" contains two TMN Management Service Components (TMN-MSC):

1)	Tariff Administration.

2)	Call detail Data Collection Management.

Component 1, Tariff Administration, is outside the scope of this Recommendation. Component 2, Call Detail Data Collection Management, is relevant for the billing and usage functionality specified in this Recommendation along with Data Collection for Analysis purposes. It includes the specification of the Call Detail Record (CDR) to be generated and collected, as well as the mechanisms required for the transfer of these CDRs to the OS. 

In Figure 3, the functionality of the data collection management is illustrated.

�� EMBED Designer.Drawing.7  ���

Figure 3/Q.825 – Management service components

6.1	CDR generation control

The generation, intermediate storage and transmission of CDRs consume considerable amounts of network and TMN resources. The CDR generation control permits the OS to configure and optimise both the generation of records within the NEs and the contents of those records according to the needs of the network operator.

6.2	CDR transfer control 

The CDRs produced by the NEs must be transmitted to, or collected by, the appropriate OS for subsequent processing. 

The CDR transfer control provides mechanisms for the transfer of CDRs, both individually (real�time event reporting) and in bulk (by file transfer), between the NEs and OSs.

7	Conceptual model

The conceptual model as described below is depicted in Figure 4.

The components of the model are designed to be useful for billing, QoS monitoring, fraud investigation, and Network Traffic Management (NTM). 

CDR generation

The model consists of simpleUsageMeteringControl object class and configurableSimpleUsageMeteringControl Object class to control the collection of CDRs. The simpleUsageMeteringControl object class is used for usage metering purposes only; whereas the �configurableSimpleUsageMeteringControl object class is used for Analysis purposes. This allows generation of a single set of records/notifications that can be used for both billing and analysis functions. Both object classes define the triggers that will create CDR Data Objects, which generate the CDR notifications. Both object classes define the triggers that will create callDetailData objects. A single set of callDetailData objects (no more than one callDetailData per call and per associated accountable object) are generated. Each callDetailData in turn emits CDR notifications whose contents are controlled by both control objects.

CDR transfer

There are three ways of transferring the CDR notifications (records) to the OS:

1)	Real-time Transfer: A single CDR record can be transferred in real-time by filtering with an EFD for immediate notification.

2)	Near Real-time Transfer: Blocks of CDRs can be by grouping CDR notifications and storing these notifications first in the Blocked Generation Log. Subsequently, blockRecordNotification is sent out via EFD.

3)	Bulk Transfer: CDR notifications are stored in the File Generating Log. Upon OS request, they are transferred to a file for subsequent transfer of the file using a file transfer protocol.

The volume and nature of CDRs available to the manager can be managed through appropriate usage of filtering criteria (i.e. the discriminating construct) in the EFD or log.

This Recommendation is a specialization of Recommendation X.742; it defines the following object classes to control and collect the CDR:

–	Simple Usage Metering Control Object and Configurable Simple Usage Metering Control Object: used to control the collection of CDR for accountable objects;

–	Call Detail Data that contains the collected information.

The accountable object may be any resource (logical or physical) for which usage is to be measured. Examples of such resources could be: Directory Numbers (DNs), incoming or outgoing trunks and trunk groups (e.g. CEP or CEPSG), etc. callDetailData object is used for collecting usage data from the accountable object associated with the control objects triggers. Notifications containing the measured data will be emitted by the callDetailData object, and may be stored in the local log thus forming the call detail record, or may be transmitted to a remote OS as specified in an event forwarding discriminator. In addition, for efficient transmission, the individual notifications may be grouped into blocks for near real-time usage data reporting. This grouping is accomplished by temporarily storing the CDRs in a blockGeneratingLog and then generating a new notification from that log at a later time.

callDetailData objects are created and deleted implicitly, that is, they are created and deleted upon the occurrence of defined trigger events and are not explicitly manipulated by a managing system; i.e. a manager cannot perform any operations on instances of this object class. To support recording data in a callDetailData object, two sets of triggers coming from the control objects are defined:

–	Creation Triggers: events that cause creation of a callDetailData object. These triggers are defined as part of the control object.

–	Termination Triggers: events that cause deletion of a callDetailData object. Currently the termination is implicit as part of the data object behaviour. Deletion occurs upon completion of usage data collection for that instance of service.

CDR notifications are emitted in response to reporting triggers (Recommendation X.742) that are also defined as part of the control objects.

�This Recommendation defines the simpleUsageMeteringControl object class that allows the definition of different types of triggers for usage metering recording. One of the defined reporting triggers requires periodic reporting and can be used to transfer partial service usage data to a log or remote OS for long duration service usage. Another type of trigger is determined by the occurrence of a particular event during usage of the service, e.g. completion of usage. This object class fully supports the requirements for usage metering. If only Usage Metering application is required, then simpleUsageMeteringControl object class shall be used.

If Analysis function is required, then configurableSimpleUsageMeteringControl shall be used. The configurableSimpleUsageMeteringControl object allows sampling rate and allows configuration of all the optional parameters, as defined by configurationMask, to be included in the Call Detail Record Notification. The object class controls the creation of callDetailData objects based on the configurableSimpleUsageMeteringControl creationTriggerList, and the samplingRate. The configurationMask will control the additional data to be included in the CDR notification emitted by the CDR for QoS, Fraud and Network Traffic Management purpose.

When both usage and analysis functions are needed, both configurableSimpleUsageMeteringControl and simpleUsageMeteringControl shall be used. The callDetailData is generated by the earliest event in the creationTriggerList of either object. The sampling rate applies to the rate at which the callDetailData is generated based on the creationTriggerList from the simpleUsageMeteringControl, and enriched by the analysis information as defined by the configurationMask.

� EMBED Designer.Drawing.7  ���

Figure 4/Q.825 – Data collection model

Figure 4 shows how CDRs may be emitted in real-time (use of single CDRs), or near real-time (use of block CDRs) or may be transferred by a file (file transfer). In the case of file transfer, the 'file status information' is an event report indicating that a file has been prepared for transfer.

�8	Management functions and Services

This Recommendation defines three new functions. These functions make use of call detail record services defined in Recommendations X.730 (Object Management Function), X.734 (Event Reporting Management Function) and X.735 (Log Control Function).

8.1	Call detail recording control function

This function controls the generation and reporting of call detail records and allows specification of the events and resources whose usage is to be recorded. Only one instance of this function is present for each accountable object.

The following control functions are available:

1)	Recording control: Recording controls allow specification of the event that, if it occurs, will cause creation of a call detail data object which will generate call detail records. This control function enables the reduction of records collected in a NE. The record generation can be triggered to make a record for events based on several types of criteria such as off�hook (seizure), first digit dialled, ACM received, B-answer (ANM received), supplementary service invocation and supplementary service input. These criteria may be described in the creationTriggers attributes of the control object. If a potentially recordable call (i.e. a call associated with an identified accountable resource) matches the criteria specified in the creationTriggers attribute, call detail data object will be created resulting in the generation of one or more call detail records. If the creationTriggers list is empty, no CDRs will be generated.

	For applications that do not require all instances of usage to be recorded, a sampling rate may be specified.

2)	Reporting control: This control allows specification of the conditions under which a CDR notification will be emitted by the call detail data object. The triggers may be events occurring during the life of the service or based on elapsed time since the last notification or start of service. This function covers the need for specification of a partial record interval timer for long hold calls. The timer may take any value within the range of 10 minutes to 24 hours. A value 0 (no reporting trigger) means that no partial records will be generated.

To manage the call detail recording control, the operations required are:

–	initiate call detail recording;

–	terminate call detail recording;

–	get call detail recording control data;

–	modify call detail recording control data.

8.1.1	Initiate call detail recording

The PT-CREATE service defined in Recommendation X.730 is used to create an instance of the simpleUsageMeteringControl or configurableSimpleUsageMeteringControl object (only one instance is allowed per NE). Data are collected and reported as indicated by the attribute values. The formal description of the attributes can be found in Annex A.

The following describes the values that will be assigned to the control attributes of the simpleUsageMeteringControl object in response to a PT-CREATE request:

�reportingTriggers

This attribute is defined in Recommendation X.742. It is used only for periodic partial record generation. It is defined as a CHOICE of which only "time period" is supported by this Recommendation. If this parameter is not specified, the default value of 30 minutes is used.

accountableObjectReferenceList

This attribute is defined in Recommendation X.742. Object classes which describe accountable resources should be known at design time. This Recommendation makes use of two accountable object classes representing DN and trunk group. However, in the context of this Recommendation, this list may refer to objects representing a group of accountable objects. This is because, for this model to be practical, it must be realized that accountable objects must be defined in some kind of group basis. For example, accountable object might represent all the DNs in the switch.

The specification of further (new or existing) accountable object classes may be operator- or manufacturer-specific and is outside the scope of this Recommendation.

dataObject referenceList

This attribute is defined in Recommendation X.742. The reference list contains pointers to the data objects controlled by the control object, and points to the call detail data. The list is maintained by the NE and cannot be specified as part of the create request, nor can it be queried by the manager.

creationTriggerList

This attribute specifies the events leading to the implicit creation of a callDetailData object. Triggers can be call related (Seizure or first digit or ACM received or ANM received) and non-call related (supplementary service invocation and/or input).

The following describes the additional values that will be assigned to the control attributes of the configurableSimpleUsageMeteringControl object in response to a PT-CREATE request:

samplingRate

The sampling rate is expressed as an integer n and will result in every nth callDetailData object being flagged at creation time with the instruction to set the sample bit in the CDR purpose field. 

configurationMask

This attribute is a bitmap that allows identification of the optional notification parameters that must be included in notifications generated by callDetailData objects that are under control of this control object.

8.1.2	Terminate call detail recording for usage

The PT-DELETE service defined in Recommendation X.730 is used to delete an instance of the simpleUsageMeteringControl. 

8.1.3	Terminate call detail recording for analysis

The PT-DELETE service defined in Recommendation X.730 is used to delete an instance of the configurableSimpleUsageMeteringControl object.

8.1.4	Get call detail control data

The PT-GET service defined in Recommendation X.730 may be used to retrieve the values of the readable attributes except for the attribute dataObjectReferenceList. The data object reference list is maintained by the NE and changes continually (it contains an entry for each connection of which data are being collected).

�8.1.5	Modify call detail control data

The PT-SET service defined in Recommendation X.730 is used to manage the settable attributes.

8.2	Call detail data function

This function emits the CDR notification for usage selected by the call detail control function. A CDR notification may be sent out if one of the following events occurs during the transaction:

–	termination of a service;

–	change of service, e.g. due to change of charging conditions;

–	reaching a volume threshold – this may also be due to NE internal reasons;

–	at regular intervals during a practical service transaction;

–	expiration of the periodic timer (defined in the Call detail recording control function);

–	supplementary service.

8.3	CDR transfer

The PT-EVENT-REPORT service defined in Recommendation X.730 is used to report the collected call detail record.

The formal description of the attributes can be found in Annex A.

8.3.1	Real-time CDR reporting function

This function controls the generation and transmission of notifications from NE to the OS.

In this case, EFD is used to filter on the attributes of the notification and decide whether to send or not an event-report.

For billing purposes, a special field "immediateNotificationForUsageMetering" may be used. The value of this field is controlled by call processing only and may be derived in the NE from a subscriber characteristic or action, or by any other means specific to the implementation. Typically, the use of this field facilitates the selection of CDRs related to hot-billing.

The presence of more than one OS interested in real-time collection of usage metering data may be a practical issue. "Hot billing" may be requested for different purposes by different OSs at the same time. The possibility of multiple instances of the EFD is therefore not precluded.

The operations required to manage the CDR reporting are:

–	initiation of event report forwarding;

–	termination of event report forwarding;

–	event forwarding modification;

–	retrieval of eventForwardingDiscriminator attributes;

–	supplementary service input.

All these operations are defined in the Event Report Systems Management Function (clause 9/X.734).

To transmit the real-time billing information, the NE uses the CDR-REPORT service. Table 1 shows the parameters required in the event report. The event reports may be sent in confirmed or unconfirmed mode. Determination of whether or not a confirmation is required is determined by the mode parameter in the EFD. Confirmations contain no parameters. 

�Table 1/Q.825 – CDR-REPORT

Parameter name�Req/Ind�Resp/Cfm��Invoke Identifier�P�P��Mode�P�–��Managed Object Class�P�–��Managed Object Instance�P�–��Event Type�P�P��Event Time�U�U��Event Information�����Record Id�U�–��Record Type�M�–��Start Time Stamp�M�–��Participant Information�M�–��Bearer Service�C c9�–��Service User�M�–��Call Identification Number�M�–��Supplementary Service List�C c1�–��Immediate Notification for Usage Metering�U�–��Cause�C c3�–��Data Validity�C c2���IN Specific Information�C c4�–��Service Specific IN Information List�C c4�–��Partial Generation�C c5�–��Exchange Information�U�–��Related Call Number�U�–��CDR Purpose�U�–��Additional Participant Information�U�–��Calling Party Category�U�–��Calling Party Type�U�–��Charging Information�U�–��Progress�U�–��Access Delivery�U�–��Trunk Group Outgoing�U�–��Trunk Group Incoming�U�–��Network Provider Id�C c6�–��Fallback Bearer Service�C c7�–��Teleservice�U�–��Call Duration�C c8�–��User-to-User Information Counter �U�–���Table 1/Q.825 – CDR-REPORT (concluded)

Parameter name�Req/Ind�Resp/Cfm��B-Party Category�U�–��ISUP Preferred�U�–��Network Management Control�U�–��Glare�U�–��Standard Extensions�U�–��Record Extension�U�–��Current Time�–�U��Event Reply�–�–��Errors�–�P��Call Status�U�–��Carrier Id�C c10�–��DPC�U�–��OPC�U�–��c1:	This parameter must be present if this is a supplementary service input record or if the user invoked a supplementary service as part of the call.

c2:	This parameter must be present if the exchange has determined that the data contained in this notification may be inaccurate.

c3:	This parameter must be present if the exchange has determined that the call has failed.

c4:	This parameter must be present if an IN service is invoked that makes use of this information.

c5:	This parameter must be present if this notification only contains information about a part of the call.

c6:	This parameter must be present if the exchange supports multiple network providers.

c7:	This parameter must be present only if the network is providing the fallback bearer service instead of the bearer service requested by the user.

c8:	This parameter must be present in any CDR related to a call [recordType="call"].

c9:	This parameter is only for ISDN calls.

c10:	This parameter must be present if carrier selection has been applied to the call.��8.3.2	Near Real-time CDR reporting function

To enable the NEF to transfer blocks of CDRs to the OSF with a higher efficiency than the standard EFD, individual notifications may be grouped and transferred as a single unit by first storing these notifications in the blockGeneratingLog. To further add to the efficiency, only stripped records are sent in the notification. These stripped records have similar syntax to RecordContent except that the choice productions omit information added by the log, i.e. current logging time, managed object class, managed object instance and logRecordId. The presence of more than one OS interested in collection of usage metering data may be a practical issue. Usage metering data may be requested for different purposes by different OSs at the same time. The possibility of multiple instances of the blockGeneratingLog is therefore not precluded.

�Emission of a blockRecordNotification (and subsequent emptying of the log) may be triggered by one of the following events:

•	maximum time period elapsed;

•	maximum number of CDRs reached;

•	internal size limit reached.

The operations required to manage the near Real-time CDR reporting are:

•	initiation of CDR Block logging;

	(see 8.3.2.1)

•	termination of logging; 

	(defined in the Log Control Function in clause 9/X.735)

•	modification of log attributes;

	(defined in the Log Control Function in clause 9/X.735)

•	retrieval of log attributes;

	(defined in the Log Control Function in clause 9/X.735)

•	initiation of event report forwarding; 

	(defined in the Event Report Systems Management Function in clause 9/X.734)

•	termination of event report forwarding;

	(defined in the Event Report Systems Management Function in clause 9/X.734)

•	event forwarding modification;

	(defined in the Event Report Systems Management Function in clause 9/X.734)

•	retrieval of eventForwardingDiscriminator attributes;

	(defined in the Event Report Systems Management Function in clause 9/X.734).

NOTE – Other operations on the log described in in clause 9/X.735 (retrieval of log records, deletion of log records ) are not allowed.

8.3.2.1	Initiation of CDR Block logging

The PT-CREATE service defined in Recommendation X.730 is used to create an instance of the CDR blockGeneratingLog. The CDR blockGeneratingLog behaves as if it were a log of infinite size derived from the Log described in Recommendation X.735.

If there is an internal implementation specific limit of the length of the blockGeneratingLog or notification (buffer size, max. message length, etc.), this may be signalled internally to the blockGeneratingLog, which will emit a notification with the trigger cause "internalSizeLimitReached".

The conditional packages from Recommendation X.735 (finite log size package, scheduling package and alarm package) are irrelevant.

The formal description of the blockGeneratingLog can be found in Annex A.

The following describes the values that will be assigned to the attributes in response to a PT�CREATE request:

Log full action: This attribute is inherited from Recommendations X.735 and X.721. It specifies the action to be taken when the maximum capacity of the log is reached. Only "wrap" is allowed (infinite log). If this parameter is not specified, "wrap" is assumed.

Discriminator construct: This attribute is inherited from Recommendations X.735 and X.721. It specifies the test conditions which will be used by the log in testing potential log records.

�Administrative state: This attribute is inherited from Recommendations X.735 and X.721. It specifies the administrative state in which the log is to be created. The log may be created in a Unlocked or Locked state. If no administrative state is specified, the Unlocked state is assumed.

Packages: None of the inherited conditional packages will be instantiated.

Maximum Block size: This maxBlockSize attribute specifies the maximum number of CDRs to be included in a blockRecord notification. An internal counter within the blockGeneratingLog counts the number CDRs currently in the log. If the value maxBlockSize is reached, a "blockRecordNotification" is emitted, the contained records are deleted and the internal counter is reset to zero. The internal counter is also reset to zero if another notification triggering event happens.

Maximum Time Period: The maxTimeInterval attribute specifies the maximum amount of time that is allowed to elapse prior to generation of a blockRecordNotification. That means, that the block transfer is triggered periodically e.g. every n seconds or minutes. The time period is also reset to zero if another event happens.

8.3.2.2	Transmission of Record Blocks

Block CDRs are sent via the CDR-BLOCK-RECORD REPORT to the OS. The CDR-BLOCK-RECORD REPORT has the parameters shown in Table 2:

Table 2/Q.825 – CDR-BLOCK-RECORD REPORT

Parameter name�Req/Ind�Resp/Cfm��Invoke Identifier�P�P��Mode�P�–��Managed Object Class�P�–��Managed Object Instance�P�–��Event Type�P�P��Event Time�U�U��Event Information����	Block Header Record�–�–��	Usage Records�M�–��Current Time�–�U��Event Reply�–�–��Errors�–�P��blockHeaderRecord: This parameter carries information that is common to all records in the block or pertains to the record as a whole, such information consists of exchangeInfo, reasonForOutput, a sequenceNumber and managementExtensions.

usageRecords: This parameter consists of a sequence of records. The content of each record is the set of parameters specified in the CDR-REPORT table, with the Exchange Information parameter optionally moved to the block header.

8.3.3	CDR transfer via file Generating Log

The special "fileGeneratingLog" is derived from the standard log function as defined in Recommendation X.735.

�The record notifications generated by the callDetailData object instance are stored locally in the NE using the logging functionality described in Recommendation X.735.

Besides the functions for retrieval and deletion of log entries provided by the Recommendation X.735 log control, the fileGeneratingLog control provides an extra functionality to support retrieval of CDR records by a suitable file transfer protocol. This functionality is supported by providing for the creation of a callDetailRecordFile either by means of create request from the OS or a trigger event internal to the NE. When the file has been created, a notification is emitted allowing managing systems to be notified of the existence of the file. The records are deleted after they have been successfully copied in to the file.

To add to the efficiency of CDR transfer, only stripped records are placed in the created file. These stripped records have similar syntax to RecordContent except that the choice productions omit information added by the log, i.e. current logging time, managed object class, managed object instance and logRecordId.

The presence of more than one OS interested in collection of call detail data may be a practical issue. Call detail data may be requested for different purposes by different OSs at the same time. The possibility of multiple OS's accessing the fileGeneratingLog and callDetailRecordFile is therefore not precluded.

For the internal creation triggers, the filename is created automatically by the NE and the file name is communicated to the OS through a notification (UM-FILE-CREATION-REPORT). In case of the action from the OS, a specific filename can be given, which overrides the automatic file naming.

The operations required to manage CDR logging and CDR file handling are:

–	initiation of CDR logging;

–	termination of CDR logging; 

–	modification of log attributes;

–	retrieval of log attributes;

–	retrieval of log records;

–	generation of a CDR file;

–	CDR file transfer.

8.3.3.1	Initiation of CDR logging

The PT-CREATE service defined in Recommendation X.730 is used to create an instance of the fileGeneratingLog. The fileGeneratingLog is derived from the Log described in Recommendation X.735.

The formal description of the fileGeneratingLog can be found in Annex A.

The following describes the values that will be assigned to the attributes in response to a PT�CREATE request:

Inherited attributes: For the values of the inherited attributes and conditional packages: See in clause 9/X.735 (initiation of logging).

Times of day: This (optional) attribute specifies a list of times at which a callDetailRecordFile should be automatically generated on a daily bases.

Periodic Trigger: This (optional) attribute specifies periodic time intervals at which a callDetailRecordFile should be generated automatically on a continuous basis.

�8.3.3.2	Generation of a CDR File

The CDR- FILE -CREATION ACTION is used to generate a callDetailRecordFile containing CDRs collected in the log.

Table 3 describes the parameters that are supported in the action information and the action result.

Table 3/Q.825 – CDR-FILE-CREATION-ACTION

Parameter name�Req/Ind�Resp/Cfm��Invoke Identifier�P�P��Mode�P�–��Managed Object Class�P�–��Managed Object Instance�P�–��Action Type�P�P��Current Time�–�U��Action Reply�–�–��Errors�–�P��8.3.3.3	Internal generation of a CDR File

CDR files may be generated by the fileGeneratingLog due to internal file creation triggers. The internal file creation trigger events can be one of the following:

•	daily absolute times;

•	internal size limit reached.

	If there is an internal implementation specific limit of the length of a file or Log (buffer size, max. message length, etc.), this may be signalled internally to the Log, which may create a new file with the file trigger cause "internalSizeLimitReached".

When a file is created due to an internal trigger, the CDR-FILE-CREATION-REPORT may be used to report the existence of this file to external systems. The CDR-FILE-CREATION-REPORT has the following parameters (see Table 4):

�Table 4/Q.825 – CDR-FILE-CREATION-REPORT

Parameter name�Req/Ind�Resp/Cfm��Invoke Identifier�P�P��Mode�P�–��Managed Object Class�P�–��Managed Object Instance�P�–��Event Type�P�P��Event Time�U�U��Event Information����File Name�M�–��File Size�–�U��Reason For Output�M�–��Current Time�–�U��Action Reply�–�–��Errors�–�P��reason for output: This parameter specifies for which reason the file has been created.

file name: This parameter contains the file name of the generated callDetailRecordFile.

file size: This optional parameter indicates the size in octets of the generated callDetailRecordFile.

8.3.3.4	File transfer

File transfer from the NE is initiated by the OS and, for example, in the TMN environment uses FTAM.

If the receiving OS is an intermediate OS that makes data available to other OSs, that data can be stored in its original record format by expanding the stripped records with the information from the header and including the current logging time and a new logRecordId in the record (manipulation of the header of the log file). Upstream OSs can then use this log like any other fileGeneratingLog.

9	Functional units

See Table 5.

�Table 5/Q.825 – Functional units

Functional unit�Managed Object (MO) class�Function (Services)��Call detail Recording �callDetailData

�Internal NE function; i.e. emit notification from Call Detail Data��Call detail Control for Usage Metering�simpleUsageMeteringControl�Call detail recording control Function:

–	initiate usage metering; (PT-CREATE)

–	terminate usage metering; (PT-DELETE)

–	get usage metering control data; (PT-GET)

–	modify usage metering control data; (PT-SET)��Call detail Control for Analysis purpose�configurableSimpleUsage

	MeteringControl�Call detail recording control Function:

–	initiate usage metering; (PT-CREATE)

–	terminate usage metering; (PT-DELETE)

–	get usage metering control data; (PT-GET)

–	modify usage metering control data; (PT-SET)��Real-time CDR transfer

(It uses the standardized Event Reporting Management Functions as defined in Rec. X.734)�eventForwardingDiscriminator�Real-time CDR reporting Function:

–	initiation of EFD; (PT-CREATE)

–	termination of EFD; (PT-DELETE)

–	EFD modification; (PT-SET)

–	retrieval of discriminator attributes; (PT-GET)��Near real-time CDR transfer

(It uses the standardized Event Reporting Management Functions as defined in Rec. X.734)�blockGeneratingLog







eventForwardingDiscriminator�–	initiation of CDR block logging; (PT-CREATE)

–	termination of logging; (PT-DELETE)

–	modification of log attributes; (PT-SET)



–	initiation of event report forwarding; (PT-CREATE)

–	termination of event report forwarding; (PT-DELETE)

–	event forwarding modification; (PT-SET)  

–	retrieval of discriminator attributes; (PT-GET) ��CDR transfer via file �fileGeneratingLog

callDetailLogRecord�–	initiation of CDR logging; (PT-CREATE)

–	termination of CDR logging; (PT-DELETE)

–	modification of log attributes; (PT-SET)

–	generation of CDR file; (PT-ACTION)

�NOTE – For transfer via file, file transfer protocol can be used.��CDR log record retrieval from fileGeneratingLog�fileGeneratingLog

callDetailLogRecord�–	retrieval of log records; (PT-GET)���9.1	Functional Units (FUs)

a)	FU-A: Call Detail Recording – This FU creates CallDetailData object for generation of CDR notification.

b)	FU-B: Call Detail Control for Usage Metering – This FU allows to control the criteria for generation of CDRs for Usage Metering purposes.

c)	FU-C: Call Detail Control for Analysis Purpose – This FU allows to control the criteria for generation and reporting of CDRs for Analysis purpose.

d)	FU-D: Real-time transfer of CDR – This FU allows real-time transfer of CDRs via EFD.

e)	FU-E: Near real-time transfer of CDR – This FU allows to log CDRs in blockGeneratingLog, and subsequently transfer them as notification via EFD.

f)	FU-F: CDR transfer via File – This FU allows to log CDRs in fileGeneratingLog, generate a CDR file, which can be subsequently transferred via file transfer protocol.

g)	FU-G: CDR Log Retrieval from fileGeneratingLog – This FU allows to retrieve callDetailLogRecord from fileGenerationLog.

9.2	Functional units from other Recommendations

This Recommendation also supports the functional units out of the Event Report Management Functional Unit (Recommendation X.734). It corresponds to CDR (near) real-time reporting function unit in Table 5.

9.3	Negotiation of functional units

This Recommendation assigns the following object identifier value:



{ itu-t(0) recommendation(0) q(17) q825(825) informationModel(0) functionalUnitPackage(1)}

as a value of the ASN.1 type FunctionalUnitPackageId defined in Recommendation X.701 to use for negotiating the following functional units:

0	FU-A: Call Detail Recording.

1	FU-B: Call Detail Control for Usage Metering.

2	FU-C: Call Detail Control for Analysis Purpose.

3	FU-D: Real-time transfer of CDR.

4	FU-E: Near real-time transfer of CDR.

5	FU-F: CDR transfer via File.

6	FU-G: CDR Log Retrieval from fileGeneratingLog.

Where the number identifies the bit positions in the BIT STRING assigned to the functional units, and the names reference the functional units as defined above.

Within the Systems management application context, the mechanism for negotiating the functional units is described by Recommendation X.701.

�10	Conformance

10.1	Static conformance

An implementation claiming conformance in either a manager or an agent role shall support the following capabilities:

a)	FU-A, Call Detail Recording must be supported (only in Agent).

b)	At least one of the following must be supported:

1)	FU-D, Real-time transfer of CDR.

2)	FU-E, Near real-time transfer of CDR.

3)	FU-F, CDR transfer via File.

The managed objects and services that need to be supported are listed in Table 5 and formally described in Annex A.

The implementation shall support:

•	Q3 interface as defined in Recommendations Q.811 and Q.812;

•	transfer syntax derived from the basic encoding rules specified in Recommendation X.209 for the data types referenced by the definitions for which support is claimed.

10.2	Dynamic conformance

The system shall, in the role(s) for which conformance is claimed, support the elements of procedure defined in:

•	Recommendation X.730 for the PT-GET, PT-CREATE, PT-DELETE, PT-SET, PT-EVENT-REPORT, PT-ACTION, object creation reporting, object deletion reporting and attribute change reporting services.

Annex A

Information model

A.1	Overview

This Annex contains the formal description of the information model for this Recommendation. It consists of a simplified Entity-Relationship (ER) model by way of introduction, together with an object model specified in terms of the GDMO templates defined in Recommendation X.722 "Guidelines for the Definition of Managed Objects".

The generation of usage metering records is controlled by the Usage Metering Control object together with the optional triggering objects.

The notifications generated by the Usage Metering Data object presented to the EFD may also be logged locally within the NE. Each managed NE may contain a usage metering record log. 

The usage metering record log may contain one or more usage metering log entries each of which in turn contains one or more usage metering record notifications.

�� EMBED Designer.Drawing.7  ���

Figure A.1/Q.825 – The call detail recording view

A.2	Naming hierarchy

The naming (containment) tree for the objects defined within this Annex is illustrated in Figure A.2 below. It should be noted that all of the object classes are shown relative to the "managedElement".

�� EMBED Designer.Drawing.7  ���

Figure A.2/Q.825 – The naming tree

A.3	Inheritance

The inheritance tree for this specification is illustrated in Figure A.3. The object classes "log", "logRecord", "eventLogRecord", and "eventForwardingDiscriminator" are defined in Recommendation X.721.

� EMBED Designer.Drawing.7  ���

Figure A.3/Q.825 – The inheritance tree

�A.4	Managed object classes

A.4.1	Block Generating Log

This managed object class is a subclass of the "Log" class described in Recommendation X.735 and defined in Recommendation X.721 and therefore inherits all of the properties of the "log" class.



blockGeneratingLog MANAGED OBJECT CLASS

   DERIVED FROM       "ITU-T Rec. X.721 | ISO/IEC 10165-2 : 1992":log;

   CHARACTERIZED BY

      blockGeneratingLogPkg PACKAGE

         BEHAVIOUR

            blockGeneratingLogBhv BEHAVIOUR

               DEFINED AS

                  "This log is considered to be infinite and therefore it does not�                  have to instantiate the finite-log size package from its superclass�                  log. The blocking-log stores all records that satisfy its�                  discriminator construct. The log provides the value (sequence�                  number) for the 'recordId' (field of CDR). An instance of this log�                  emits the block record notification when any of the following events�                  occurs:

                  –	the number of records in the log becomes equal to the maximum block size,�                  –	the time interval elapsed since the first record currently�                      contained in the log exceeds the value maxTimeInterval attribute,�                  –	an internal system limitation has been exceeded, including the�                  	blocking log itself overflowing.

                 Upon emitting the block record notification all records stored in�                 the blocking log are deleted and the log is ready to store new�                 records. Because of the self-emptying nature of this log, any�                 log-full action may be selected and the behavior of the log will not�                 change.";;

         ATTRIBUTES

            maxBlockSize GET-REPLACE,

            maxTimeInterval GET-REPLACE;

         NOTIFICATIONS

            blockRecordNotification;;;

REGISTERED AS {q825ObjectClass 1};

A.4.2	Call detail data



callDetailData MANAGED OBJECT CLASS

   DERIVED FROM       "ITU-T Rec. X.721 | ISO/IEC 10165-2 : 1992":top;

   CHARACTERIZED BY

      callDetailDataPackage PACKAGE

         BEHAVIOUR

            callDetailDataBehaviour BEHAVIOUR

               DEFINED AS

                  "This managed object class emits the CDR notification for

                  telecommunication events selected by the control object. (The

                  selection is made based on either one or the combination of the

                  simpleUsageMeteringControl object and

                  configurableSimpleUsageMeteringControl object. No management

                  operations are permitted on this object class. A CDR notification is

                  sent out if one of the following events occurs:

                  –	termination of a service;

                  –	change of service e.g. due to change of charging conditions;

                  –	reaching a volume threshold;

                  –	at regular intervals during a practical service transaction;

                  –	supplementary service input.

                  Additional conditions that cause call detail record notification for

�                  analysis purposes such as quality of service measurement, fraud

                  detection or traffic management are:

                  –	denial of a service;

                  –	change of observed service quality;

                  –	reaching a quality threshold;

                  –	call failures

                  ";;

         ATTRIBUTES

            callDetailDataId GET;

         NOTIFICATIONS

            callDetailRecordNotification;;;

REGISTERED AS {q825ObjectClass 2};

A.4.3	Call Detail Log Record

This managed object class is a subclass of the "eventLogRecord" class described in Recommendation X.735 and defined in Recommendation X.721 and therefore inherits all of the properties of both the "logRecord" and the "eventLogRecord" classes.



callDetailLogRecord MANAGED OBJECT CLASS

   DERIVED FROM       "ITU-T Rec. X.721 : 1992":eventLogRecord;

   CHARACTERIZED BY

      callDetailLogRecordPackage PACKAGE

         BEHAVIOUR

            callDetailLogRecordBehaviour BEHAVIOUR

               DEFINED AS

                  "This object class describes the layout of a log record created for

                  each single call detail record notification";;

         ATTRIBUTES

            recordType GET,

            startTimeStamp GET,

            serviceUser GET,

            callIdentificationNumber GET;;;

   CONDITIONAL PACKAGES

      accessDeliveryPackage  PRESENT IF  "this parameter was present in the

         received notification",

      accountCodeInputPackage  PRESENT IF  "this parameter was present in

         the received notification",

      bearerServicePackage  PRESENT IF  "this parameter was present in the

         received notification",

      b-PartyCategoryPackage  PRESENT IF  "this parameter was present in

         the received notification",

      calledPartyNumberPackage  PRESENT IF  "this parameter was present in

         the received notification",

      callingPartyCategoryPackage  PRESENT IF  "this parameter was present

         in the received notification",

      callingPartyNumberPackage  PRESENT IF  "this parameter was present in

         the received notification",

      callingPartyNumberNotScreenedPackage  PRESENT IF  "this parameter was

         present in the received notification",

      callingPartyTypePackage  PRESENT IF  "this parameter was present in

         the received notification",

      callStatusPackage  PRESENT IF  "this parameter was present in the

         received notification",

      carrierIdPackage  PRESENT IF  "this parameter was present in the

         received notification",

      causePackage  PRESENT IF  "this parameter was present in the received

         notification",

      cDRPurposePackage  PRESENT IF  "this parameter was present in the

         received notification",

�      chargedDirectoryNumberPackage  PRESENT IF  "this parameter was

         present in the received notification",

      chargedParticipantPackage  PRESENT IF  "this parameter was present in

         the received notification",

      chargingInformationPackage  PRESENT IF  "this parameter was present

         in the received notification",

      conversationTimePackage  PRESENT IF  "this parameter was present in

         the received notification",

      dataValidityPackage  PRESENT IF  "this parameter was present in the

         received notification",

      dPCPackage  PRESENT IF  "this parameter was present in the received

         notification",

      durationTimeACMPackage  PRESENT IF  "this parameter was present in

         the received notification",

      durationTimeB-AnswerPackage  PRESENT IF  "this parameter was present

         in the received notification",

      durationTimeNoB-AnswerPackage  PRESENT IF  "this parameter was

         present in the received notification",

      exchangeInfoPackage  PRESENT IF  "this parameter was present in the

         received notification",

      fallbackBearerServicePackage  PRESENT IF  "this parameter was present

         in the received notification",

      glarePackage  PRESENT IF  "this parameter was present in the received

         notification",

      immediateNotificationForUsageMeteringPackage  PRESENT IF  "this

         parameter was present in the received notification",

      iNPackage  PRESENT IF  "this parameter was present in the received

         notification",

      iSUPPreferredPackage  PRESENT IF  "this parameter was present in the

         received notification",

      networkManagementControlsPackage  PRESENT IF  "this parameter was

         present in the received notification",

      networkProviderIdPackage  PRESENT IF  "this parameter was present in

         the received notification",

      oPCPackage  PRESENT IF  "this parameter was present in the received

         notification",

      operatorSpecific1AdditionalNumberPackage  PRESENT IF  "this parameter

         was present in the received notification",

      operatorSpecific1NumberPackage  PRESENT IF  "this parameter was

         present in the received notification",

      operatorSpecific2AdditionalNumberPackage  PRESENT IF  "this parameter

         was present in the received notification",

      operatorSpecific2NumberPackage  PRESENT IF  "this parameter was

         present in the received notification",

      operatorSpecific3AdditionalNumberPackage  PRESENT IF  "this parameter

         was present in the received notification",

      operatorSpecific3NumberPackage  PRESENT IF  "this parameter was

         present in the received notification",

      originalCalledNumberPackage  PRESENT IF  "this parameter was present

         in the received notification",

      partialGenerationPackage  PRESENT IF  "this parameter was present in

         the received notification",

      percentageToBeBilledPackage  PRESENT IF  "this parameter was present

         in the received notification",

      personalUserIdPackage  PRESENT IF  "this parameter was present in the

         received notification",

      physicalLineCodePackage  PRESENT IF  "this parameter was present in

         the received notification",

      progressPackage  PRESENT IF  "this parameter was present in the

         received notification",

�      queueInfoPackage  PRESENT IF  "this parameter was present in the

         received notification",

      receivedDigitsPackage  PRESENT IF  "this parameter was present in the

         received notification",

      recordIdPackage  PRESENT IF  "this parameter was present in the

         received notification",

      recordExtensionsPackage  PRESENT IF  "this parameter was present in

         the received notification",

      redirectingNumberPackage  PRESENT IF  "this parameter was present in

         the received notification",

      redirectionNumberPackage  PRESENT IF  "this parameter was present in

         the received notification",

      relatedCallNumberPackage  PRESENT IF  "this parameter was present in

         the received notification",

      serviceSpecificINInformationPackage  PRESENT IF  "this parameter was

         present in the received notification",

      standardExtensionsPackage  PRESENT IF  "this parameter was present in

         the received notification",

      supplementaryServicePackage  PRESENT IF  "this parameter was present

         in the received notification",

      teleservicePackage  PRESENT IF  "this parameter was present in the

         received notification",

      trunkGroupIncomingPackage  PRESENT IF  "this parameter was present in

         the received notification",

      trunkGroupOutgoingPackage  PRESENT IF  "this parameter was present in

         the received notification",

      uUInfoPackage  PRESENT IF  "this parameter was present in the

         received notification";

REGISTERED AS {q825ObjectClass 3};

A.4.4	Configurable Simple Usage Metering Control Object



configurableSimpleUsageMeteringControl MANAGED OBJECT CLASS

   DERIVED FROM       simpleUsageMeteringControl;

   CHARACTERIZED BY

      configurableSimpleUsageMeteringControlPackage PACKAGE

         BEHAVIOUR

            configurableSimpleUsageMeteringControlBehaviour BEHAVIOUR

               DEFINED AS

                  "This object class is used if Analysis function is required.

                  configurableSimpleUsageMeteringControl object allows sampling rate

                  and allows configuration of all the optional parameters, as defined

                  by configurationMask, to be included in the Call Detail

                  Notification. The object class controls the creation of call Detail

                  Data objects based on the creationTriggerList, and the samplingRate.

                  

                  The configurationMask together will control the additional data to

                  be included in the CDR notification emitted by the CDR object for

                  QoS, Fraud and Traffic Network Management purpose. The

                  characteristic additional is with reference to the data already

                  included in the notification for the purpose of usage metering";;

         ATTRIBUTES

            samplingRate GET-REPLACE,

            configurationMask GET-REPLACE;;;

REGISTERED AS {q825ObjectClass 4};

�A.4.5	File Generating Log

This managed object class is a subclass of the "Log" class described in Recommendation X.735 and defined in Recommendation X.721 and therefore inherits all of the properties of  the "log" class.



fileGeneratingLog MANAGED OBJECT CLASS

   DERIVED FROM       "ITU-T Rec. X.721 | ISO/IEC 10165-2 : 1992":log;

   CHARACTERIZED BY

      fileGeneratingLogPkg PACKAGE

         BEHAVIOUR

            fileGeneratingLogBhv BEHAVIOUR

               DEFINED AS

                  "This log is used to create files that can be exchanged using an

                  appropriate file transfer protocol. The log provides the value

                  (sequence number) for the 'recordId' (field of the CDR). The action

                  create file is used to generate the file to be exchanged. The file

                  created consists of a concatenation of the content of the call

                  detail records; i.e. the call detail records without the record

                  overhead (Record Id, Managed Object Class and Instance and Logging

                  Time). To avoid duplication of CDRs, logging of blockedRecord

                  notifications emitted by the blocking log should be excluded by

                  configuration of the file generating log's discriminator construct.

                  Files may also be created due to internal trigger events. One such

                  internal trigger is based on time of day. The records are deleted

                  after successful copying of the records to the file.";;

         ACTIONS

            createFile;;;

   CONDITIONAL PACKAGES

      dailyTriggeringPackage  PRESENT IF  "if the file creation is to be

         triggered on a daily basis [time(s) of day]",

      periodicTriggeringPackage  PRESENT IF  "if the file creation is to be

         triggered on a periodic basis",

      fileCreationNotificationPackage  PRESENT IF  "if the file creation is

         triggered using any one of the scheduling mechanisms in the daily,

         weekly and monthly triggering methods or an internal mechanism.";

REGISTERED AS {q825ObjectClass 5};

A.4.6	Simple Usage Metering Control Object



simpleUsageMeteringControl MANAGED OBJECT CLASS

   DERIVED FROM       "ITU-T Rec. X.742 | ISO/IEC 10164-10 : 1995":usageMeteringControlObject;

   CHARACTERIZED BY

      simpleUsageMeteringControlObjectPackage PACKAGE

         BEHAVIOUR

            usageMeteringControlBehaviour BEHAVIOUR

               DEFINED AS

                  "simpleUsageMeteringControl object class allows the definition of

                  different types of triggers for usage metering recording. One of the

                  defined reporting triggers requires periodic reporting and can be

                  used to transfer partial service usage data to a log or remote OS

                  for long duration service usage. Another type of trigger is

                  determined by the occurrence of a particular event during usage of

                  the service, e.g. completion of usage. This object class fully

                  supports the requirements for usage metering. If only Usage Metering

                  application is required, then simpleUsageMeteringControl object

                  class shall be used. This object class controls the generation of

                  CDRs in the NE. CDRs are actually produced if the transaction

                  fulfils the conditions defined in the creationTriggerList attribute.

                  If the creationTriggerList is empty, no CDR will be generated. The

                  value of the time period in reportingTriggers defines the time

�                  interval between partial records. No attribute value change

                  notification is sent by this object when data object reference list is changed and

                  no attribute change value reflects this data object reference list.";;

         ATTRIBUTES

            creationTriggerList

               DEFAULT VALUE Q825-CDR-ASN1Module.defaultCreationTrigger

              GET-REPLACE,

            "ITU-T Rec. X.742 | ISO/IEC 10164-10 : 1995":reportingTriggers

               REPLACE-WITH-DEFAULT

               DEFAULT VALUE Q825-CDR-ASN1Module.reportingTriggersDefault

               PERMITTED VALUES Q825-CDR-ASN1Module.PermittedReportingTriggers

              GET-REPLACE ADD-REMOVE;;;

REGISTERED AS {q825ObjectClass 6};

A.5	Packages

A.5.1	Access Delivery Package



accessDeliveryPackage PACKAGE

	ATTRIBUTES

	accessDelivery GET;

REGISTERED AS {q825Package 1};

A.5.2	Account Code Input Package



accountCodeInputPackage PACKAGE

	ATTRIBUTES

	accountCodeInput GET;

REGISTERED AS {q825Package 2};

A.5.3	b-Party Category Package



b-PartyCategoryPackage PACKAGE

	ATTRIBUTES

	b-PartyCategory GET;

REGISTERED AS {q825Package 3};

A.5.4	Bearer Service Package



bearerServicePackage PACKAGE

	ATTRIBUTES

	bearerService GET;

REGISTERED AS {q825Package 58};

A.5.5	CDR Purpose Package



cDRPurposePackage PACKAGE

	ATTRIBUTES

	cDRPurpose GET;

REGISTERED AS {q825Package 10};

A.5.6	Call Status Package



callStatusPackage PACKAGE

	ATTRIBUTES

	callStatus GET;

REGISTERED AS {q825Package 59};

�A.5.7	Called Party Number Package



calledPartyNumberPackage PACKAGE

	ATTRIBUTES

	calledPartyNumber GET;

REGISTERED AS {q825Package 4};

A.5.8	Calling Party Category Package



callingPartyCategoryPackage PACKAGE

	ATTRIBUTES

	callingPartyCategory GET;

REGISTERED AS {q825Package 5};

A.5.9	Calling Party Number Not Screened Package



callingPartyNumberNotScreenedPackage PACKAGE

	ATTRIBUTES

	callingPartyNumberNotScreened GET;

REGISTERED AS {q825Package 7};

A.5.10	Calling Party Number Package



callingPartyNumberPackage PACKAGE

	ATTRIBUTES

	callingPartyNumber GET;

REGISTERED AS {q825Package 6};

A.5.11	Calling Party Type Package



callingPartyTypePackage PACKAGE

	ATTRIBUTES

	callingPartyType GET;

REGISTERED AS {q825Package 8};

A.5.12	Carrier Id Package



carrierIdPackage PACKAGE

	ATTRIBUTES

	carrierId GET;

REGISTERED AS {q825Package 60};

A.5.13	Cause Package



causePackage PACKAGE

	ATTRIBUTES

	cause GET;

REGISTERED AS {q825Package 9};

A.5.14	Charged Directory Number Package



chargedDirectoryNumberPackage PACKAGE

	ATTRIBUTES

	chargedDirectoryNumber GET;

REGISTERED AS {q825Package 11};

�A.5.15	Charged Participant Package



chargedParticipantPackage PACKAGE

	ATTRIBUTES

	chargedParticipant GET;

REGISTERED AS {q825Package 13};

A.5.16	Charging Information Package



chargingInformationPackage PACKAGE

	ATTRIBUTES

	chargingInformation GET;

REGISTERED AS {q825Package 12};

A.5.17	Conversation Time Package



conversationTimePackage PACKAGE

	ATTRIBUTES

	conversationTime GET;

REGISTERED AS {q825Package 14};

A.5.18	DPC Package



dPCPackage PACKAGE

	ATTRIBUTES

	dPC GET;

REGISTERED AS {q825Package 61};

A.5.19	Daily Triggering Package



dailyTriggeringPackage PACKAGE

	ATTRIBUTES

	timesOfDay GET-REPLACE;

REGISTERED AS {q825Package 15};

A.5.20	Data Validity Package



dataValidityPackage PACKAGE

	ATTRIBUTES

	dataValidity GET;

REGISTERED AS {q825Package 16};

A.5.21	Duration Time  ACM Package



durationTimeACMPackage PACKAGE

	ATTRIBUTES

	durationTimeACM GET;

REGISTERED AS {q825Package 17};

A.5.22	Duration Time B-answer Package



durationTimeB-AnswerPackage PACKAGE

	ATTRIBUTES

	durationTimeB-Answer GET;

REGISTERED AS {q825Package 19};

�A.5.23	Duration Time No B-answer Package



durationTimeNoB-AnswerPackage PACKAGE

	ATTRIBUTES

	durationTimeNoB-Answer GET;

REGISTERED AS {q825Package 20};

A.5.24	Exchange Info Package



exchangeInfoPackage PACKAGE

	ATTRIBUTES

	exchangeInfo GET;

REGISTERED AS {q825Package 21};

A.5.25	Fallback Bearer Service Package



fallbackBearerServicePackage PACKAGE

	ATTRIBUTES

	fallbackBearerService GET;

REGISTERED AS {q825Package 22};

A.5.26	File Creation Notification Package



fileCreationNotificationPackage PACKAGE

	NOTIFICATIONS

	fileCreationNotification;

REGISTERED AS {q825Package 23};

A.5.27	Glare Package



glarePackage PACKAGE

	ATTRIBUTES

	glare GET;

REGISTERED AS {q825Package 24};

A.5.28	IN Package



iNPackage PACKAGE

	ATTRIBUTES

		iNServiceInformationList GET,

	iNSpecificInformation GET;

REGISTERED AS {q825Package 27};

A.5.29	ISUPPreferred Package



iSUPPreferredPackage PACKAGE

	ATTRIBUTES

	iSUPPreferred GET;

REGISTERED AS {q825Package 28};

A.5.30	Immediate Notification for Usage MeteringPackage



immediateNotificationForUsageMeteringPackage PACKAGE

	ATTRIBUTES

	immediateNotificationForUsageMetering GET;

REGISTERED AS {q825Package 25};

�A.5.31	Network Management Controls Package



networkManagementControlsPackage  PACKAGE

	ATTRIBUTES

	networkManagementControls GET;

REGISTERED AS {q825Package 29};

A.5.32	Network Provider Id Package



networkProviderIdPackage PACKAGE

	ATTRIBUTES

	networkProviderId GET;

REGISTERED AS {q825Package 30};

A.5.33	OPC Package



oPCPackage PACKAGE

	ATTRIBUTES

		oPC GET;

REGISTERED AS {q825Package 62};

A.5.34	Operator Specific1 Additional Number Package



operatorSpecific1AdditionalNumberPackage PACKAGE

	ATTRIBUTES

		operatorSpecific1AdditionalNumber GET;

REGISTERED AS {q825Package 31};

A.5.35	Operator Specific1 Number Package



operatorSpecific1NumberPackage PACKAGE

	ATTRIBUTES

		operatorSpecific1Number	 GET;

REGISTERED AS {q825Package 32};

A.5.36	Operator Specific2 Additional Number Package



operatorSpecific2AdditionalNumberPackage PACKAGE

	ATTRIBUTES

	operatorSpecific2AdditionalNumber   GET;

REGISTERED AS {q825Package 33};

A.5.37	Operator Specific2 Number Package



operatorSpecific2NumberPackage PACKAGE

	ATTRIBUTES

	operatorSpecific2Number	 GET;

REGISTERED AS {q825Package 34};

A.5.38	Operator Specific3 Additional Number Package



operatorSpecific3AdditionalNumberPackage	  PACKAGE

	ATTRIBUTES

		operatorSpecific3AdditionalNumber GET;

REGISTERED AS {q825Package 35};

�A.5.39	Operator Specific3 Number Package



operatorSpecific3NumberPackage PACKAGE

	ATTRIBUTES

		operatorSpecific3Number GET;

REGISTERED AS {q825Package 36};

A.5.40	Original Called Number Package



originalCalledNumberPackage PACKAGE

	ATTRIBUTES

		originalCalledNumber GET;

REGISTERED AS {q825Package 37};

A.5.41	Partial Generation Package



partialGenerationPackage PACKAGE

	ATTRIBUTES

		partialGeneration GET;

REGISTERED AS {q825Package 38};

A.5.42	Percentage To Be Billed Package



percentageToBeBilledPackage PACKAGE

	ATTRIBUTES

		percentageToBeBilled GET;

REGISTERED AS {q825Package 39};

A.5.43	Periodic Triggering Package



periodicTriggeringPackage PACKAGE

	ATTRIBUTES

		periodicTrigger GET-REPLACE;

REGISTERED AS {q825Package 40};

A.5.44	Personal User Id Package



personalUserIdPackage PACKAGE

	ATTRIBUTES

		personalUserId GET;

REGISTERED AS {q825Package 41};

A.5.45	Physical Line Code Package



physicalLineCodePackage PACKAGE

	ATTRIBUTES

		physicalLineCode GET;

REGISTERED AS {q825Package 42};

A.5.46	Progress Package



progressPackage PACKAGE

	ATTRIBUTES

		progress GET;

REGISTERED AS {q825Package 43};

�A.5.47	Queue Info Package



queueInfoPackage PACKAGE

	ATTRIBUTES

		queueInfo GET;

REGISTERED AS {q825Package 44};

A.5.48	Received Digits Package



receivedDigitsPackage PACKAGE

	ATTRIBUTES

		receivedDigits GET;

REGISTERED AS {q825Package 45};

A.5.49	Record Extensions Package



recordExtensionsPackage PACKAGE

	ATTRIBUTES

		recordExtensions GET;

REGISTERED AS {q825Package 46};

A.5.50	Record Id Package



recordIdPackage PACKAGE

	ATTRIBUTES

		recordId GET;

REGISTERED AS {q825Package 63};

A.5.51	Redirecting Number Package



redirectingNumberPackage		PACKAGE

ATTRIBUTES

	redirectingNumber GET;

REGISTERED AS {q825Package 48};

A.5.52	Redirection Number Package



redirectionNumberPackage PACKAGE

	ATTRIBUTES

		redirectionNumber GET;

REGISTERED AS {q825Package 49};

A.5.53	Related Call Number Package



relatedCallNumberPackage PACKAGE

	ATTRIBUTES

		relatedCallNumber GET;

REGISTERED AS {q825Package 47};

A.5.54	Service specific IN information Package



serviceSpecificINInformationPackage PACKAGE

	ATTRIBUTES

		serviceSpecificINInformation GET;

REGISTERED AS {q825Package 50};

�A.5.55	Standard Extensions Package



standardExtensionsPackage PACKAGE

	ATTRIBUTES

		standardExtensions GET;

REGISTERED AS {q825Package 51};

A.5.56	Supplementary Service Package



supplementaryServicePackage PACKAGE

	ATTRIBUTES

		supplementaryServices GET;

REGISTERED AS {q825Package 52};

A.5.57	Teleservice Package



teleservicePackage PACKAGE

	ATTRIBUTES

		teleservice GET;

REGISTERED AS {q825Package 54};

A.5.58	Trunk Group Incoming Package



trunkGroupIncomingPackage PACKAGE

	ATTRIBUTES

		trunkGroupIncoming GET;

REGISTERED AS {q825Package 55};

A.5.59	Trunk Group Outgoing Package



trunkGroupOutgoingPackage PACKAGE

	ATTRIBUTES

		trunkGroupOutgoing GET;

REGISTERED AS {q825Package 56};

A.5.60	User-to-User Info Package



uUInfoPackage PACKAGE

	ATTRIBUTES

		uUInfo GET;

REGISTERED AS {q825Package 57};

A.6	Attributes

A.6.1	Access Delivery



accessDelivery	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.AccessDelivery;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	accessDeliveryBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the Access delivery information which indicates if the call has been delivered to the called subscriber. Normally the value is derived from the ISUP signalling. In the case of local calls, where no ISUP signalling is used, a similar value shall be adopted. Access delivery can be used for statistical purposes or to determine whether the charged subscriber should be charged with call attempt charge or not.";;

REGISTERED AS {q825Attribute 1};

�A.6.2	Account Code Input



accountCodeInput ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.AccountCodeInput;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	accountCodeInputBehaviour BEHAVIOUR

	DEFINED AS "This information element shall contain the code to which accounting is to be referred and input by the subscriber. Examples of use are authorisation code, PI, bank account, PV code or credit card number to be used for billing.";;

REGISTERED AS {q825Attribute  2};

A.6.3	Additional Participant Info



additionalParticipantInfo ATTRIBUTE

	WITH ATTRIBUTE SYNTAX    Q825-CDR-ASN1Module.AdditionalParticipantInfo;

	MATCHES FOR EQUALITY;

REGISTERED AS {q825Attribute 78};

A.6.4	B-party Category 



b-PartyCategory ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.B-PartyCategory;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	b-PartyCategoryBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the called subscriber category.";;

REGISTERED AS {q825Attribute 5};

A.6.5	Bearer Service



bearerService	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module. BearerService;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	bearerServiceBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the bearer capability information for a call or an event concerning a supplementary service. This attribute exists only for ISDN calls.";;

REGISTERED AS {q825Attribute  4};

A.6.6	CDR Purpose



cDRPurpose	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.CDRPurpose;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	cDRPurposeBehaviour BEHAVIOUR

	DEFINED AS "This attribute provides information to the OS on the reason for triggering of the record. In most networks the OS will be aware of the triggering conditions of the network and hence this information will not be required.";;

REGISTERED AS {q825Attribute 13};

A.6.7	Call Detail Data Identifier



callDetailDataId  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.CallDetailDataId;

	BEHAVIOUR

	callDetailDataIdBehaviour BEHAVIOUR

	DEFINED AS "This attribute uniquely identifies the callDetailData object.";;

REGISTERED AS {q825Attribute 70};

�A.6.8	Call Duration



callDuration ATTRIBUTE

	WITH ATTRIBUTE SYNTAX    Q825-CDR-ASN1Module.CallDuration;

	MATCHES FOR EQUALITY;

REGISTERED AS {q825Attribute 79};

A.6.9	Call Identification Number



callIdentificationNumber  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.CallIdentificationNumber;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	callIdentificationNumberBehaviour BEHAVIOUR

	DEFINED AS "An identification number that identifies the call. All records produced for the same call have the same call identification number.�With the call identification number it is possible to link partial outputs, outputs due to supplementary services during the call and to discriminate between simultaneous call establishments. If a global call reference is received through signalling, this can be used as call identification number thus allowing the correlation of CDRs generated for the same call in different NEs. If no global call reference is signalled in the network, the call identification will only have local significance.";;

REGISTERED AS {q825Attribute 6};

A.6.10	Call Status



callStatus  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.CallStatus;

	BEHAVIOUR

	callStatusBehaviour BEHAVIOUR

	DEFINED AS "This attribute identifies whether the call is answered or not. A call is  considered answered if ANM or its equivalent has been received.";;

REGISTERED AS {q825Attribute 73};

A.6.11	Called Party Number



calledPartyNumber	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.CalledPartyNumber;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	calledPartyNumberBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the telephone number of the called subscriber if the CDR is generated due to a call.�If the CDR is generated due to activation or invocation of the diversion supplementary service, then attribute contains the telephone number of the diverted-to number. In special cases this attribute may contain the translated number. This would be the case when abbreviated dialling is used. As an option, this information element may also include information to indicate the Numbering Plan Identification (NPI) and the Type Of Number (TON) of the called party number.";;

REGISTERED AS {q825Attribute 9};

A.6.12	Calling Party Category



callingPartyCategory	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.CallingPartyCategory;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	callingPartyCategoryBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the calling subscriber category. This information may also be related to the redirecting party for a forwarded call.";;

REGISTERED AS {q825Attribute 7};

�A.6.13	Calling Party Number



callingPartyNumber	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.CallingPartyNumber;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	callingPartyNumberBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the telephone number of the calling party.�The calling party will, for non-UPT calls, be identical to the calling party user. For call type calls and when subscribing to either MSN or DDI the information element indicates the screened and verified number for transfer towards the called subscriber. This attribute contains the default number if the exchange is requested to use that number for transfer towards the called subscriber. If the CDR is generated due to an event concerning a supplementary service then this attribute contains the telephone number of the subscriber that caused the event.�It should be noted that in the case of an exchange with a diverted call, the subscriber for which the CDR is generated is indicated by the redirecting number.";;

REGISTERED AS {q825Attribute 8};

A.6.14	Calling Party Number Not Screened



callingPartyNumberNotScreened	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.CallingPartyNumberNotScreened;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	callingPartyNumberNotScreenedBehaviour BEHAVIOUR

	DEFINED AS  "This attribute contains an additional user-provided, not screened telephone number of the calling party if the CDR is generated due to a call.�This attribute is relevant in connection with users subscribing to the special arrangement according to e.g. the CLIP supplementary service.";;

REGISTERED AS {q825Attribute  10};

A.6.15	Calling Party Type



callingPartyType ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.CallingPartyType;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	callingPartyTypeBehaviour BEHAVIOUR

	DEFINED AS "This information element shall contain the calling subscriber type. This information may also be related to the redirecting party for a forwarded call.�The Calling party Type is defined as:

	–	Analogue;

	–	Customer link (2 Mbit/s PSTN digital access);

	–	Basic Access;

	–	Primary Rate Access;";;

REGISTERED AS {q825Attribute  11};

A.6.16	Carrier Id



carrierId  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.CarrierId;

	BEHAVIOUR

	carrierIdBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the carrier Id to which the call is sent.";;

REGISTERED AS {q825Attribute 74};

�A.6.17	Cause



cause	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.Cause;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	causeBehaviour BEHAVIOUR

	DEFINED AS "This attribute indicates the cause and location value for the termination of the call. For ISDN/ISUP, it will contain ITU-T Recommendation Q.850 cause values (sent or received). For analogue call, or intra-switch call, the agent will provide the best mapping corresponding to the cause value of Q.850.";;

REGISTERED AS {q825Attribute 12};

A.6.18	Charged Directory Number



chargedDirectoryNumber  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.ChargedDirectoryNumber;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	chargedDirectoryNumberBehaviour BEHAVIOUR

	DEFINED AS "This information element shall contain the charged directory number in the case where the number cannot be indicated by the charged participant element.";;

REGISTERED AS {q825Attribute 14};

A.6.19	Charged Participant



chargedParticipant  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.ChargedParticipant;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	chargedParticipantBehaviour BEHAVIOUR

	DEFINED AS "This information element shall contain the participant, i.e. calling, called, redirecting, redirection or original called party, to be charged for the usage.";;

REGISTERED AS {q825Attribute 16};

A.6.20	Charging Information



chargingInformation	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.ChargingInformation;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	chargingInformationBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the charging information generated by an NE which is capable of charging. This information can either be passed on to the billing application or in the case where the NE calculates charging information for presentation to the user, this information can be used to compare the values generated by the off-line charging application. The attribute contains the charged amount either in recorded currency or call charge units.";;

REGISTERED AS {q825Attribute 15};

�A.6.21	Configuration Mask 



configurationMask ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.ConfigurationMask;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	configurationMaskBehaviour BEHAVIOUR

	DEFINED AS "This attribute is only valid for answered calls. If no partial records are generated this attribute contains the time consumption from B-answer to termination time. For partial call records the first record contains the time consumption from B-answer until reporting time. The intermediate partial records contain the time consumption since the previous reporting. The final partial record contains the time consumption from the previous reporting until termination time. The call is considered terminated when communication between the parties is no longer possible.";;

REGISTERED AS {q825Attribute 17};

A.6.22	Conversation Time



conversationTime	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.ConversationTime;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	conversationTimeBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the time consumption from B�answer or equivalent. The call is considered terminated when communication between the parties is no longer possible.";;

REGISTERED AS {q825Attribute 18};

A.6.23	Creation Trigger List



creationTriggerList	ATTRIBUTE

WITH ATTRIBUTE SYNTAX	Q825-CDR-ASN1Module.CreationTriggerList;

	MATCHES FOR EQUALITY, SET-INTERSECTION;

	BEHAVIOUR

	creationTriggerListBehaviour  BEHAVIOUR

	DEFINED AS "This attribute consists of a list of values that specify the conditions that will lead to the creation of  Call Detail data objects. Triggers can be call related (Seizure or first digit or ACM received or ANM received) and non-call related (supplementary service invocation and/or input). When multiple triggers are present, the recording process is activated by the first recordable event";;

REGISTERED AS {q825Attribute  19};

A.6.24	DPC



dPC ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.PointCode;

	BEHAVIOUR

	dPCBehaviour BEHAVIOUR

	DEFINED AS "This attribute is used for analysis purpose. It contains the destination point code.";;

REGISTERED AS {q825Attribute  75};

A.6.25	Data Validity 



dataValidity	ATTRIBUTE

WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.DataValidity;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	dataValidityBehaviour  BEHAVIOUR

	DEFINED AS "This attribute indicates that the NE is having problems and that the content of the generated CDR is not reliable. This indication enables the OS to take the necessary actions on the collected CDRs.";;

REGISTERED AS {q825Attribute 20};

�A.6.26	Duration Time Until ACM



durationTimeACM	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.DurationTimeACM;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	durationTimeACMBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the time consumption from seizure time until received ACM. This attribute is only included if ACM is received.";;

REGISTERED AS {q825Attribute  23};

A.6.27	Duration Time B-Answer



durationTimeB-Answer	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.DurationTimeANM;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	durationTimeB-AnswerBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the time consumption from the seizure time until received B-Answer. This attribute is included if B-Answer is received.";;

REGISTERED AS {q825Attribute 21};

A.6.28	Duration Time No B-Answer



durationTimeNoB-Answer	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.DurationTimeNoANM;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	durationTimeNoB-AnswerBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the time consumption from the seizure time to termination time when no B-Answer is received.";;

REGISTERED AS {q825Attribute  22};

A.6.29	Exchange Info



exchangeInfo ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.ExchangeInfo;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	exchangeInfoBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the exchange identity. By means of the exchange identity it is always possible to identify the exchange where a specific CDR was generated. In addition to the identity of the exchange, additional information can be added to indicate the program version of the originator. This information is provided to assist in interpreting the CDR when new software releases, that affect the CDR format, are introduced in the network. When all records to be transferred originate from the same exchange, this information can optionally be provided in the file header.";;

REGISTERED AS {q825Attribute 25};

A.6.30	Fallback Bearer Service



fallbackBearerService	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.FallbackBearerService;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	fallbackbearerServiceBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the fallback bearer capability information for a call or an event concerning a supplementary service. This indicator is only provided when, in the case of fallback, the bearer service is not identical to the initial requested service.";;

REGISTERED AS {q825Attribute 26};

�A.6.31	Glare



glare ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.Glare;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	glareBehaviour BEHAVIOUR

	DEFINED AS "This attribute indicates if a glare condition was encountered even if it does not cause the call to fail.";;

REGISTERED AS {q825Attribute 27};

A.6.32	IN Service Information List



iNServiceInformationList	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.INServiceInformationList;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	iNSpecificInformationListBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains information about the use of one or more IN service. Information about the use of more than one supplementary service is only possible in connection with a call record type. For each service, this attribute contains the IN service code, possible queue information and additional information (OCTET STRING).";;

REGISTERED AS {q825Attribute 31};

A.6.33	IN Specific Information



iNSpecificInformation	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.INSpecificInfo;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	iNSpecificInformationBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains information about the use of one IN service.";;

REGISTERED AS {q825Attribute 32};

A.6.34	ISUP Preferred



iSUPPreferred ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.ISUPPreferred;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	iSUPPreferredBehaviour BEHAVIOUR

	DEFINED AS "This attribute indicates if an ISUP preference was requested. For ISUP, this attribute contains the ISUP Preference Indicator field of Forwarding Call Call Indicator parameter defined in ITU�T Recommendation Q.763. If  ISUP is not used this attribute is set not applicable.";;

REGISTERED AS {q825Attribute 33};

A.6.35	Immediate Notification for Usage Metering



immediateNotificationForUsageMetering ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.ImmediateNotification;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	immediateNotificationForUsageMeteringBehaviour BEHAVIOUR

	DEFINED AS "This attribute shall contain an indication that the record requires immediate data transfer to the OS. This standard does not specify if this indication is due to a subscriber action or contained in the user data. This attribute may be used to define the filter of an event forwarding discriminator.";;

REGISTERED AS {q825Attribute 28};

�A.6.36	Max Block Size



maxBlockSize ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.MaxBlockSize;

	MATCHES FOR EQUALITY, ORDERING;

	BEHAVIOUR

	maxBlockSizeBehaviour BEHAVIOUR

	DEFINED AS "The value of this attribute specifies the maximum number of CDRs that  may be contained in the blockRecordNotification emitted by the blockGeneratingLog. If the value '0' is entered, it means that this output trigger is not used.";;

REGISTERED AS {q825Attribute 34};

A.6.37	Max Time Interval



maxTimeInterval  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.MaxTimeInterval;

	MATCHES FOR EQUALITY, ORDERING;

	BEHAVIOUR

	maxTimeIntervalBehaviour BEHAVIOUR

	DEFINED AS "The value of this attribute specifies the maximum time interval that may elapse from receipt of the first record currently in the log to the time at which a blockRecordNotification must be emitted. This value, therefore, specifies the maximum latency with which near-real-time CDR data will be transmitted to the upstream system. If the value '0' is entered, it means that this output trigger is not used.";;

REGISTERED AS {q825Attribute 35};

A.6.38	Network Management Controls 



networkManagementControls ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.NetworkManagementControls;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	networkManagementControlsBehaviour BEHAVIOUR

	DEFINED AS "This attribute indicates which Traffic Network Management control has affected the call.  This means that calls which are blocked by a traffic control will be taken into account. A call is affected by a Traffic Network Management control if the normal call routing process has been altered by a Traffic Network Management control.";;

REGISTERED AS {q825Attribute 36};

A.6.39	Network Provider ID 



networkProviderId ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.NetworkProviderId;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	networkProviderIdBehaviour BEHAVIOUR

	DEFINED AS "This attribute indicates the network provider for whom the CDR is generated. This information is only necessary in the case of multiple providers.";;

REGISTERED AS {q825Attribute 37};

A.6.40	OPC



oPC ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.PointCode;

	BEHAVIOUR

	oPCBehaviour BEHAVIOUR

	DEFINED AS "This attribute is used for analysis purpose. It contains the originating point code of the failed call.";;

REGISTERED AS {q825Attribute 76};

�A.6.41	Operator Specific1 Additional Number



operatorSpecific1AdditionalNumber	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.OperatorSpecific1AdditionalNumber;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	operatorSpecific1AdditionalNumberBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains operator defined additional participant information and is used when the elements defined elsewhere are not adequate.";;

REGISTERED AS {q825Attribute 38};

A.6.42	Operator Specific1 Number



operatorSpecific1Number  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.OperatorSpecific1Number;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	operatorSpecific1NumberBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains operator defined participant information and is used when the elements defined elsewhere are not adequate.";;

REGISTERED AS {q825Attribute 39};

A.6.43	Operator Specific2 Additional Number



operatorSpecific2AdditionalNumber  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.OperatorSpecific2AdditionalNumber;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	operatorSpecific2AdditionalNumberBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains operator defined additional participant information and is used when the elements defined elsewhere are not adequate.";;

REGISTERED AS {q825Attribute 40};

A.6.44	Operator Specific2 Number



operatorSpecific2Number  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.OperatorSpecific1Number;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	operatorSpecific2NumberBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains operator defined participant information and is used when the elements defined elsewhere are not adequate.";;

REGISTERED AS {q825Attribute  41};

A.6.45	Operator Specific3 Additional Number



operatorSpecific3AdditionalNumber  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.OperatorSpecific3AdditionalNumber;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	operatorSpecific3AdditionalNumberBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains operator defined additional participant information and is used when the elements defined elsewhere are not adequate.";;

REGISTERED AS {q825Attribute 42};

�A.6.46	Operator Specific3 Number



operatorSpecific3Number  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.OperatorSpecific3Number;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	operatorSpecific3NumberBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains operator defined participant information and is used when the elements defined elsewhere are not adequate.";;

REGISTERED AS {q825Attribute 43};

A.6.47	Original Called Number



originalCalledNumber  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.OriginalCalledNumber;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	originalCalledNumberBehaviour BEHAVIOUR

	DEFINED AS "This information element shall contain the telephone number of the original called party. This is the information, when using ISUP, that is sent in the forward direction when a call is redirected and identifies the original called party.";;

REGISTERED AS {q825Attribute 44};

A.6.48	Partial Generation



partialGeneration  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.PartialGeneration;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	partialGenerationBehaviour BEHAVIOUR

	DEFINED AS "This attribute is included if the CDR output is partial. Included in the element is a field indicating the partial record number and the reason for partial output.�The partial record number, is a sequential number which consecutively numbers the partial records in a specific call.";;

REGISTERED AS {q825Attribute  45};

A.6.49	Participant Info



participantInfo ATTRIBUTE

	WITH ATTRIBUTE SYNTAX    Q825-CDR-ASN1Module.ParticipantInfo;

	MATCHES FOR EQUALITY, SET-COMPARISON, SET-INTERSECTION;

REGISTERED AS {q825Attribute 77};

A.6.50	Percentage To Be Billed



percentageToBeBilled  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.PercentageToBeBilled;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	percentageToBeBilledBehaviour BEHAVIOUR

	DEFINED AS "This information element shall contain the percentage to be billed in the case where normal billing rules are not followed.";;

REGISTERED AS {q825Attribute 46};

�A.6.51	Periodic Trigger 



periodicTrigger	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.Period;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	periodicTriggerBehaviour BEHAVIOUR

	DEFINED AS "This attribute defines the periodic interval at which the file should be created.";;

REGISTERED AS {q825Attribute 47};

A.6.52	Personal User Identification



personalUserId 	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.PersonalUserId;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	personalUserIdBehaviour BEHAVIOUR

	DEFINED AS "This element has only relevance when UPT calls are made. It is a network requirement that the provided international personal User Identity is unique and verified by the network.";;

REGISTERED AS {q825Attribute 48};

A.6.53	Physical Line Code



physicalLineCode	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.PhysicalLineCode;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	physicalLineCodeBehaviour BEHAVIOUR

	DEFINED AS "This information element shall be used to identify the physical line used by the calling subscriber. This element is used for customer care purposes and is only relevant in the case where multiple lines are used by a common directory number.";;

REGISTERED AS {q825Attribute 49};

A.6.54	Progress



progress	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.Progress;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	progressBehaviour BEHAVIOUR

	DEFINED AS "This attribute describes an event which has occurred during the life of a call. The attribute contains a location value as well as the progress description. Progress information can be used e.g. for determining whether the subscriber shall be charged for the use of a supplementary service towards a non-ISDN user. Only the Progress indicator received from the far end is of relevance and should be used. In the case where more than one value is received only the last Progress indicator is used.";;

REGISTERED AS {q825Attribute  50};

A.6.55	Queue Info



queueInfo ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.QueueInfo;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	queueInfoBehaviour BEHAVIOUR

	DEFINED AS "Queue information is stored in connection with IN calls to record usage of queueing resources. This information element is present only if the call was queued during its treatment by the IN and contains the time when the call was queued and the duration of the queueing.";;

REGISTERED AS {q825Attribute  51};

�A.6.56	Received Digits



receivedDigits	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.ReceivedDigits;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	receivedDigitsBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the digits dialled by the subscriber or sent by the subscriber terminal. Normally this information is only included for customer care purposes. It should however be noted that the use of functional signalling in ISDN as well as requirements to restrict this information will require operator or manufacturer specific coding of this element. Alternatively functional information should be included in the supplementary service field.";;

REGISTERED AS {q825Attribute 52};

A.6.57	Record Extensions



recordExtensions  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.RecordExtensions;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	recordExtensionBehaviour BEHAVIOUR

	DEFINED AS "This attribute shall contain information elements that network operators and/or manufacturers have added to the standard usage metering record. This information element contains a set of  'management extensions' as defined in ITU-T Recommendation  X.721.";;

REGISTERED AS {q825Attribute 53};

A.6.58	Record Id



recordId ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.RecordId;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	recordIdBehaviour BEHAVIOUR

	DEFINED AS "This attribute is used for audit trail purposes to check if all logged CDRs are transferred to the OS. It is a sequential number.";;

REGISTERED AS {q825Attribute 80};

A.6.59	Record Type



recordType  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.RecordType;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	recordTypeBehaviour BEHAVIOUR

	DEFINED AS "This information element indicates the type of the CDR and it also indicates the way some of the CDR elements are used.";;

REGISTERED AS {q825Attribute 54};

�A.6.60	Redirecting Number



redirectingNumber	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.RedirectingNumber;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	redirectingNumberBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the telephone number of the party who has redirected the call. This is the information, when using ISUP, that is sent in the forward direction when a call is redirected, indicating the number from which the call was diverted. This number is identical to the original called number for a single diversion. In the case of a diverted call the Service User will be indicated by the redirecting number.";;

REGISTERED AS {q825Attribute 82};

A.6.61	Redirection Number



redirectionNumber	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.RedirectionNumber;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	redirectionNumberBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the telephone number of the party that the call must be forwarded to. This is the information, when using ISUP, that is sent in the backward direction indicating the number towards which the call must be re-routed or has been forwarded. The redirection number is identical to the diverted-to number which can be either a forwarded to number or a deflected to number.";;

REGISTERED AS {q825Attribute 56};

A.6.62	Related Call Number



relatedCallNumber	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.RedirectionNumber;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	relatedCallNumberBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the Call identification number of an associated call record. This number is used for associating different call records generated in the same exchange.";;

REGISTERED AS {q825Attribute 57};

A.6.63	Sampling Rate



samplingRate ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.Integer;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	samplingRateBehaviour BEHAVIOUR

	DEFINED AS "This attribute indicates that every n-th callDetailData will be flagged for analysis purpose and all subsequent CDRs emitted will indicate Analysis in the cDRPurpose.";;

REGISTERED AS {q825Attribute 58};

A.6.64	Service Specific IN Information



serviceSpecificINInformation ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.ServiceSpecificINInformation;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	serviceSpecificINInformationBehaviour BEHAVIOUR

	DEFINED AS "This information element shall contain the information provided by the IN-service-script to enable the billing of new services. This field contains a set of 'management extensions' as defined in ITU-T Recommendation  X.721.";;

REGISTERED AS {q825Attribute 59};

�A.6.65	Service User



serviceUser	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.ServiceUser;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	serviceUserBehaviour BEHAVIOUR

	DEFINED AS "This attribute provides information of the party whose use of resources has been accounted for. The information provided in this element is a pointer to the party number provided elsewhere in the record. In the case where e.g. the CDR is generated on a transit exchange and no calling party number is available, this element will indicate that the service user is unknown.";;

REGISTERED AS {q825Attribute 60};

A.6.66	Standard Extensions



standardExtensions  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.StandardExtensions;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	standardExtensionBehaviour BEHAVIOUR

	DEFINED AS "This attribute shall contain standardized extensions that are added to the standard usage metering record. This field contains a set of 'management extensions' as defined in ITU-T Recommendation  X.721.";;

REGISTERED AS {q825Attribute 61};

A.6.67	Start Time Stamp



startTimeStamp	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.StartTimeStamp;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	startTimeStampBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains a time stamp either the seizure time for an un�answered call (when the exchange is ready to receive digit) or the answer time for an answered call. For transit calls the start time stamp (seizure) will relate to the first message received e.g. IAM.�If the CDR is generated by an event concerning a supplementary service not related to a call, then this information element contains the time stamp for that event.�Date and time values shall be derived from the exchange clock. This attribute includes year, month, day, hour, minute and second and centisecond.�For partial outputs the start date time is the end time when the previous output was made.";;

REGISTERED AS {q825Attribute  62};

A.6.68	Supplementary services



supplementaryServices	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.SupplementaryServices;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	supplementaryServicesBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains information about the use of one or more supplementary services. Information about the use of one or more supplementary service is only possible in connection with a call record type. Each supplementary service is included as a SEQUENCE OF SupplementaryService.�Each SupplementaryService construct contains relevant information about the use of a supplementary service. The attribute contains a service code identifying the supplementary service and an addition information about the action taken. An optional time stamp is provided for time dependent charging of supplementary services. The time stamp is a duration time relative to the startTimeStamp and is recorded in centisecond.

�	For each service code information is provided indicating the action taken. The following actions are defined:

	•	Provision of supplementary service;

	•	Withdrawal of supplementary service;

	•	Registration of supplementary service;

	•	Erasure of supplementary service;

	•	Activation of supplementary service;

	•	Deactivation of supplementary service;

	•	Invocation of supplementary service;

	•	Disabling of supplementary service;

	•	Interrogation of supplementary service;

	If provided by the network, actions can be made on supplementary service by the operator. In such a case the service user element in the CDR will indicate the operator as the initiator of the action. �In the case of functional signalling on ISDN further information of the supplementary service usage is provided by the optional functional information field. This field contains a set of 'management extensions' as defined by ITU-T Recommendation X.721.";;

REGISTERED AS {q825Attribute 63};

A.6.69	Teleservice



teleservice	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.Teleservice;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	teleserviceBehaviour BEHAVIOUR

	DEFINED AS "This information element shall contain the high layer compatibility information for a call or an event concerning a supplementary service. �This information element is only relevant in connection with ISDN.";;

REGISTERED AS {q825Attribute 66};

A.6.70	Times of Day



timesOfDay  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.TimesOfDay;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	timesOfDayBehaviour BEHAVIOUR

	DEFINED AS "This attribute specifies the times at which the log will automatically generate a file.";;

REGISTERED AS {q825Attribute 67};

A.6.71	Trunk Group Incoming



trunkGroupIncoming	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.TrunkGroupIncoming;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	trunkGroupIncomingBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the symbolic name of the trunk that is used by the incoming call as seen by the NE. If the calling subscriber is a local subscriber then this information element is not relevant.";;

REGISTERED AS {q825Attribute  68};

�A.6.72	Trunk Group Outgoing



trunkGroupOutgoing	ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.TrunkGroupOutgoing;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	trunkGroupOutgoingBehaviour BEHAVIOUR

	DEFINED AS "This attribute contains the symbolic name of the trunk that is used by the outgoing call as seen from the NE. If the called subscriber is a local subscriber then this information is not relevant.";;

REGISTERED AS {q825Attribute  69};

A.6.73	User-to-User Info



uUInfo  ATTRIBUTE

	WITH ATTRIBUTE SYNTAX Q825-CDR-ASN1Module.UUInfo;

	MATCHES FOR EQUALITY;

	BEHAVIOUR

	uUInfoBehaviour BEHAVIOUR

	DEFINED AS "This attribute shall contain the User-to-User info data counter. The counter shall indicate the total number of octets or elements transmitted or received by the subscriber using the User-to-user signalling supplementary service. Separate counters can be assigned to the 3 different User-to-user signalling services.";;

REGISTERED AS {q825Attribute  81};

A.7	Actions

A.7.1	Create File



createFile				ACTION

BEHAVIOUR

createFileBhv	BEHAVIOUR

	DEFINED AS "Receipt of this action causes the creation of a file containing the concatenated content of the specified event records. If no parameters are specified in the action then the file is created from all the records currently contained in the log. For logs where the record number has wrapped, the record time is used to determine that this has occurred and the 'wrapped' records shall be included  in the created file.

	The action response contains the name and size of the created file.";;

MODE CONFIRMED;

WITH REPLY SYNTAX	Q825-CDR-ASN1Module.CreateFileResponse;

REGISTERED AS {q825Action 1};

A.8	Notifications

Unless otherwise stated, all notifications shall be sent via the M-EVENT-REPORT operation in confirmed mode.

A.8.1	Block Record Notification



blockRecordNotification	NOTIFICATION

BEHAVIOUR

blockRecordNotificationBhv	BEHAVIOUR

	DEFINED AS "This notification is emitted whenever one of the triggering events described in the object class template occurs. The notification consists of a concatenation of the content of the call detail records currently contained in the blockingLog; i.e. the call detail records without the record overhead (Log Record Id, Managed Object Class and Instance and Logging Time).";;

WITH INFORMATION SYNTAX	 Q825-CDR-ASN1Module.BlockRecordInfo;

REGISTERED AS {q825Notification 1};

�A.8.2	Call Detail Record Notification



callDetailRecordNotification NOTIFICATION

BEHAVIOUR

      callDetailRecordReportBehaviour BEHAVIOUR

         DEFINED AS

	"This notification is issued to transmit a call detail record. The immediate notification may be used by the Event Forwarding Discriminator to select records requiring real time handling by the OS. Remaining attributes can be used to further filter the information in the record content.";;

   WITH INFORMATION SYNTAX Q825-CDR-ASN1Module.RecordContent

      AND ATTRIBUTE IDS

         recordType                              recordType,

         startTimeStamp                          startTimeStamp,

         participantInfo                         participantInfo,

         bearerService                           bearerService,

         serviceUser                             serviceUser,

         callIdentificationNumber                callIdentificationNumber,

         supplementaryServices                   supplementaryServices,

         immediateNotificationForUsageMetering   immediateNotificationForUsageMetering,

         cause                                   cause,

         iNSpecificInformation                   iNSpecificInformation,

         partialGeneration                       partialGeneration,

         exchangeInfo                            exchangeInfo,

         relatedCallNumber                       relatedCallNumber,

         cDRPurpose                              cDRPurpose,

         additionalParticipantInfo               additionalParticipantInfo,

         callingPartyCategory                    callingPartyCategory,

         callingPartyType                        callingPartyType,

         chargingInformation                     chargingInformation,

         progress                                progress,

         accessDelivery                          accessDelivery,

         trunkGroupOutgoing                      trunkGroupOutgoing,

         trunkGroupIncoming                      trunkGroupIncoming,

         fallbackBearerService                   fallbackBearerService,

         teleservice                             teleservice,

         callDuration                            callDuration,

         uUInfo                                  uUInfo,

         standardExtensions                      standardExtensions,

         recordExtensions                        recordExtensions,

         b-PartyCategory                         b-PartyCategory,

         iSUPPreferred                           iSUPPreferred,

         networkManagementControls               networkManagementControls,

         glare                                   glare,

         recordId                                recordId,

         dataValidaty                            dataValidity,

         callStatus                              callStatus,

         carrierId                               carrierId,

         dPC                                     dPC,

         oPC                                     oPC;

REGISTERED AS {q825Notification 2};



NOTE – The ASN.1 type references in the AND ATTRIBUTES IDS clause refers to all of the records that include this name.

�A.8.3	File Creation Notification



fileCreationNotification		NOTIFICATION

BEHAVIOUR

fileCreationNotificationBhv		BEHAVIOUR

	DEFINED AS "This notification is emitted whenever the fileGenerating Log creates a new file in order to let the managing system know that the file is available for retrieval.";;

WITH INFORMATION SYNTAX	Q825-CDR-ASN1Module.FileCreationInfo;

REGISTERED AS {q825Notification 3};

A.9	Name bindings

The name binding for simple usage metering control object class and configurable simple usage metering control object class and its subclasses is derived from Recommendation X.742. The name binding of the accountable object is to be defined during the implementation.



blockGeneratingLog-managedElement NAME BINDING

   SUBORDINATE OBJECT CLASS blockGeneratingLog AND SUBCLASSES;

   NAMED BY SUPERIOR OBJECT CLASS "ITU-T Rec. M.3100: 1995":managedElement AND

     SUBCLASSES;

   WITH ATTRIBUTE "ITU-T Rec. X.721 | ISO/IEC 10165-2 : 1992":logId;

   CREATE WITH-AUTOMATIC-INSTANCE-NAMING;

   DELETE DELETES-CONTAINED-OBJECTS;

REGISTERED AS {q825NameBinding 3};



callDetailData-managedElement NAME BINDING

   SUBORDINATE OBJECT CLASS callDetailData AND SUBCLASSES;

   NAMED BY SUPERIOR OBJECT CLASS "ITU-T Rec. M.3100: 1995":managedElement AND

   SUBCLASSES;

   WITH ATTRIBUTE callDetailDataId;

   BEHAVIOUR	

      callDetaildata BEHAVIOUR

	DEFINED AS

	"callDetailData objects are created and deleted implicitly, that is, they are created and deleted upon the occurrence of defined trigger events and are not explicitly manipulated by a managing system; i.e. a manager cannot perform any operations on instances of this object class. To support recording data in a callDetailData object two sets of triggers coming from the control objects are defined:

	–	Creation Triggers: events that cause creation of a callDetailData object,  these triggers are defined as part of the control object.

	–	Termination Triggers: events that cause deletion of a callDetailData object. Currently the termination is implicit as part of the data object behaviour. Deletion occurs upon completion of usage data collection for that instance of service.

	CDR notifications are emitted in response to reporting triggers (ITU-T  Recommendation X.742) that are also defined as part of the control objects.";;

REGISTERED AS {q825NameBinding 5};



callDetailLogRecord-blockGeneratingLog NAME BINDING

   SUBORDINATE OBJECT CLASS callDetailLogRecord AND SUBCLASSES;

   NAMED BY SUPERIOR OBJECT CLASS blockGeneratingLog AND SUBCLASSES;

   WITH ATTRIBUTE "ITU-T Rec. X.721 | ISO/IEC 10165-2 : 1992":logRecordId;

   BEHAVIOUR

      callDetailLogRecordToblockGenerationLog BEHAVIOUR

	DEFINED AS

	"instance of this managed class will be created automatically when the blockGeneratingLog receives a callDetailNotification matching the discriminating construct. The records are automatically deleted after the notification is emitted.";;

REGISTERED AS {q825NameBinding 1};



�callDetailLogRecord-fileGeneratingLog NAME BINDING

   SUBORDINATE OBJECT CLASS callDetailLogRecord AND SUBCLASSES;

   NAMED BY SUPERIOR OBJECT CLASS fileGeneratingLog AND SUBCLASSES;

   WITH ATTRIBUTE "ITU-T Rec. X.721 | ISO/IEC 10165-2 : 1992":logRecordId;

   BEHAVIOUR	

      callDetailLogRecordToFileGenerationLog BEHAVIOUR

	DEFINED AS

	"instance of this managed class will be created automatically when the fileGeneratingLog receives a callDetailNotification matching the discriminating construct. The records are automatically deleted after the records are copied to the file.";;

REGISTERED AS {q825NameBinding 2};



fileGeneratingLog-managedElement NAME BINDING

   SUBORDINATE OBJECT CLASS fileGeneratingLog AND SUBCLASSES;

   NAMED BY SUPERIOR OBJECT CLASS "ITU-T Rec. M.3100: 1995":managedElement AND

     SUBCLASSES;

   WITH ATTRIBUTE "ITU-T Rec. X.721 | ISO/IEC 10165-2 : 1992":logId;

   CREATE WITH-AUTOMATIC-INSTANCE-NAMING;

   DELETE DELETES-CONTAINED-OBJECTS;

REGISTERED AS {q825NameBinding 4};

A.10	ASN.1 defined types module



Q825-CDR-ASN1Module {itu-t(0) recommendation(0) q(17) q825(825) asn1Modules(2) q825ASN1Module(0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- EXPORTS  everything

IMPORTS

ObjectInstance FROM CMIP-1 { joint-iso-ccitt ms(9) cmip(1) modules(0) protocol(3) }

--see Rec. X.711

NameType FROM ASN1DefinedTypesModule {ccitt recommendation m gnm(3100) informationModel(0) asn1Modules(2) asn1DefinedTypesModule(0)}

-- see Rec. M.3100

ManagementExtension FROM

Attribute-ASN1Module { joint-iso-ccitt ms(9) smi(3) part2(2) asn1Module(2) 1}

--see Rec. X.721

PointCode FROM MTPDefinedTypesModule { itu-t recommendation q(17) omap(751) mtp(1) informationModel(0) mtpDefinedTypesModule(0) }

--see Rec. Q.751.1

ReportingTriggers FROM Schedule-ASN1Module { joint-iso-ccitt ms(9) function(2) part15(15) modules (1)};

--see Rec. X.742

-- OBJECT IDENTIFIERS

q825-InformationModel OBJECT IDENTIFIER ::= {itu-t(0) recommendation(0) q(17) q825(825) informationModel(0)}

q825ObjectClass OBJECT IDENTIFIER ::= {q825-InformationModel managedObjectClass(3)}

q825Package OBJECT IDENTIFIER ::= {q825-InformationModel package(4)}

q825NameBinding OBJECT IDENTIFIER ::= {q825-InformationModel nameBinding(5)}

q825Attribute OBJECT IDENTIFIER ::= {q825-InformationModel attribute(6)}

q825Action OBJECT IDENTIFIER ::= {q825-InformationModel action(7)}

q825Notification OBJECT IDENTIFIER ::= {q825-InformationModel notification(8)}



-- 	DEFAULT VALUE DEFINITION

defaultCreationTrigger CreationTriggerList::={}

-- 	Supporting productions

Integer ::= INTEGER

-- BLOCK CONTENTS



BlockRecordInfo			::= 	SEQUENCE	{

	blockHeaderRecord		[0]	BlockHeaderRecord		OPTIONAL,

	usageRecords			[1]	SEQUENCE OF RecordContent }

�

BlockHeaderRecord		::= 	SEQUENCE	{

	exchangeInfo			[0]	ExchangeInfo 			OPTIONAL,

	sequenceNumber			[1]	SequenceNumber,

	reasonForOutput			[2]	ReasonForOutput 			OPTIONAL,

	extensions				[3]	ManagementExtensions 	OPTIONAL }



-- FILE CONTENTS



FileHeaderRecord		::=		SEQUENCE	{

	productionDateTime		StartDateTime,

	exchangeInfo			ExchangeInfo,

	fileName				FileName,

	reasonForOutput			ReasonForOutput,

	firstRecordId			RecordId 			OPTIONAL,

-- Present if the requested first record id is different from what was requested in the created file request

	extensions				ManagementExtensions 	OPTIONAL}



Trailer				::=	SEQUENCE	{

	numberOfRecords			[0]	INTEGER,

	lastRecordId				[1]	INTEGER }



-- USAGE METERING RECORDS



RecordContent		::= 	CHOICE	{

callRecord 				[0]	CallRecord,

supplServiceInputRecord 		[1]	SupplServiceInputRecord,

standardAdditionalRecordTypes	[2]	ManagementExtensions,

additionalRecordTypes		[3]	ManagementExtensions }



CallRecord			::= 	SET	{

	recordType			[0]	RecordType,

	startTimeStamp		[1]	StartTimeStamp,

	participantInfo		[2]	ParticipantInfo,

	bearerService		[3]	BearerService,

	serviceUser			[4]	ServiceUser,

	callIdentificationNumber	[6] 	CallIdentificationNumber,

	supplementaryServices	[5]	SupplementaryServices	OPTIONAL,

	immediateNotificationForUsageMetering [7] ImmediateNotification 	OPTIONAL,

	cause				[8]	Cause				OPTIONAL,

	iNSpecificInfo		[9]	INSpecificInfo 		OPTIONAL,

	partialGeneration		[10]	PartialGeneration		OPTIONAL,

	exchangeInfo		[11]	ExchangeInfo		OPTIONAL,

	relatedCallNumber		[12]	RelatedCallNumber	OPTIONAL,

	cDRPurpose			[13]	CDRPurpose		OPTIONAL,

	additionalParticipantInfo	[14]	AdditionalParticipantInfo	OPTIONAL,

	callingPartyCategory	[15]	CallingPartyCategory	OPTIONAL,	

	callingPartyType		[16]	CallingPartyType 		OPTIONAL,

	chargingInformation	[17]	ChargingInformation 	OPTIONAL,

	progress			[18]	Progress			OPTIONAL,

	accessDelivery		[19]	AccessDelivery		OPTIONAL,

	trunkGroupOutgoing	[20]	TrunkGroupOutgoing	OPTIONAL,

	trunkGroupIncoming	[21]	TrunkGroupIncoming	OPTIONAL,

	fallbackBearerService	[22]	FallbackBearerService	OPTIONAL,

	teleservice			[23]	Teleservice			OPTIONAL,

	callDuration			[24]	CallDuration		OPTIONAL,

	uUInfo			[25] 	UUInfo			OPTIONAL,	

	standardExtensions		[26]	StandardExtensions	OPTIONAL,

	recordExtensions		[30]	RecordExtensions		OPTIONAL,

	b-PartyCategory		[31]	B-PartyCategory		OPTIONAL,

	iSUPPreferred		[32]	ISUPPreferred		OPTIONAL,  

�	networkManagementControls	[33]	NetworkManagementControls		OPTIONAL,

	glare				[34]	Glare				OPTIONAL,

	recordId 			[35]	RecordId			OPTIONAL,

	dataValidity			[36]	DataValidity			OPTIONAL, 

	callStatus			[37]	CallStatus			OPTIONAL,

	carrierId			[38]	CarrierId			OPTIONAL,

	dPC				[39]	PointCode			OPTIONAL,

	oPC				[40]	PointCode			OPTIONAL

	}



SupplServiceInputRecord 		::= 	CallRecord

	(WITH COMPONENTS {

	recordType			PRESENT,

	startTimeStamp		PRESENT,

	participantInfo		PRESENT,

	bearerService		PRESENT,

	serviceUser			PRESENT,

	callIdentificationNumber	PRESENT,

	supplementaryServices	PRESENT,

	immediateNotificationForUsageMetering	OPTIONAL,

	cause							OPTIONAL,

	iNSpecificInfo					OPTIONAL,

	exchangeInfo					OPTIONAL,

	cDRPurpose						OPTIONAL,

	additionalParticipantInfo				OPTIONAL,

	callingPartyCategory				OPTIONAL,

	callingPartyType					OPTIONAL,

	chargingInformation				OPTIONAL,

	standardExtensions					OPTIONAL,

	recordExtensions					OPTIONAL,

	recordId						OPTIONAL })



AccessDelivery 		::= 	BIT STRING  {

		setupMessageGenerated (0)} (SIZE(8))

--	Bit 0 (setupMessageGeneration) has the following meaning:

--		0	No set-up message generated

--		1	Set-up message generated

--	Bit 1 to 7 are not used.



AccountCodeInput		::=	OCTET STRING (SIZE (1..18))

--	This type is used to represent information, which is provided by the subscriber necessary for

--	use by some services.

--	a)	bits	876:	Encoding scheme

--			000	BCD even (even number of digits)

--			001	BCD odd (odd number of digits)

--			010	IA5 character

--			011	Binary coded

--			100

--			...	spare

--			111

--

�--	b)	bits	54321:	Type of digits

--			00000	reserved for account code

--			00001	reserved for authorization code

--			00010	reserved for private network travelling class mark

--			00011	reserved for business communication

--			00100

--			...	spare for international use

--			01111

--			10000

--			...	spare for national use

--			11111

--

--	c)	Digits:

--		Coding in accordance to the coding scheme and type of digits.



AdditionalParticipantInfo		::= 	SET {

physicalLineCode 				[0]	PhysicalLineCode 					OPTIONAL,

receivedDigits 				[1]	ReceivedDigits 					OPTIONAL,

operatorSpecific1AdditionalNumber 	[2]	OperatorSpecific1AdditionalNumber 	OPTIONAL,

operatorSpecific2AdditionalNumber 	[3]	OperatorSpecific2AdditionalNumber 	OPTIONAL,

operatorSpecific3AdditionalNumber 	[4]	OperatorSpecific3AdditionalNumber 	OPTIONAL}



Amount				::=	SEQUENCE {

					currencyAmount	[0]	NumberOfUnits,

					multiplier		[1]	Multiplier}



BearerService 			::= 	SEQUENCE {

			capability	ENUMERATED {

				speech (0),

				audio3dot1kHZ (1),

				uni64 (2),

				uni64withT-A (3),

				multipleRate (4),

				packetModeB-Ch (5) },

		multiplier INTEGER (2..30) OPTIONAL}

--	Multiplier present only if capability = multipleRate



B-PartyCategory 			::= 	BIT STRING(SIZE(8))



CallStatus				::=	 ENUMERATED {

				answered	(0),

				notanswered 	(1) }



CallDuration				::=	SET {

	conversationTime		[0] 	ConversationTime 		OPTIONAL,

	durationTimeACM		[1] 	DurationTimeACM 	OPTIONAL,

	durationTimeB-ans		[2] 	DurationTimeANM 	OPTIONAL,

	durationTimeNoANM	[3] 	DurationTimeNoANM 	OPTIONAL }



CalledPartyNumber			::=	Number



CallIdentificationNumber		 ::=	OCTET STRING

--	Octet string identifying the call.



CallingPartyCategory 		::= 	BIT STRING(SIZE(8))



CallingPartyNumber			::=	Number

CallingPartyNumberNotScreened	::=	Number



�CallingPartyType 			::= 	ENUMERATED {

		analogue (0),

		customerLink (1),		-- 	2Mbit/s PSTN digital access

		basicAccess (2),

		primaryRateAccess (3) }

CarrierId					::=	VisibleString (SIZE(1..11))

Cause						::=	SEQUENCE {

		causeValue      		CauseValue,

		location			Location}



CauseValue 					::=	BIT STRING (SIZE(8))

--	Coded according to ITU-T Recommendation Q.850, Table 1/Q.850



CDRPurpose					::=	BIT STRING {

			usagemetering (0),

			analysis (1) }  (SIZE(2) )

	

ChargedDirectoryNumber			::=	Number



ChargingInformation			::= 	CHOICE {

	recordedCurrency			[0]	RecordedCurrency,

	recordedUnitsList			[1]	RecordedUnitsList,

	freeOfCharge			[2] 	NULL,

	chargeInfoNotAvailable		[3]	NULL}



ChargedParticipant		::= 	ParticipantType



ConfigurationMask		 ::= 	BIT STRING {

		exchangeInfo			(0),

		relatedCallNumber			(1),

		additionalParticipantInfo		(2),

		callingPartyCategory		(3),

		callingPartyType			(4),

		progress				(5),

		accessDelivery			(6),

		trunkGroupOutgoing		(7),

		trunkGroupIncoming		(8),

		teleservice				(9),

		standardExtensions			(10),

		recordExtensions			(11),

		b-partyCategory			(12),	

		iSUPPrefferred			(13),

		networkManagementControls	(14),

		glare					(15)	 }



ConversationTime			::=	Duration



Count 					::= 	OCTET STRING (SIZE (1..3))

--	A maximum 3-byte counter.



CreateFileResponse			::= 	SEQUENCE {

		fileName	GraphicString,

		fileSize	INTEGER OPTIONAL }

--  	number of octets in file.



CreationTriggerList 			::= 	SET OF CreationTrigger



CreationTrigger			::= 	ENUMERATED   {

		seizure(0),

		firstDigitReceived(1),

�		aCMReceived(2),

		aNMReceived(3),

		supplementaryServiceInvocation(4),

		supplementaryServiceInput(5) }

		

--	ACMreceived is defined as the receipt of ACM for an external call. In the case of a

--	terminating exchange the event ACM received will occur when the ACM is normally

--	generated. This is either when a B-subscriber port is free (early ACM) or when

--	a B-subscriber terminal is free (late ACM).

--	The choice between early and late ACM is a network option in the terminating network.

--	 For local calls a corresponding time shall be used.



DataValidity			::=  	ENUMERATED {

		possibleduplicated (0),

		requireddatamissing (1),

		other			(2) }



Duration				::=	OCTET STRING (SIZE(1..3))

--	Duration in centiseconds. Maximum value in centiseconds is approx. 46 hours.



DurationTimeACM 			::= 	Duration



DurationTimeANM			::= 	Duration  --fix to match reference syntax



DurationTimeNoANM			::= 	Duration  --fix to match reference syntax



ExchangeInfo				::=  	SET {

		exchangeID			[0] 	ExchangeID		OPTIONAL,

         		softwareVersion		[1]	SoftwareVersion	OPTIONAL}



ExchangeID 				::= 	VisibleString (SIZE (1..11))



FallbackBearerService		::= 	BearerService



FileName 				::= 	NameType



FileCreationInfo			::=	SEQUENCE{

		fileName		FileName,

		reasonForOutput	ReasonForOutput}



Glare					::= 	BOOLEAN

ImmediateNotification		::= 	BOOLEAN



INServiceInformationList		::=	SEQUENCE OF INServiceInformation



INServiceInformation		::= 	SEQUENCE {

				inServiceCode	[0] INServiceCode,

				queueInfo		[1] QueueInfo 	OPTIONAL,

				serviceSpecificINInformation	[2] OCTET STRING OPTIONAL}



INSpecificInfo			::=	SET {

	personalUserId				[0] PersonalUserId					OPTIONAL,

	chargedParticipant				[1] ChargedParticipant 			OPTIONAL,

	chargedDirectoryNumber			[2] ChargedDirectoryNumber 		OPTIONAL,

	percentageToBeBilled			[3] PercentageToBeBilled 			OPTIONAL,

	accountCodeInput				[4] AccountCodeInput 			OPTIONAL,

	iNServiceCode				[5] INServiceCode 					OPTIONAL,

	queueInfo					[6] QueueInfo	 				OPTIONAL,

	serviceSpecificINInformation		[7] ServiceSpecificINInformation 		OPTIONAL }



�INServiceCode 			::= 	OCTET STRING (SIZE (2))



ISUPPreferred			::= 	ENUMERATED {

		preferred	(0),

		notrequired	(1),

		required	(2),

		notapplicable	(3)}



Location				::= 	INTEGER {

		user (0),

		localUserPrivateNetwork (1),

		localUserPublicNetwork (2),

		transitNetwork (3),

		remoteUserPublicNetwork (4),

		remoteUsePrivateNetwork (5),

		internationalNetwork (7),

		beyondInterworkPoint (10) }

--	See  ITU-T Recommendation Q.850, 2.2.3



MaxBlockSize			::= 	INTEGER (0..32767)



MaxTimeInterval			::=	INTEGER (0..32767)

--	time interval in seconds.



ManagementExtensions		::=	SET OF ManagementExtension

-- 	A set of network/manufacturer specific extensions.



Multiplier				::=	ENUMERATED {

		oneThousandth	(0),

		oneHundredth	(1),

		oneTenth		(2),

		one			(3),

		ten			(4),

		hundred		(5),

		thousand		(6)}



NetworkManagementControls	::= 	ENUMERATED {

		acc				(0),

		adc				(1),

		cancelFrom			(2),

		cancelRerouted		(3),

		cancelTo			(4),

		destinationCodeControl	(5),

		scr				(6),

		skip				(7),

		tarfrom			(8),

		tarto				(9)}



NetworkProviderId			::=	 VisibleString (SIZE(1..11))

Number				::=	OCTET STRING (SIZE (1 .. 14))

-- 	This type is used to represent a number for addressing purposes. It is composed of:

-- 	a)	one octet for odd/even indicator and nature of address indicator:

--			bits	8:		Odd/even indicator

--				0		even number of address signals

--				1		odd number of address signals



-- 			bits	7654321:	Nature of address indicator

--				0000000	spare

--				0000001	subscriber number

--				0000010	unknown

--				0000011	national (significant) number

�--				0000100	international number

--				0000101	(

--				to	(	spare

--				1101111	(

--				1110000	(

--				to	(	reserved for national use

--				1111110	(

--				1111111	spare

--	b)	one octet for numbering plan indicator:

--			bits	765:		numbering plan indicator

--				000		spare

--				001		ISDN (Telephony) Number Plan (ITU-T Recommendation E.164)

--		 		010		spare

--				011		data numbering plan (ITU-T Recommendation  X.121)

--				100		telex numbering plan (ITU-T Recommendation  F.69)

--				101		reserved for national use

--				110		reserved for national use

--				111		spare

--	c)	digits of the address encoded as TBCD String:

--      	The following octets representing digits of an address encoded as a TBCD-STRING.

--	TBCD-STRING		::=	OCTETSTRING

--	This type (Telephony Binary Coded Decimal String) is used to represent digits from 0

--	through 9, *, #, a, b, c, two digits per octet, each digit encoded 0000 to 1001 (0 to 9),

--	1010 (*) 1011(#), 1100 (a), 1101 (b) or 1110 (c); 1111 (end of pulsing signal-ST); 0000 is

--	used as a filler when there is an odd number of digits.

--	The most significant address signal is sent first. Subsequent address signals are sent in

--	successive 4-bit fields.



NumberOfUnits 				::=	INTEGER (0..16777215)



OperatorSpecific1AdditionalNumber	::=	VisibleString



OperatorSpecific2AdditionalNumber	::=	VisibleString



OperatorSpecific3AdditionalNumber	::=	VisibleString



OperatorSpecific1Number		::=	Number



OperatorSpecific2Number		::=	Number



OperatorSpecific3Number		::=	Number



OriginalCalledNumber			::=	Number



ParticipantId				::=	CHOICE {

		callingPartyNumber [0]			CallingPartyNumber,

		calledPartyNumber [1] 			CalledPartyNumber,

		redirectingNumber [2]			RedirectingNumber,

		redirectionNumber [3] 			RedirectionNumber,

		originalCalledNumber [4]			OriginalCalledNumber,

		callingPartyNumberNotScreened [5]	CallingPartyNumberNotScreened,

		operatorSpecific1Number [6] 		OperatorSpecific1Number,

		operatorSpecific2Number [7] 		OperatorSpecific2Number,

		operatorSpecific3Number [8] 		OperatorSpecific3Number}



ParticipantInfo			::=	SET OF ParticipantId



ParticipantType			::= 	ENUMERATED {

		callingPartyNumber (0),

		calledPartyNumber (1),

		redirectingNumber (2),

�		redirectionNumber (3),

		originalCalledNumber (4),

		callingPartyNumberNotScreened (5),

		operatorSpecific1Number (6),

		operatorSpecific2Number (7),

		operatorSpecific3Number (8),

		operator (9),

		unknown(10)}



PartialRecordNumber	 	::= 	BIT STRING (SIZE (8))

--	A sequential number in the range 0-255 indicating the partial record generated for the same call



PartialGeneration 			::= 	SET  {

       	  	partialRecordNumber	[0]	PartialRecordNumber,

		partialRecordReason 	[1] 	PartialRecordReason}



PartialRecordReason 		::= 	ENUMERATED {

		timeLimit			(0),

--	This is used for long duration calls.

		serviceChange		(1),

		overflow			(2),

		networkInternalReasons	(3),

		lastCDR			(4),

		timeChange			(5)}



Period				::= 	INTEGER (0..512) --   Elapsed time in minutes



PercentageToBeBilled		::=	INTEGER (0..99)



PersonalUserId		::=	OCTET STRING (SIZE (1..10))

-- 	This type is used to represent the Personal User Id. For UMT the Personal User Id is defined

--	according to E.212 as an International Mobile Station Identity (IMSI). Accordingly only numerical

--	characters (0-9) are used.

--	The PersonalNumber type does however not exclude the use of other formats. These formats

--	can be indicated in the numbering plan indicator.

--	The type is composed of:

--	a)	one octet for odd/even indicator and numbering plan indicator:

--			bits	8:	Odd/even indicator

--				0	even number of address signals

--				1	odd number of address signals

--			bits	765:	numbering plan indicator

--				000	E.212 (IMSEI)

--				001	ISDN (Telephony) Number Plan (ITU-T Recommendation E.164)

--				010	spare

--				011	spare

--				100	spare

--	b)	digits of the address encoded as TBCD String:

--	The following octets representing the personal number encoded as a TBCD-STRING.

-- 	TBCD-STRING	::=	OCTET STRING

--	This type (Telephony Binary Coded Decimal String) is used to represent digits from 0

--	through 9, *, #, a, b, c, two digits per octet, each digit encoded 0000 to 1001 (0 to 9),

--	1010 (*), 1011(#), 1100 (a), 1101 (b) or 1110 (c); 1111 (end of pulsing signal-ST); 0000 is

--	used as a filler when there is an odd number of digits.

--	The most significant address signal is sent first. Subsequent address signals are sent in

--	successive 4-bit fields.



PhysicalLineCode 			::=	VisibleString



Progress				::=	 SEQUENCE {

		description 		ProgressDescription,

		location		Location }

�

ProgressDescription 			::=	 INTEGER {

		notEndToEndISDN (1),

		nonISDNDestination (2),

		nonISDNOrigination (3),

		returnedToISDN (4),

		interworkingServiceChange (5),

		inBandInfo (8) }



QueueInfo				::=	SEQUENCE{

		queueTimeStamp		[0]	StartDateTime,

		queueDuration		[1]	Duration}



ReasonForOutput			::= 	ENUMERATED {

		absoluteTimeEvent		(0),

		maxBlockSizeReached	(1),

		maxTimeIntervalElapsed	(2),

		internalSizeLimitReached	(3),

		oSAction			(4)}



ReceivedDigits			::=	OCTET STRING (SIZE (1 .. 18))

--	This type is used to represent digits input by the subscriber. It is composed of:

--	a)	one octet for odd/even indicator:

--			bits	8:	Odd/even indicator

--				0	even number of address signals

--				1	odd number of address signals

--	b)	digits of the address encoded as TBCD String

--      	The following octets representing the received digits encoded as a TBCD-STRING.

-- 	TBCD-STRING	::=	OCTET STRING

--	This type (Telephony Binary Coded Decimal String) is used to represent digits from 0

--	through 9, *, #, a, b, c, two digits per octet, each digit encoded 0000 to 1001 (0 to 9),

--	1010 (*), 1011(#), 1100 (a), 1101 (b) or 1110 (c); 1111 (end of pulsing signal-ST);

--	0000 is used as a filler when there is an odd number of digits.

--	The most significant address signal is sent first. Subsequent address signals are sent in

--	successive 4-bit fields.

RecordedCurrency			::=	CHOICE {

		currency		[0]	IA5String (SIZE (1..10)),

		amount		[1]	Amount	}



RecordExtensions			::=	ManagementExtensions



RecordedUnitsList			::=	SEQUENCE SIZE (1.. 32) OF RecordedUnits



RecordedUnits			::=	SEQUENCE{

			units	CHOICE {

			recordedNumberOfUnits [0]	NumberOfUnits,

			notAvailable [1]			NULL },

			recordedTypeOfUnits		INTEGER(1..16) OPTIONAL }



RecordId			::=	Count

--The record Id is a sequence number that is incremented for each logged CDR,

--it is generated by the log



RecordType			 ::= 	INTEGER {

		call (0),

		supplServiceInputRecord(1) }

RedirectingNumber		::=	Number



RedirectionNumber		::=	Number



RelatedCallNumber		::= 	CallIdentificationNumber

�

ServiceSpecificINInformation	::=	ManagementExtensions



ServiceUser			::=	ParticipantType



SequenceNumber		::= 	Count

-- The record block sequence number is incremented for each block generation



SoftwareVersion 		::= 	VisibleString (SIZE(1..12))



StandardExtensions		::=	ManagementExtensions



StartTimeStamp		::=	CHOICE {

		answerTime 		[0]		StartDateTime,

		seizureTime 		[1]		StartDateTime,

--	For calls a choice between seizure time or answer time

--	is dependent on the occurrence of a B-answer (ANM).

		partialTime		[2]		StartDateTime,

--	Partial time is used for partial records.

		eventTime 		[3]		StartDateTime}

--	Event time is used in connection with supplementary service input  records.



StartDateTime 				::= 	OCTET STRING (SIZE(7))

--	YYMMDDHHmmSSCC (Year, Month, Day, Hour, Minute, Second, Centisecond),

--	each field one digit, two digits per octet, the digits 0 through 9, encoded as

--	0000 to 1001 "hstring". 1st digit in the LSB.



SupplementaryServices	::= 	SEQUENCE OF SupplementaryService



SupplementaryService	::= 	SEQUENCE {

		supplementaryServiceCode	SupplementaryServiceCode,

		supplementaryAction		SupplementaryAction,

		supplementarytimestamp		Duration 				OPTIONAL,

		functionalInformation		ManagementExtensions	 OPTIONAL }



SupplementaryServiceCode 	::= 	OCTET STRING (SIZE (2))



SupplementaryAction 		::=  	ENUMERATED {

		provision (0),

		withdrawal (1),

		registration (2),

		erasure (3),

		activation (4),

		deactivation (5),

		invocation (6),

		disabling (7),

		interrogation (8)}



Teleservice 				::=	BIT STRING (SIZE(8))



TrunkGroupIncoming 		::=	TrunkGroupId

TrunkGroupOutgoing 		::=	TrunkGroupId



TrunkGroupId			::=	SEQUENCE {

		trunkGroupId		[0] NameType,

		trunkId			[1] NameType	OPTIONAL,

		pCMId			[2] NameType	OPTIONAL,

		channelNumber		[3] INTEGER	OPTIONAL}



�TimesOfDay				::= 	OCTET STRING (SIZE(2))

-- 	HHmm (Hours, Minutes)

--	each field one digit, two digits per octet, the digits o through 9 encoded as

--	0000 to 1001 "hstring", 1st digit in the LSB



CallDetailDataId			::=	NameType





UUInfo				::= 	SET {

		uu1Info		[0]	UUxInfo		OPTIONAL,

		uu2Info		[1]	UUxInfo		OPTIONAL,

		uu3Info		[2]	UUxInfo		OPTIONAL}



UUxInfo				::= 	SET {

		receivedMessages		[0]	Count		OPTIONAL,

		transmittedMessages	[1]	Count		OPTIONAL,

		receivedOctets		[2]	Count		OPTIONAL,

		transmittedOctets		[3]	Count		OPTIONAL}

	

--Subtype to use with  permitted values for reporting triggers from usage metering



PermittedReportingTriggers ::= ReportingTriggers ((WITH COMPONENT( WITH COMPONENTS {periodic PRESENT} ))  (SIZE(0..1)))



-- Default value for subtyped attribute



reportingTriggersDefault  PermittedReportingTriggers ::= { periodic : minutes : 30 }

 



END	-- of Q.825-CDR-ASN1Module 

Annex B

Call Detail  Records

B.1	General

This Annex includes a specification of the record types and the information elements valid for inclusion in the CDR. To enable for a more flexible formatting of the CDR, dynamic encoding rules shall be adopted. The rules imply that the CDR and each information element has a tag and length parameter for identifying the type and length of the information.

The main advantage of this approach is that only relevant data is generated. To enhance this feature, it shall be possible using MML commands to control the number of information elements in the CDR. The control of the number of information elements is not included in this specification.

B.1.1	Use of record types

As the records may be generated under different conditions and therefore may also include different information elements, specific record types have been defined. The following two record types are defined for the purpose of this Recommendation:

•	call;

•	supplementary service input record.

Two extra types are however provided so that standardized (standardAdditionalTypes) or operator specific (additionalRecordTypes) types can be added. The record type may have impact on the way the OS interprets the information elements included in the record. The control to select from the �potential recordable events, the events for which a record should be generated, is outlined in 8.1. It should be noted that the absence of certain trigger criteria could lead to the exclusion of a record type as generation is dependent on the criteria selected.

B.1.2	Partial records

In order to increase the security of the recording process and to simplify post-processing, it may be desirable to generate a sequence of call records to describe the service usage. In the case of connections of extended duration, loss of a single record may result in an unacceptable loss of revenue. If a record is only produced after extended duration, employment of credit checking by the billing system will be impossible. The periodic timer provided for the generation of partial CDRs is outlined in 8.1.

All of the records defined in this specification are of variable length and some are potentially unlimited in size. However, due to internal limitations in the NEF, partial records may be required to circumvent internal resource limitations.

All partial records for the same call shall contain the same call identification number (see Recommendation Q.931) and shall be ordered via a running sequence number. The time stamps involved shall apply to the individual partial records rather than the call as a whole, i.e. the "end" time stamp (StartTimeStamp + duration) of a record shall coincide with the "start" time stamp of the next. Each time a new partial record is created the reason for partial generation may be included in the partial generation element. The partial records generated may repeat each of the non-varying fields contained in the initial record. Alternatively, a reduced partial record may be generated which includes only those fields required to identify the initial record together with the field(s) that actually change.

B.1.3	Use of supplementary services

There are basically two kinds of supplementary service actions, call related and non-call related. The non-call related action is recorded in the supplementary service input type record. For the call related supplementary action, the call record type is used. Supplementary services that are not time- or duration-dependent for charging will not include the optional time stamp.

B.2	Record contents

Table B.1 describes the contents of each of the record types defined in this specification. 

It should be noted that the elements are ordered in the CDR as indicated in the table. The mandatory elements are grouped in the first part of the CDR and will therefore be easy to recognize as a fixed part of the record. To enable the OS to access and retrieve the CDR data elements from the fileGeneratingLog, each of the above elements are defined as attributes in A.6. For a description of the elements please refer to the defined by statements of that section.

Each element in the table contains the name of the information element and a key indicating whether or not the field is mandatory. The key has the following meaning:

–	This element is not relevant for this record type.

M	This element is mandatory and always present.

C	This element is only available under certain conditions. Under these conditions the field is mandatory.

O	This element is optional and configurable either via additional TMN management  functions or using MML commands. For clarification of doubt, optional does not mean that the parameter is not supported by the network element.

�Table B.1/Q.825 – Contents of the record types

Information element�Call�Supplementary service input��record type�M�M��start time stamp�M�M��participant Info�– calling party number�– called party number�– redirecting number�– redirection number�– original called number�– calling party number not screened�– operator specific1 number�– operator specific2 number�– operator specific3 number�M�M��bearer service �M�M��service User�C�M��call identification number�M�M��record Id�O�O��data Validity�C�C��networkProvidedId�C�C��supplementary services�C�M��immediate notification�C�C��cause�C�C��IN specific info�•	personal user identification�•	charged participant�•	charged directory number�•	percentage to be billed�•	account code input�•	IN service code�•	queue info�•	service specific IN information�C�C��partial generation�C�–��exchange Info�O�O��related call number�O�–��CDR purpose�O�O��additional participant Information�•	physical line code�•	received digits�•	operator specific1 additional number�•	operator specific2 additional number�•	operator specific3 additional number�O�O��calling party category�O�O��calling party type�O�O���Table B.1/Q.825 – Contents of the record types (concluded)

Information element�Call�Supplementary service input��charging information�O�O��progress�O�–��access delivery�O�–��trunk group outgoing�O�–��trunk group incoming�O�–��fallback bearer service �O�–��teleservice�O�–��call Duration�•	conversation time�•	duration time ACM�•	duration time ANM�•	duration time no ANM�O�–��user-to-user info counter�O�–��standard extensions�O�O��record extensions�O�O��call status�O�–��CarrierId�C�–��DPC�O�–��OPC�O�–��appendix I

Alternative ASN.1 definitions

I.1	Definition of ASN.1 modules for use of 1994 version of ASN.1

The use of the 1994 version of ASN.1 (Recommendations X.680, X.681, X.682 and X.683) is illustrated by providing an ASN.1 module, ASN1DefinedTypesModuleNew, which defines the data types that are affected by the new notation in a way similar to the way they are defined in the ASN1DefinedTypesModule module in A.10.



ASN1DefinedTypesModuleNew {ccitt(0) identified-organisation(4) etsi(0)        usageMeteringInformationManagement(43321) informationModel(0) asn1Modules(2)        ASN1DefinedTypesModuleNew(1)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- EXPORTS everything



-- The following ASN.1 provides the means for extending records, record types, IN specific information�-- and record block information. The Management Extension type is redefined using the useful�-- information object class type TYPE-IDENTIFIER. The construct is equivalent to ITU-T Recommendation X.721:�-- Attribute -ASN1Module ManagementExtension type and does not use the ANY DEFINED BY construct �-- which is not supported any more by ITU-T Recommendation  X.680.



�MANAGEMENT-EXTENSION    ::=     TYPE-IDENTIFIER�

AdditionalRecordType	 ::= INSTANCE OF MANAGEMENT-EXTENSION ({AllowedAdditionalRecordTypes})



-- The AllowedAdditionalRecordTypes is the constraint that allows only certain types to be set as�-- AdditionalRecordType.



AdditionalRecordTypes	 ::= SET OF AdditionalRecordType



-- The AdditionalRecordTypes type is to be used in the RecordContent type of �-- ASN1DefinedTypesModule module.



BlockExtension		 ::= INSTANCE OF MANAGEMENT-EXTENSION ({AllowedBlockExtensions})



-- The AllowedBlockExtensions is the constraint that allows only certain types to be set as BlockExtension.



BlockExtensions 	::= SET OF BlockExtension



-- The BlockExtensions type is to be used in the BlockHeaderRecord type of 

-- ASN1DefinedTypesModule

-- module



RecordExtension	 ::= INSTANCE OF MANAGEMENT-EXTENSION ({AllowedRecordExtensions})



-- The AllowedRecordExtensions is the constraint that allows only certain types to be set as

-- RecordExtension



RecordExtensions ::= SET OF RecordExtension



-- The RecordExtensions type is to be used in the CallRecord type of

-- ASN1DefinedTypesModule module



ServiceSpecificINInformation ::= INSTANCE OF MANAGEMENT-EXTENSION

                                        ({AllowedServiceSpecificINInformations})



-- The AllowedServiceSpecificINInformations is the constraint that allows only certain types to be set as

-- ServiceSpecificINInformation.



ServiceSpecificINInformations ::= SET OF ServiceSpecificINInformation



-- TheServiceSpecificINInformations type is to be used in the CallRecord type of

-- ASN1DefinedTypesModule module.



StandardAdditionalRecordType ::= INSTANCE OF MANAGEMENT-EXTENSION

                                        ({AllowedStandardAdditionalRecordTypes})



-- The AllowedStandardAdditionalRecordTypes is the constraint that allows only certain types to be set as

-- StandardAdditionalRecordType.



StandardAdditionalRecordTypes ::= SET OF StandardAdditionalRecordType



-- TheStandardAdditionalRecordTypes type is to be used in the RecordContent type of

-- ASN1DefinedTypesModule module.



StandardExtension 	::= INSTANCE OF MANAGEMENT-EXTENSION

                                        ({AllowedStandardExtensions})



-- The AllowedStandardExtensions is the constraint that allows only certain types to be

-- set as StandardExtension.



�StandardExtensions 	::= SET OF StandardExtension



-- TheStandardExtensions type is to be used in the CallRecord type of

-- ASN1DefinedTypesModule module

-- Usually the constraint on the type to be used for extensions is not known and must be specified

-- at implementation time (in the Protocol Implementation Conformance Statement, PICS).

-- However, in the case of standard extensions the allowed types for the constraint may be already defined.

-- Example:

-- TypeA and TypeB types are to be used as a constraint to StandardAdditionalRecordTypes.

-- The AllowedStandardAdditionalRecordType will be then:

-- AllowedStandardAdditionalRecordTypes STANDARD-EXTENSION ::= {

--                      { TypeA IDENTIFIED BY objectIdentifierA } |

--                      { TypeB IDENTIFIED BY objectIdentifierB} }

-- where objectIdentifierA and objectIdentifierB are the OBJECT IDENTIFIER values associated

-- to TypeA and TypeB respectively.



END  -- End of ASN1DefinedTypesModuleNew

I.2	Rules of extensibility

NOTE – This paragraph substitutes the paragraph on rules of extensibility at the start of A.10.

Recommendation X.680, Amendment 1, shall be used to extend the following types defined in ASN1UsageInformationModule module:

•	CallRecord.

•	RecordContent.

•	AdditionalParticipantInfo.

•	CreationTrigger.

•	INSpecificInfo.

•	ParticipantId.

•	ParticipantType.

•	PartialRecordReason.

•	ReasonForOutput.

•	StartTimeStamp.

•	TimeDuration.

•	CDRPurpose.

I.3	Use of Management Extension

The use of Management Extensions requires the definition of an object identifier that uniquely refers to the type being used for the extension. This definition avoids any ambiguity when receiving the type at the Q3 interface.

Example

The information model defined in this Annex is, for instance, to be used to collect usage information from ATM switches. An ATM connection record is standardized in an TMSwitchASN1Module in another document. The definition of this type in that document should be done as follows:



ATMSwitchASN1Module  { ccitt(0) identified-organisation(4) etsi(0) ATMSwitchStandard(XXX) informationModel(0) asn1Module(2) ATMSwitchASN1Module(0) }

DEFINITIONS ....

...

�ExampleATMConnectionRecordType    OBJECT IDENTIFIER       ::=

                        {ATMSwitchASN1Module ExampleATMConnectionRecordType(0) }

-- This object identifier uniquely identifies type ExampleATMConnectionRecord.

...

ExampleATMConnectionRecord      ::=     -- Type definition

...

When the usage metering information management model with a ManagementExtension is used, the object identifier ExampleATMConnectionRecordType should be used so that the ANY DEFINED BY clause in the Management Extension unambiguously determines the ExampleATMConnectionRecord type.

The ExampleATMConnectionRecord must be specified in the Protocol Implementation Conformance Statement (PICS) of the information model at the time of implementation.

appendix ii

Application of the model

Figure II.1 shows a typical configuration that can be used to generate files for analysis purposes on a sample basis and billing usage records for only successful calls. Note that this configuration will collect records at the specified sample rate for both complete and failed calls, generate usage records for only completed successful calls and generate real-time bills for successful calls. All other CDR notifications will be discarded and not logged.

� EMBED Designer.Drawing.7  ���

Figure II.1/Q.825

�APPENDIX III

Examples of the use of Call Detail Records

Table III.1 – Example of a transit exchange record for accounting allocation

Parameter name���Event Information���Record Type�Call Record��Start Time Stamp�M��Participant Information�M��Bearer Service�M��Service User�M��Call Identification Number�M��Supplementary Service List�O��Immediate Notification for Usage Metering�–��DPC�–��OPC�–��Call Status�–��Carrier Id�–��Cause�–��Data Validity�–��IN Specific Information�–��Service Specific IN Information List�–��Partial Generation���Exchange Information�U��Related Call Number�U��CDR Purpose�U��Additional Participant Information�U��Calling Party Category�–��Calling Party Type�–��Charging Information�–��Progress�–��Access Delivery�–��Trunk Group Incoming�xxxxx  ��Trunk Group Outgoing�yyyyy��Network Provider Id�–��Fallback Bearer Service�–��Teleservice�–��Call Duration�C��User-to-User Information Counter �U��Standard Extensions�U��Record Extension�U���APPENDIX IV

Application of CDR (for information only)

Table IV.1 shows how a given CDR may be used for QoS (Quality of Service), Network Management (NM) and Fraud investigation (Fraud). The last column explains how the QoS, NM, Fraud requirements are covered by the model presented in Recommendation Q.825.

Table IV.1/Q.825 – Application of CDR to the quality of service,�network management and fraud investigation

CDR content & derived information�QoS�NM�Fraud�Covered in Q.825 ?��1)	A-party number�X�X�X�Yes��2)	A-party category�X��X�Yes��3)	B-party number�X�X�X�Yes��4)	B-party category�X��X�Yes��5)	Incoming Circuit ID�X�X�X�No, but Trunk Group ID��6)	Outgoing Circuit ID�X�X�X�No, but Trunk Group ID��7)	Date for start of charging�X��X�No, not needed as per Q.4/2��8)	Time for start of charging�X��X�No, not needed as per Q.4/2��9)	Time for end of charging�X��X�No, not needed as per Q.4/2��10)	Charge duration�X��X�No, not needed as per Q.4/2��11)	Tariff Class�X��X�No, not a unique identifiable field��12)	Exchange ID�X�X�X�Yes��13)	Fault codes (Cong., Tech fault,�COTF, etc.)�X�X�X�Yes, partly in Cause field��14)	Answer�X�X�X�Yes��15)	Station Busy�X�X�X�Yes, in Cause field (Rec. Q.850)��16)	ISUP preferred/ISUP required�X��X�Yes��17)	ISDN user-network information�X��X�No��18)	Bearer capability�X���Yes��19)	Cause values TUP & ISUP (NNC,�ADI, UNN, etc.)�X�X�X�Yes, in Cause field (Rec. Q.850)��20)	User-to-User�X��X�Yes��21)	Holding time�X��X�Yes��22)	Actual Conversation Time�X���Yes��23)	ISDN flag�X��X�Derivable from Bearer Services, and Supplementary Service��24)	Network management controls�X��X�Yes��25)	Glare (dual seizure)�X���Yes��26)	CLI�X��X�Yes��27)	All Trunks Busy�X���Yes, in Cause field (Rec. Q.850)��28)	Vacant Codes�X���Yes, in Cause field (Rec. Q.850)���Table IV.1/Q.825 – Application of CDR to the quality of service,�network management and fraud investigation (concluded)

CDR content & derived information�QoS�NM�Fraud�Covered in Q.825 ?��29)	Closed user group�X���Derivable from ParticipationInfo��30)	Ringing abandoned call�X��X�Yes, through causeValue��31)	G4 FAX�X���No��32)	Non-circuit related traffic (MAP, OMAP, INAP, etc.)�X���No, Rec. Q.752 covers it��33)	Incoming trunk group��X�X�Yes��34)	Outgoing trunk group��X�X�Yes��35)	Date and Time of Incoming BID��X�X�Not needed as per SG 2 liaison��36)	Failed due to ISUP Preference Indicator��X��No, not recognizable as a unique call failure cause��37)	Failed due to Transmission Requirements��X��Yes, through causeValue��38)	Service Type (Rec. E.412)��X�X�Derivable, supplementaryService or bearerService��39)	Internal Congestion��X��Yes, in Cause field (Rec. Q.850)��40)	External Congestion��X��Yes, in Cause field (Rec. Q.850)��41)	Subscriber faults��X��Yes, in Cause field (Rec. Q.850)��42)	Other (Vendor specific)����Open��

�

ITU-T  RECOMMENDATIONS  SERIES��Series A�Organization of the work of the ITU-T��Series B�Means of expression: definitions, symbols, classification��Series C�General telecommunication statistics��Series D�General tariff principles��Series E�Overall network operation, telephone service, service operation and human factors��Series F�Non-telephone telecommunication services��Series G�Transmission systems and media, digital systems and networks��Series H�Audiovisual and multimedia systems��Series I�Integrated services digital network��Series J�Transmission of television, sound programme and other multimedia signals��Series K�Protection against interference��Series L�Construction, installation and protection of cables and other elements of outside plant��Series M�TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits��Series N�Maintenance: international sound programme and television transmission circuits��Series O�Specifications of measuring equipment��Series P�Telephone transmission quality, telephone installations, local line networks��Series Q�Switching and signalling��Series R�Telegraph transmission��Series S�Telegraph services terminal equipment��Series T�Terminals for telematic services��Series U�Telegraph switching��Series V�Data communication over the telephone network��Series X�Data networks and open system communications��Series Y�Global information infrastructure��Series Z�Programming languages��

____________________



















� PAGE �38�	Recommendation Q.825    (06/98)



�PAGE  �2�





		Recommendation Q.825    (06/98)	� PAGE �39�



















Printed in Switzerland

Geneva, 1998



Printed in Switzerland

Geneva, 1998


















