
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION


	ITU-T
	Series Q

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Supplement 11

(12/1999)   


SERIES Q: SWITCHING AND SIGNALLING

Technical Report TRQ.2010: B-ISDN signalling interworking requirements

ITU-T  Q-series  Recommendations – Supplement 11

(Formerly  CCITT  Recommendations)
ITU-T  Q-SERIES  RECOMMENDATIONS

SWITCHING AND SIGNALLING

	
	

	SIGNALLING IN THE INTERNATIONAL MANUAL SERVICE
	Q.1–Q.3

	INTERNATIONAL AUTOMATIC AND SEMI-AUTOMATIC WORKING
	Q.4–Q.59

	FUNCTIONS AND INFORMATION FLOWS FOR SERVICES IN THE ISDN
	Q.60–Q.99

	CLAUSES APPLICABLE TO ITU-T STANDARD SYSTEMS
	Q.100–Q.119

	SPECIFICATIONS OF SIGNALLING SYSTEMS No. 4 AND No. 5
	Q.120–Q.249

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 6
	Q.250–Q.309

	SPECIFICATIONS OF SIGNALLING SYSTEM R1
	Q.310–Q.399

	SPECIFICATIONS OF SIGNALLING SYSTEM R2
	Q.400–Q.499

	DIGITAL EXCHANGES
	Q.500–Q.599

	INTERWORKING OF SIGNALLING SYSTEMS
	Q.600–Q.699

	SPECIFICATIONS OF SIGNALLING SYSTEM No. 7
	Q.700–Q.849

	DIGITAL SUBSCRIBER SIGNALLING SYSTEM No. 1
	Q.850–Q.999

	PUBLIC LAND MOBILE NETWORK
	Q.1000–Q.1099

	INTERWORKING WITH SATELLITE MOBILE SYSTEMS
	Q.1100–Q.1199

	INTELLIGENT NETWORK
	Q.1200–Q.1699

	SIGNALLING REQUIREMENTS AND PROTOCOLS FOR IMT-2000
	Q.1700–Q.1799

	BROADBAND ISDN
	Q.2000–Q.2999

	
	


For further details, please refer to ITU-T List of Recommendations.

	SUPPLEMENT 11 TO ITU-T Q-SERIES RECOMMENDATIONS

Technical Report TRQ.2010: B-ISDN SIGNALLING
INTERWORKING REQUIREMENTS


	Summary

This Supplement specifies the B-ISDN signalling interworking requirements.


	Source
Supplement 11 to ITU-T Q-Series Recommendations, was prepared by ITU-T Study Group 11 (1997-2000) and was approved under the WTSC Resolution No. 5 procedure on 3 December 1999.


	


FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU‑T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU‑T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this publication, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.
INTELLECTUAL PROPERTY RIGHTS
The ITU draws attention to the possibility that the practice or implementation of this publication may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the publication development process.

As of the date of approval of this publication, the ITU had not received notice of intellectual property, protected by patents, which may be required to implement this publication. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS


Page

1
Scope

1
2
References

1
3
Definitions

1
4
Abbreviations

1
5
Introduction

2
5.1
Possible solutions

2
5.2
Communication scenarios

3
6
N-ISDN (64 kbit/s based) interface requirements

3
6.1
Scenario A

4
6.1.1
Introduction

4
6.1.2
Emulation of the B and D channels

6
6.1.3
Emulation of the basic and primary rate access

6
6.2
Scenario B

7
6.3
Interworking requirements for access signalling

8
6.4
Interworking requirements for network signalling

8
7
Interworking of release 1 and release 2

9
7.1
Requirements for B-ISDN signalling protocol evolution

9
7.2
Scenarios for interworking

9
7.2.1
Scenario A – Communication between a release 1 exchange/terminal and a release 2/3 exchange/terminal

9
7.2.2
Scenario B – Communication between two release 2 exchange/terminal in case of a release 1 service request

10
7.2.3
Scenario C – Communication between two release 2 exchange/terminal in case of a release 2/3 service request

10
7.2.4
Scenario D – Communication via a version 1 transit node for release 2/3 services.

10
7.3
Support of release 2 services by using release 1 equipment

11
Supplement 11 to ITU-T Q-Series Recommendations

Technical Report TRQ.2010: B-ISDN SIGNALLING
INTERWORKING REQUIREMENTS

1
Scope

This Supplement specifies the general aspects for the development of B-ISDN signalling requirements. The general aspects are defined in terms of signalling interworking requirements.

2
References

The following Technical Reports and other references contain provisions which, through reference in this text, constitute provisions of this Supplement. At the time of publication, the editions indicated were valid. All supplements and other references are subject to revision; all users of this Supplement are therefore encouraged to investigate the possibility of applying the most recent edition of the supplements and other references listed below. A list of the currently valid ITU-T Recommendations and supplements is regularly published.

[1]
ITU-T Q-series Recommendations ( Supplement 7 (1999), Technical Report TRQ.2001: General aspects for the development of unified signalling requirements.

[2]
ITU-T Q-series Recommendations ( Supplement 10 (1999): Technical Report TRQ.2002: Information Flow Elements.

[3]
ITU-T Recommendation F.811 (1996), Broadband connection-oriented bearer service.

[4]
ITU-T Recommendation Q.921 (1997), ISDN user-network interface – Data link layer specification.

[5]
ITU-T Recommendation Q.931 (1998), ISDN user-network interface layer 3 specification for basic call control.

[6]
ITU-T Recommendation I.431 (1993), Primary rate user-network interface – Layer 1 specification.

[7]
ITU-T Recommendation I.430 (1995), Basic user-network interface – Layer 1 specification.

[8]
CCITT Recommendation Q.767 (1991), Application of the ISDN user part of CCITT Signalling System No. 7 for international ISDN interconnections.

3
Definitions

This Supplement defines the following term:

3.1
call: An end-to-end communications service between two or more call party end points, or between one call party end point and its serving node.
4
Abbreviations

This Supplement uses the following abbreviations:

AAL
ATM Adaptation Layer

ATM
Asynchronous Transfer Mode

BC
Bearer Control

B-ISDN
Broadband Integrated Services Digital Network

B-NT2
Broadband Network Termination type 2

B-TE
Broadband Terminal Equipment

CC
Call Control

ET
Exchange Termination

ISDN
Integrated Services Digital Network

ISUP
Integrated Services User Part

IWU
InterWorking Unit

MTP
Message Transfer Part

NA
Network Adaptor

NNI
Network node interface

NT2
Network Termination Type 2

N-TE
Narrowband Terminal Equipment

PSTN
Public Switched Telephone Network

QOS
Quality Of Service

TE
Terminal Equipment

TEI
Terminal Equipment Identifier

TEX
Transit EXchange

UNI
User-Network Interface

5
Introduction

When a new or enhanced signalling protocol is introduced into a network, it is important that existing operations are not impaired. It is important that any signalling requirements for B-ISDN should take into account the need to maintain the stability of existing standards.

5.1
Possible solutions

Since the target solution is longer term, the probability is that a completely stand-alone network will support B-ISDN. However, two possible options can be applied (the selection will depend on the needs of a network operator), as follows:

i)
integration with the existing networks; or

ii)
a stand-alone overlay network.

With the second option, contact with the existing network(s) will be through clearly identified interworking nodes and there will be a requirement to clearly specify the interworking mechanism to be employed. Impact on existing operations will be minimal and policing at those nodes common to both networks to ensure no "overflowing" of signalling should suffice. The solution itself being new should not impact on the existing standard.

The first option, however, will present greater problems in minimising the impact of an enhanced protocol. To avoid abortive processing and signalling, "signalling route" policing will be required, i.e. B-ISDN signalling will only be allowed on to the appropriate routes. In addition, the enhancements to the existing protocol and procedures should be in accordance with the rules for compatibility contained in current Recommendations.

In the last case, not only should existing operations be protected but the standards will also need to be secured form any possible destabilisation, as a result of enhancement. This can be achieved by using a similar approach to producing the enhancement as that employed for the existing standard. As stated above, any enhancement produced should align with the specified compatibility requirements for procedures, messages, etc. for the standard being enhanced.

Protection of existing standards, compatibility and ease of interworking are important requirements of any signalling solution produced for B-ISDN.

5.2
Communication scenarios

Considering the interworking configuration of ISDN having both 64 kbit/s based ISDN capabilities and broadband capabilities, the communication scenarios are identified as shown in Figure 5-1. These scenarios also apply to any two networks with different signalling protocols, and are as follows:

•
scenario I is an inter-exchange scenario between B-ISDN and 64 kbit/s based ISDN;

•
scenario II is a network concatenation interworking scenario, but the interfaces and services are the same as those which are currently provided by 64 kbit/s based ISDN;

•
in scenario III, the service capabilities provided between broadband user access points are restricted to 64 kbit/s based ISDN capabilities; and

•
in scenario IV, end-to-end access has broadband capabilities and can provide the services currently provided by 64 kbit/s based ISDN. The 64 kbit/s based ISDN services provided by this scenario are similar to those provided by scenario I and III.


[image: image2.wmf]T11109250-00

TE

TE

TE

TE

I 

II

III 

IV

B-ISDN

64 kbit/s 

based ISDN


Figure 5-1 – Communication scenarios between B-ISDN and 64 kbit/s based ISDN

6
N-ISDN (64 kbit/s based) interface requirements

From the view of the B-ISDN user, 64 kbit/s based ISDN services are accessible to them without any restriction. In order to fulfil this requirement, two possible interface scenarios may be considered:

Scenario A:
To provide access to 64 kbit/s based ISDN through a B-ISDN (Figure 6-1); or

Scenario B:
To integrate all 64 kbit/s based ISDN services in B-ISDN with a broadband signalling protocol (see Figure 6-4).

6.1
Scenario A

6.1.1
Introduction

In this scenario a transparent ATM connection, either permanent, semi-permanent or on-demand is used. The corresponding bearer service is broadband connection-oriented bearer service-A as described in Recommendation F.811 [3].

The introduction of ATM in B-ISDN should allow the functionalities operated in the existing ISDN to be re-used.

It should be possible to allow the N-TEs via B-ISDN premises to get access to ISDN services. One possible arrangement may require the B-ISDN to provide transparent connections to connect these N-TEs to an IWU.

This scenario may have two possible cases:

Case 1:
Emulation of the B and D channels of basic and primary rate access.

Case 2:
Emulation of the basic and primary rate access.


[image: image3.wmf]T11109260-00

N-TE

B-NT2 

NA

S

S

N-TE

B-ISDN

64 kbit/s based

ISDN

T

B


Figure 6-1 – Scenario A

This scenario, as depicted in Figures 6-2 and 6-3, may have two possible cases.


[image: image4.wmf]T11109270-00

TE

B-NT2 

NA

S 

TE

S 

T 

Q.931 

S 

T 

S 

T 

Q.931/Q.921 

Q.931/

Q.921

Q.931/

Q.921

ATM 

AAL 

ATM 

ATM 

AAL 

ATM 

AAL 

ATM 

ATM 

AAL 

ATM 

AAL 

ATM 

ATM 

AAL 

Q.931/

Q.921

ISUP/

MTP

Q.931/

Q.921

S 

T 

ATM 

AAL 

ATM 

ATM 

AAL 

B-ISDN

Interworking configuration

case 1.a):

Signalling (Note) 

User information

User information transfer (interworking in the U-plane) 

case 1.b):

Signalling (Note) 

User information

User information transfer (interworking in the U-plane) 

64 kbit/s 

ISDN

Type 1

Type 1

Type 1

C.O. data AAL

NOTE – 

In this scenario C-Plane is not used for signalling for call establishment within B

ISDN.

The signalling protocol will be transferred through ATM connection as user information.

ISUP/

MTP

T

B 

T

B 

T

B 

T

B 

T

B 


Figure 6-2 – Example of interworking Scenario A

6.1.2
Emulation of the B and D channels

a)
B-channels supported by AAL type 1;


D-channel supported by "connection-oriented data AAL".

b)
B-channels supported by AAL type 1;


D-channel supported by AAL type 1.

A B-NT2 is used to connect the TEs (64 kbit/s TEs) to the B-ISDN. At the physical layer activation of the TEs, one permanent connection (semi-permanent or on-demand ATM connection) will link the TEs to the NA. The B-NT2 will packetize/depacketize information from/to D channel in an ATM cell flow. The data link connection will be established between the TEs and the NA and the Recommendation Q.921 [4] TEI procedures may then take place under the control of the NA. The NA may then play the role of the NT2 or ET. Following that step the circuit connection control protocol of Recommendation Q.931 [5] may take place. During the circuit connection establishment phase, the allocation of the B channels will result in dynamic establishment and allocation of ATM connections between the B-NT2 and the NA.

6.1.3
Emulation of the basic and primary rate access

In this scheme the B-NT2 will have a very generic function which is reduced to the circuit emulation function foreseen in AAL class A. The 64 kbit/s S interface will be prolonged from the B-NT2 to the NA. In this scenario the B-ISDN is transparent to the 64 kbit/s ISDN circuit-switched call control. The NA is seen from the B-ISDN as a specialized service provider (server for 64 kbit/s ISDN services access). It is viewed as the NT2 or the ET from the 64 kbit/s based ISDN network.

Two subcases may be distinguished:

a)
ITU-T Recommendation I.431 [6] emulation: in this case the whole physical layer I.431 [6] is emulated by the AAL connection class A; and

b)
ITU-T Recommendation I.430 [7] emulation: in this case only the 2B + D (144 kbit/s) is emulated by the AAL connection class A. The Echo channel E handling will be taken over by the B-NT2 device.


[image: image5.wmf]T11109280-00

TE

B-NT2 

NA

S 

TE

I.431 I.430

Q.931/

Q.921

Q.931/

Q.921

S 

T 

ATM 

AAL 

ATM 

ATM 

AAL 

B-ISDN

Interworking configuration

64 kbit/s 

ISDN

Type 1

NOTE – 

In this scenario C-Plane is not used for signalling for call establishment within B

ISDN.

The signalling protocol will be transferred through ATM connection as user information.

Signalling (Note) and u

ser information transfer

case 2):

ISUP/

MTP

T

B 

T

B 


Figure 6-3 – Example of interworking Scenario A

6.2
Scenario B

In this scenario, network interworking takes place by interconnecting trunk line with broadband signalling protocol.

As a basis for interworking with N-ISDN, the support of N-ISDN services as defined in Recommendation Q.767 [8] according to the most recent stages 1 and 2 Recommendations will be supported. The number of interworking units for one connection should be minimized to prevent degradation of QOS.


[image: image6.wmf]  

T11109290-00

B-TE

B-NT2

NA

S 

S

N-TE

T

NT2

N-TE

B-ISDN

64kbit/s based

ISDN

T

B

S

B


Figure 6-4 – Scenario B

This scenario is depicted in Figure 6-5. In this scenario, B-ISDN shall support not only broadband capabilities but also 64 kbit/s based ISDN capabilities.

Interworking functions must take into account the mapping of protocols with respect to coding, sequencing, timing, etc. These mapping functions employed for interworking between broadband services and 64 kbit/s ISDN services require further study.


[image: image7.wmf]T11109300-00

B-TE

B-NT2

NA

S 

S

NT2

N-TE

T 

AAL

ATM

UNI 

NNI 

Q.931/

Q.921

AAL

ATM

AAL

ATM

S

S

AAL

ATM

ATM

AAL

ATM

S

S

AAL

ATM

AAL

ATM

ATM

ATM

B-ISDN

64 kbit/s 

ISDN

User information

a) 

Interworking configuration

b) 

Signalling (interworking in the C-plane)

User information

c-1) 

User information transfer (interworking in the U-plane) 

        64 kbit/s based ISDN TE connected to B-ISDN 

c-2) 

User information transfer (interworking in the U-plane) 

       B-TE supporting 64 kbit/s based ISDN services 

T

B

S

B

T

B

T

B

ISUP/

MTP

T

B


Figure 6-5 – Example of interworking Scenario B

6.3
Interworking requirements for access signalling

In B-ISDN en-bloc sending and receiving is recommended. For access signalling in the case of narrowband ISDN and PSTN interworking overlap sending and receiving may be applied.

6.4
Interworking requirements for network signalling

Network signalling will support both overlap (sending and receiving) and en-bloc sending and receiving.

7
Interworking of release 1 and release 2

7.1
Requirements for B-ISDN signalling protocol evolution

Taking into account the investment made for release 1 equipment, the requirements for B‑ISDN signalling protocol evolution are:

a)
release 1 protocols applicable across the UNI and NNI are based on the corresponding narrowband ISDN protocols, i.e. the existing Q.931 and ISUP protocols respectively;

b)
release 1 terminals must be able to be connected to a release 2 networks and retain the release 1 services;

c)
the network will allow release 1 and release 2 terminals simultaneously to be used on the same access in release 2 networks; and

d)
future B-ISDN signalling protocols shall support the use of release 1 terminals which use the release 1 UNI protocol.

7.2
Scenarios for interworking

It is desirable that peer-to-peer compatibility between protocols for release 1 and future releases is achieved (for release 1 services).

It is required that release 1 call/bearer control has sufficient functionality to provide the minimum set of release 2/3 bearer control functionality.

Several scenarios of peer-to-peer communication are envisaged of how protocol evolution from release 1 protocol to release 2 protocol could take place by fully reusing the release 1 protocol in a context sensitive manner controlled by the service.

7.2.1
Scenario A – Communication between a release 1 exchange/terminal and a release 2/3 exchange/terminal

Scenario A is represented in Figure 7-1.


[image: image8.wmf]T11109310-00

BC/CC-1

CC-

2/3

BC/CC-1

Version 2 

protocol for Release 2/3 services 

Version 1 

protocol for Release 1 services 

Release 1 service

(CC/BC)


Figure 7-1 – Scenario A

To provide release 1 services, the call control function CC-2/3 is not used and would not be addressed by a call set-up from a "version 1 exchange". For a call set-up from a "version 2 exchange" to a "version 1 exchange" requesting a release 2/3 service, the forward compatibility rules, which have to be included in release 1 protocol, will result in normal unsuccessful case.

7.2.2
Scenario B – Communication between two release 2 exchange/terminal in case of a release 1 service request

Scenario B is represented in Figure 7-2.


[image: image9.wmf]T11109320-00

BC/CC-1

CC-

2/3

BC/CC-1

CC-

2/3

Version 2 

protocol for Release 2/3 services 

Version 2 

protocol for Release 2/3 services 

Release 1 service

(CC/BC)


Figure 7-2 – Scenario B

7.2.3
Scenario C – Communication between two release 2 exchange/terminal in case of a release 2/3 service request

Scenario C is represented in Figure 7-3.

In this case from a context sensitive manner controlled by the service, the CC-2/3 function (supported by CC-2/3 protocol) is used instead of the CC-1 function. However, the bearer control protocol remains the same as in the previous cases shown above.


[image: image10.wmf]T11109330-00

BC/CC-1

CC

BC

CC-

2/3

BC/CC-1

CC-

2/3

Release 2/3 service

Version 2 

protocol for Release 2/3 services 

Version 2 

protocol for Release 2/3 services 


Figure 7-3 – Scenario C

7.2.4
Scenario D – Communication via a version 1 transit node for release 2/3 services.

Scenario D is represented in Figure 7-4.

A "version 1 exchange" can serve as transit node for the bearer connection. It was agreed that there is the requirement for a release 1 protocol to transfer information which is not understood transparently, or instruct appropriately.


[image: image11.wmf]T11109340-00

BC/CC-1

CC

BC

CC-

2/3

BC/CC-1

CC-

2/3

BC

BC/CC-1

Release 2/3 service

Versión 2 

protocol for Release 2/3 services 

Versión 2 

protocol for Release 2/3 services 


Figure 7-4 – Scenario D

7.3
Support of release 2 services by using release 1 equipment

For a number of release 2 specific services, no specific release 2 bearer control is needed – e.g. bandwidth modification. Therefore, in principle, it is possible to use release 1 TEXs to establish the bearers for these services. This allows a smooth introduction of release 2. This is depicted in Figure 7-5.


[image: image12.wmf]T11109350-00

DCA

LCA

DCA 

LCA 

EC

LC

DC

LC

LC

DC

LC

LC

LC

EC

A 

C 

A 

C 

A 

C 

A 

C 

DCA

LCA

DCA 

LCA 

EC

LC

DC

LC

LC

DC

LC

EC

A 

C 

A 

C 

A 

C 

Q.71

FE3 

A 

C 

A 

C 


Figure 7-5 – For some release 2 specific services both release 1 and 2 TEXs may be used

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	


	ITU-T  RECOMMENDATIONS  SERIES

	Series A
	Organization of the work of the ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	













Q series – Supplement 11    (12/1999)
ii

_1038384288.unknown

_1041832117.unknown

_1041832290.unknown

_1041832448.unknown

_1038384390.unknown

_1038384520.unknown

_1038383173.unknown

_1038383617.unknown

_1038135583.unknown

_1038135860.unknown

_1038135393.unknown

