
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION


	ITU-T
	T.37

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Amendment 1

(09/99)   


SERIES T: TERMINALS FOR TELEMATIC SERVICES

Procedures for the transfer of facsimile data via store-and-forward on the Internet

Amendment 1:  Full Mode
ITU-T  Recommendation  T.37  –  Amendment 1

(Previously  CCITT  Recommendation)
ITU-T  T-SERIES  RECOMMENDATIONS

TERMINALS FOR TELEMATIC SERVICES
For further details, please refer to ITU-T List of Recommendations.

ITU-T  RECOMMENDATION  T.37

PROCEDURES  FOR  THE  TRANSFER  OF  FACSIMILE  DATA
VIA  STORE-AND-FORWARD  ON  THE  INTERNET
AMENDMENT  1

Full Mode
Summary
This amendment adds support for the Full Mode of Internet fax to Recommendation T.37. The additional functionality includes support for the transmission and reception of additional TIFF image formats, capabilities exchange and acknowledgement of receipt.

Source
Amendment 1 to ITU-T Recommendation T.37 was prepared by ITU-T Study Group 8 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on 24 September 1999.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of tele​communi​cations. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to stan​dard​izing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU‑T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU‑T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administrations" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL  PROPERTY  RIGHTS
The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2000

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page
11)
Clause 3



12)
Subclause 6.2



23)
Appendix I



54)
New Appendix III



55)
Add a new Appendix IV





Recommendation T.37

Recommendation T.37/Amd.1     (09/99)

PROCEDURES  FOR  THE  TRANSFER  OF  FACSIMILE  DATA
VIA  STORE-AND-FORWARD  ON  THE  INTERNET

AMENDMENT  1

Full Mode

(Geneva, 1999)

1)
Clause 3

Delete the following references:
–
CCITT Recommendation T.6 (1988), Facsimile coding schemes and coding control functions for Group 4 facsimile apparatus.

–
RFC 1725, Post Office Protocol – Version 3.

Add the following references and Note:
–
RFC 1869, SMTP Service Extensions.
–
RFC 1939, Post Office Protocol – Version 3.
–
RFC 2060, Internet Message Access Protocol – Version 4rev1.
–
RFC 2298, An Extensible Message Format for Message Disposition Notifications.
–
RFC 2532, Extended Facsimile Using Internet Mail.
–
RFC 2530, Indicating Supported Media Features Using Extensions to DSN and MDN.
–
RFC 2531, Content Feature Schema for Internet Fax.
NOTE – Embedded references within RFCs referenced in this Recommendation, which are described as "work in progress", are informative only and not normative.

2)
Subclause 6.2

Amend subclause 6.2 to read:
6.2
Full Mode

Full Mode supports the transfer of image data, capabilities exchange and acknowledgement of receipt, per the requirements of Recommendation F.185.

6.2.1
Procedures for Full Mode

Procedures for Full Mode are defined in RFC 2532, unless stated otherwise in Table 2. A functional summary and related references for Full Mode are included in the table.

Table 2/T.37 – Functional summary of procedures for Full Mode

	Addressing
	Procedures:  RFC 2305 section 3
Email:  RFC 822
Offramp:  RFC 2303, RFC 2304

	Image format
	TIFF Profiles:  RFC 2301

	Acknowledgement of receipt
	Delivery Confirmation:  RFC 1891, Delivery Status Notification (DSN)
Processing Confirmation:  RFC 2298, Message Disposition Notification (MDN)

	Capabilities exchange
	Capabilities Format:  RFC 2531
Capabilities Mechanism:  RFC 2530

	Transport
	ESMTP:  RFC 1869

	Mailbox access 
	POP3:  RFC 1939
IMAP4:  RFC 2060

	NOTE – The requirement for onramp and offramp gateways to support G4 facsimile terminals is for further study.


3)
Appendix I

Add a new subclause I.2:
I.2
Full Mode implementation requirements

Table I.2/T.37 – Implementation requirements for Full Mode

	Sender

	Required
	Send image data as a single MIME multi-page RFC 2301 file
	RFC 2305 § 2.2.3

	
	Provide notice in case of relay problems
	RFC 2305 § 2.3.1

	
	Provide a return address in the SMTP mail from envelope header
	RFC 2305 § 2.3.1

	
	Request Delivery Status Notification, if the sender desires delivery confirmation
	RFC 2532 § 2.1.1
(Note 3)

	
	Request Message Disposition Notification, if the sender desires processing confirmation
	RFC 2532 § 2.1.2
(Note 3)

	Strongly recommended
	Include Message-ID
	RFC 2305 § 2.2.1

	
	Use Base 64 encoding for image data
	

	
	Be able to recognize and process the feature tags as defined in RFC 2531 when reviewing the capabilities presented by a potential recipient
	RFC 2532 § 3
(Note 2)

	Optional
	Use other TIFF Profiles if it has prior knowledge that such profiles are supported by the receiver
	RFC 2305 § 4

	
	Provide notice on receipt of DSN or other notifications
	RFC 1894
(Note 3)

	
	Request a positive DSN and an MDN, if DSN/MDN capability response is desired
	RFC 2532 § 3.3
(Notes 1, 2 and 4)

	
	Send a message containing both minimum subset of TIFF and a higher quality TIFF
using multipart/alternative
	RFC 2532 § 3

	
	Manually override the default setting
	RFC 2532 § 3.1

	
	Access directory structure of recipient capabilities
	RFC 2532 § 3.2

	
	Use any of the TIFF Profiles defined in RFC 2301 when sending to a recipient with the corresponding capabilities based on an MDN/DSN capability response
	RFC 2532 § 3
(Notes 1, 2 and 4)


Table I.2/T.37 (continued)

	Receiver

	Required
	Be MIME compliant except that it is not required to offer to place a MIME attachment in a file and may print a received file rather than display 
	RFC 2305 § 2.2.2

	
	Be capable of processing multiple MIME RFC 2301 Profile S image files within a single message
	RFC 2305 § 2.2.4

	
	Provide notice in case of reception or processing problems
	RFC 2305 § 2.3.2
(Note 3)

	
	Implement Message Disposition Notification
	RFC 2532 § 2.2
(Notes 1, 2 and 4)

	
	Indicate supported media features in DSN and/or MDN messages per RFC 2530
	RFC 2532 § 2.2

	
	Configurable to silently ignore a request for MDN
	RFC 2532 § 2.2.1

	
	Not generate an unsolicited MDN to indicate successful processing
	RFC 2532 § 2.2.1

	
	Possible for an operator to disable automatic MDN responses
	RFC 2532 § 2.2.1

	
	Not generate DSN when using POP3 and IMAP4
	RFC 2532 § 2.2.2

	Strongly recommended
	If the MTA determines the message cannot be processed, reject the message with a status code 5.6.1
	RFC 2530
(Note 3)

	Optional
	Be capable of processing any TIFF Profiles
	RFC  2305 § 4

	
	Automated system be configurable to always respond to MDN request
	RFC 2532 § 2.2.1

	Sender infrastructure

	Required
	Support for DSN
	RFC 2532 § 2.3.1
(Note 5)

	Receiver infrastructure

	Required
	Support for DSN
	RFC 2532 § 2.3.2
(Note 5)

	Offramp gateway (When implemented)

	Required
	Be SMTP compliant
	RFC 0821

	
	Provide delivery failure notification
	RFC 1894,
RFC 2305 § 2.3.1
(Note 3)

	
	Be able to process PSTN/FAX Email addresses
	RFC  2303,
RFC  2304

	
	Comply with the relevant ITU Recommendations relating to facsimile transmission 
	T.30

	
	Attempt to relay authorized Email to the corresponding G3 facsimile terminals
	RFC 2305 § 3.2

	
	Ensure that local legal requirements relating to facsimile transmissions are met
	

	Strongly recommended
	Use DSN for delivery failure notification
	RFC 2305 § 2.3.1 RFC 1894
(Note 3)

	
	Use an approved mailbox access protocol when serving multiple users
	RFC 2305 § 2.1.3

	Optional
	Translate image data into a format acceptable by the receiving G3 facsimile terminal
	RFC 2305 § 2.1.2

	
	Use a mailbox access protocol when serving a single mail recipient
	RFC 2305 § 2.1.3


Table I.2/T.37 (end)

	NOTE 1 – Typical Full Mode implementation includes capability exchange. The available method for eliciting a capability response is by requesting an MDN or DSN. In response, recipient should provide an indication of capabilities using DSN/MDN extensions per RFC 2530 extension. The detailed method of requesting MDN/DSN and use of other capabilities request and indication methods are for further study.

NOTE 2 – For the typical Full Mode implementation, the DSN or MDN that is returned should contain information describing the recipient's capabilities. The sender should use this information for subsequent communications with that recipient. For example, a sender may use any of the TIFF Profiles defined in RFC 2301 when sending to a recipient with the corresponding capabilities based on an MDN/DSN capability response.

NOTE 3 – For the typical Full Mode implementation, delivery confirmation and processing confirmation are available using DSN and MDN. Clarification of MDN processing is for further study. Inclusion of additional data such as total page number and error page number in MDN and DSN messages is for further study.

NOTE 4 – Some implementers may wish to add private extensions to the standard Full Mode implementation. For example, venders could find company-specific MIME types and X- Email headers to help implement specific applications.

NOTE 5 – Per RFC 2532, DSN support is required within the sender and receiver infrastructure. Full Mode terminals may still operate in situations where some MTAs in the mail relay path do not support DSN. Delivery confirmation via DSN for the final destination will not be provided in this case. However, processing confirmation via MDN may still be possible.


4)
New Appendix III

Add a new Appendix III:
Appendix III

Full Mode RFC references chart

This appendix provides a chart of relationships among the Internet standards that are used for achieving T.37 Full Mode functionality.

[image: image2.wmf]T0828420-98/d01

informational

EIFAX

RFC 2532

T.30 Mapping

RFC Mapping

Schema

RFC 2531

Registration

RFC 2506

Mail

routing

RFC 974

Syntax

RFC 2533

Media

features

RFC 2534

Simple

mode

RFC 2305

MDN

RFC 2298

SMTP server

extension

RFC 2034

Reporting

RFC 2530

Status

codes

RFC 1893

DSN

format

RFC 1894

TIFF for

facsimile

RFC 2301

DSN

mechanism

RFC 1891

Multipart/

report

RFC 1892


FIGURE [D01]

5)
Add a new Appendix IV

Add a new Appendix IV:
Appendix IV

TIFF-FX (RFC 2301) coding examples for profiles beyond Profile S

This appendix provides examples of coding image data for profiles beyond Profile S. It corresponds with TIFF-FX (RFC2301) Profiles F, J, C, L and M. This is provided for information only.

The actual files used to generate the attached coding examples, together with the associated test cases and other TIFF-FX information, may be accessed at the following URL:

http://www.itu.int/ITU-T/tiff-fx/index.html

Content of TIFF-FX coding examples
Page

IV.1
Profile F

7

IV.1.1
T.4 one-dimensional MH compression, with 204196 pels/25.4 mm, big endian byte order, based on test file f05x-02.tif and test case Profile F F5

7

IV.1.2
T.4 two-dimensional MR compression, with 80154 pels/cm, little endian byte order, photometric interpretation of black=0 (i.e. inverse image), cm resolution unit, based on test file f06x-01.tif and test case Profile F F6

10

IV.1.3
T.6 two-dimensional MMR compression, with 400 pels/25.4 mm, big endian byte order, multiple strips and no Global Parameters IFD (GP IFD), based on test file f03x-02.tif and test case Profile F F3

13

IV.2
Profile J


T.82 JBIG compression per T.85, with 300 pels/25.4 mm, big endian byte order and no GP IFD, based on test file j01x-02.tif and test case Profile J J1

17

IV.3
Profile C


T.81 JPEG compression with T.4 Annex E ITU L*a*b* color encoding, 4:1:1 subsampling, 200 pels/25.4 mm and little endian byte order, based on test file c03x-02.tif and test case Profile C C3

20

IV.4
Profile L


T.82 JBIG compression with T.43 color encoding

24

IV.4.1
One bit per component CMYK color representation, 400 pels/25.4 mm and big endian byte order, based on test file l02x-02.tif and test case Profile L L2

24

IV.4.2
Eight bits per component ITU L*a*b* palette color encoding, 200 pels/25.4 mm and little endian byte order, based on test file l04x-02.tif and test case Profile L L4. Note that the ColorMap tag (i.e. tag code 320), present in this sample, is no longer used and may be ignored.

27

IV.5
Profile M


T.44 MRC imaging structure for mixed text/line-art and color encoding

31

IV.5.1
Three layers with single strip per layer and little endian byte order: mask using T.6 two-dimensional MMR compression and 400 pels/25.4 mm; background and foreground using JPEG compression with T.4 Annex E ITU L*a*b* color encoding; background at 100 pels/25.4 mm and 1:1:1 subsampling, foreground at 200 pels/25.4 mm and 4:1:1 subsampling; based on test file m02x-04.tif and test case Profile M M2

31

IV.5.2
Three layers with multiple strips per layer and little endian byte order: mask using T.82 JBIG compression per T.85 constraints at 400 pels/25.4 mm; background and foreground use X and Y Position offsets (i.e. contains regions without coded data); background using JPEG compression with T.4 Annex E ITU L*a*b* color encoding at 200 pels/25.4 mm and 1:1:1 subsampling; foreground using T.82 JBIG compression with T.43 lossless color encoding of ITU L*a*b*, DefaultImageColor (LayerBaseColor) other than black, and both 200 and 400 pels/25.4 mm; based on test file m11x-02.tif and test case Profile M M11

40

IV.5.3
Two layers with single strip per layer and little endian byte order: mask and foreground use X and Y Position offsets (i.e. contains regions without coded data); mask using T.4 one-dimensional MH compression at 200 pels/25.4 mm; foreground using JPEG compression with T.4 Annex E ITU L*a*b* color encoding, DefaultImageColor (LayerBaseColor) other than black, 200 pels/25.4 mm and 4:1:1 subsampling; based on test file m06x-03.tif and test case Profile M M6

50

IV.1
Profile F coding examples

IV.1.1
T.4 one-dimensional (MH) compression

A4 page at 204x196 pels/25.4 mm, MH with non-byte aligned EOLs, single-strip image, FillOrder is MSB, 0 is white, based on test case Profile F F5

Table IV.1-1/T.37 – Coding example of TIFF header, IFD and image data

	Header
	
	
	
	
	File Length 166470 bytes

	Offset
	Description
	
	
	
	Comments
	Coding

	0000
	Octet order
	
	
	
	Big endian order
	4D 4D

	0002
	File identifier
	
	
	
	42, TIFF file
	00 2A

	0004
	1st IFD offset
	
	
	
	Offset of IFD 0 
	00 00 00 08

	IFD0  Offset 8 (0x8H) NextIFDOffset 62140 (0xf2bcH)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0008
	Number of Directory Entries
	–
	–
	–
	20 entries
	00 14

	000A
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	00 FE 00 04 00 00 00 01 00 00 00 02

	0016
	ImageWidth
	256
	Long
	1728
	A4
	01 00 00 04 00 00 00 01 00 00 06 C0

	0022
	ImageLength
	257
	Long
	n/a
	2339 lines
	01 01 00 04 00 00 00 01 00 00 09 23 

	002E
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	01 02 00 03 00 00 00 01 00 01 00 00

	003A
	Compression
	259
	Short
	3
	T.4 MH or MR
	01 03 00 03 00 00 00 01 00 03 00 00

	0046
	Photometric Interpretation
	262
	Short
	0
	0 is white
	01 06 00 03 00 00 00 01 00 00 00 00

	0052
	FillOrder
	266
	Short
	2
	MSB first
	01 0A 00 03 00 00 00 01 00 01 00 00

	005E
	DocumentName
	269
	ASCII
	n/a
	Test case D05F
	01 0D 00 02 00 00 00 0F 00 00 00 FE

	006A
	ImageDescription
	270
	ASCII
	n/a
	Page 1: French text
Page 2: kanji
	01 0E 00 02 00 00 00 22 00 00 01 0E

	0076
	StripOffsets
	273
	Long
	n/a
	444 bytes
	01 11 00 04 00 00 00 01 00 00 01 BC

	0082
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	01 15 00 03 00 00 00 01 00 01 00 00

	008E
	RowsPerStrip
	278
	Long
	(ImageLength
	2339 rows
	01 16 00 04 00 00 00 01 00 00 09 23

	009A
	StripByteCounts
	279
	Long
	n/a
	61694 bytes
	01 17 00 04 00 00 00 01 00 00 F0 FE

	00A6
	Xresolution
	282
	Rational
	204, 200
	204 pixels/inch
	01 1A 00 05 00 00 00 01 00 00 01 32

	00B2
	Yresolution
	283
	Rational
	200,196,100, 98
	196 pixels/inch
	01 1B 00 05 00 00 00 01 00 00 01 3A

	00BE
	T4Option
	292
	Long
	0
	1-dimensional MH,
byte aligned
	01 24 00 04 00 00 00 01 00 00 00  04 


	00CA
	ResolutionUnit
	296
	Short
	2
	inch
	01 28 00 03 00 00 00 01 00 02 00 00

	00D6
	PageNumber
	297
	Short
	n/a
	1 of 2
	01 29 00 03 00 00 00 02 00 00 00 02

	00E2
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	01 31 00 02 00 00 00 41 00 00 01 42


	00EE
	GlobalParametersIFD
	400
	Long
	n/a
	Offset 390
	01 90 00 04 00 00 00 01 00 00 01 86 

	
	
	
	
	
	
	Next IFD offset 00 00 F2 BC

	IFD 0 Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	00FE
	DocumentName
	Test Case D05F
	54 65 73 74 20 63 61 73 65 20 44 30
35 46 00

	00010E


00011E

00012E
	ImageDescription
	Page 1: French text


Page 2: Kanji
	50 61 67 65 20 31 3A 20 46 72 65 6E 63 68 20 74

65 78 74 20 50 61 67 65 20 32 3A 20 6B 61 6E 6A

69 00

	000132
	Xresolution 
	204 pixels/inch
	00 00 00 CC 00 00 00 01

	00013A
	Yresolution 
	196 pixels/inch
	00 00 00 C4 00 00 00 01

	000142


000152


000162


000172


000182
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 30

38 20 30 38 3A 35 38 3A 30 32 20 50 4D 20 39 39

00

	IFD 0 ( GP IFD ( Child IFD 0 Offset 390 (0x186H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	000186
	Number of Directory Entries
	–
	–
	–
	4 entries
	00 04

	000188
	ProfileType
	401
	Long
	1
	1 (Group 3 fax)
	01 91 00 04 00 00 00 01 00 00 00 01 

	000194
	FaxProfile
	402
	Byte
	1
	2 (Extended black & white lossless,
Profile F)
	01 92 00 01 00 00 00 01 02 00 00 00

	0001A0
	CodingMethod
	403
	Long
	2
	2 (MH)
	01 93 00 04 00 00 00 01 00 00 00 02

	0001AC
	VersionYear
	404
	Byte
	n/a
	1998
	01 94 00 01 00 00 00 04 31 39 39 38

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	Image data

	Offset
	Description
	Comments

	0001bcH - 00f2baH
	IFD 0 Strip 0
	Compressed data

	IFD1  Offset 62140 (0xf2bcH) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	00F2BC
	Number of Directory Entries
	–
	–
	–
	19 entries
	00 13

	00F2BE
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	00 FE 00 04 00 00 00 01 00 00 00 02

	00F2CA
	ImageWidth
	256
	Long
	1728
	1728 (A4)
	01 00 00 04 00 00 00 01 00 00 06 C0

	00F2DA
	ImageLength
	257
	Long
	n/a
	2339 lines
	01 01 00 04 00 00 00 01 00 00 09 23 

	00F2E2
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	01 02 00 03 00 00 00 01 00 01 00 00

	00F2EE
	Compression
	259
	Short
	3
	T.4 HM or MR
	01 03 00 03 00 00 00 01 00 03 00 00

	00F2FA
	Photometric Interpretation
	262
	Short
	0
	0 is white
	01 06 00 03 00 00 00 01 00 00 00 00

	00F306
	FillOrder
	266
	Short
	2
	MSB first
	01 0A 00 03 00 00 00 01 00 01 00 00

	00F312
	DocumentName
	269
	ASCII
	n/a
	Test case D05F
	01 0D 00 02 00 00 00 0F 00 00 F3 A6

	00F31E
	ImageDescription
	270
	ASCII
	n/a
	Page 1: French text
Page 2: kanji
	01 0E 00 02 00 00 00 22 00 00 F3 B6

	00F32A
	StripOffsets
	273
	Long
	n/a
	62510 bytes
	01 11 00 04 00 00 00 01 00 00 F4 2E

	00F336
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	01 15 00 03 00 00 00 01 00 01 00 00

	00F342
	RowsPerStrip
	278
	Long
	(ImageLength
	2339 rows
	01 16 00 04 00 00 00 01 00 00 09 23

	00F34E
	StripByteCounts
	279
	Long
	n/a
	103958 bytes
	01 17 00 04 00 00 00 01 00 01 96 16

	00F35A
	Xresolution
	282
	Rational
	204, 200 pixels/inch
	204 pixels/inch
	01 1A 00 05 00 00 00 01 00 00 F3 DA

	00F366
	Yresolution
	283
	Rational
	200,196,100,
98 pixels/inch
	196 pixels/inch
	01 1B 00 05 00 00 00 01 00 00 F3 E2

	00F372
	T4Option
	292
	Long
	0, 4
	1-dimensional MH,
byte aligned
	01 24 00 04 00 00 00 01 00 00 00  04


	00F37E
	ResolutionUnit
	296
	Short
	2
	inches
	01 28 00 03 00 00 00 01 00 02 00 00

	00F38A
	PageNumber
	297
	Short
	n/a
	2 of 2
	01 29 00 03 00 00 00 02 00 01 00 02

	00F396
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	01 31 00 02 00 00 00 41 00 00 F3 EA 

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1 Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	00F3A6
	DocumentName
	Test Case D05F
	54 65 73 74 20 63 61 73 65 20 44 30
33 46 00

	00F3B6


00F3C6


00F3D6
	ImageDescription
	Page 1: French text


Page 2: Kanji
	50 61 67 65 20 31 3A 20 46 72 65 6E 63 68 20 74

65 78 74 20 50 61 67 65 20 32 3A 20 6B 61 6E 6A

69 00

	00F3DA
	Xresolution 
	204 pixels/inch
	00 00 00 CC 00 00 00 01

	00F3E2
	Yresolution 
	196 pixels/inch
	00 00 00 C4 00 00 00 01

	00F3EA


00F3FA


00F40A


00F41A


00F42A
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 30

38 20 30 38 3A 35 38 3A 30 32 20 50 4D 20 39 39

00

	Image data

	Offset
	Description
	Comments

	00f42eH - 028a44H
	IFD 0 Strip 0
	Compressed data


IV.1.2
T.4 two-dimensional (MR) compression

A4 page at 80154 pels/cm, MR with non-byte aligned EOLs, single strip image, FillOrder is MSB, 0 is black, with GP IFD, based on test case Profile F F6

Table IV.1-2/T.37 – Coding example of TIFF header, IFD and image data

	Header
	
	
	
	
	File Length 236002 bytes

	Offset
	Description
	
	
	
	Comments
	Coding

	0000
	Octet order
	
	
	
	Little endian order
	49 49

	0002
	File identifier
	
	
	
	42, TIFF file
	2A 00

	0004
	1st IFD offset
	
	
	
	Offset of IFD 0 
	08 00 00 00

	IFD0  Offset 8 (0x8H) NextIFDOffset 81740 (0x13f4cH)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0008
	Number of Directory Entries
	–
	–
	–
	20 entries
	14 00

	000A
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	FE 00 04 00 01 00 00 00 02 00 00 00

	0016
	ImageWidth
	256
	Long
	1728
	A4
	00 01 04 00 01 00 00 00 C0 06 00 00

	0022
	ImageLength
	257
	Long
	n/a
	4677 lines
	01 01 04 00 01 00 00 00 45 12 00 00 

	002E
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	02 01 03 00 01 00 00 00 01 00 00 00

	003A
	Compression
	259
	Short
	3
	T.4 HM or MR
	03 01 03 00 01 00 00 00 03 00 00 00

	0046
	Photometric Interpretation
	262
	Short
	0
	0 is black
	06 01 03 00 01 00 00 00 01 00 00 00

	0052
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	005E
	DocumentName
	269
	ASCII
	n/a
	Test case D06F
	0D 01 02 00 0F 00 00 00 FE 00 00 00

	006A
	ImageDescription
	270
	ASCII
	n/a
	Page 1: French text
Page 2: kanji
	0E 01 02 00 22 00 00 00 0E  01 00 00

	0076
	StripOffsets
	273
	Long
	n/a
	444 bytes
	11 01 04 00 01 00 00 00 BC 01 00 00

	0082
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	15 01 03 00 01 00 00 00 01 00 00 00

	008E
	RowsPerStrip
	278
	Long
	(ImageLength
	4677 rows
	16 01 04 00 01 00 00 00 45 12 00 00

	009A
	StripByteCounts
	279
	Long
	n/a
	81296 bytes
	17 01 04 00 01 00 00 00 90 3D 01 00

	00A6
	Xresolution
	282
	Rational
	204, 200
	80 pixels/cm
	1A 01 05 00 01 00 00 00 32 01 00 00

	00B2
	Yresolution
	283
	Rational
	200,196,100, 98
	154 pixels/cm
	1B 01 05 00 01 00 00 00 3A 01 00 00

	00BE
	T4Option
	292
	Long
	0,4
	2-dimensional MR and not byte-aligned EOL
	24 01 04 00 01 00 00 00 01 00 00 00 


	00CA
	ResolutionUnit
	296
	Short
	2
	cm
	28 01 03 00 01 00 00 00 03 00 00 00

	00D6
	PageNumber
	297
	Short
	n/a
	1 of 2
	29 01 03 00 02 00 00 00 00 00 02 00

	00E2
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 42 01 00 00


	00EE
	GlobalParametersIFD
	400
	Long
	n/a
	Offset 390
	90 01 04 00 01 00 00 00 86 01 00 00 

	
	
	
	
	
	
	Next IFD offset 4C 3F 01 00

	IFD 0  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	000FE
	DocumentName
	Test Case D06F
	54 65 73 74 20 63 61 73 65 20 44 30
36 46 00

	00010E


00011E


00012E
	ImageDescription
	Page 1: French text


Page 2: Kanji
	50 61 67 65 20 31 3A 20 46 72 65 6E 63 68 20 74

65 78 74 20 50 61 67 65 20 32 3A 20 6B 61 6E 6A

69 00

	000132
	Xresolution 
	80 pixels/cm
	50 00 00 00 01 00 00 00

	00013A
	Yresolution 
	154 pixels/cm
	9A 00 00 00 01 00 00 00

	000142


000152


000162


000172


000182
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	IFD 0 ( GP IFD (  Child IFD 0 Offset 390 (0x186H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	000186
	Number of Directory Entries
	–
	–
	–
	4 entries
	04 00

	000188
	ProfileType
	401
	Long
	1
	1 (Group 3 fax)
	91 01 04 00 01 00 00 00 01 00 00 00 

	000194
	FaxProfile
	402
	Byte
	1
	2 (Extended black & white lossless,
Profile F)
	92 01 01 00 01 00 00 00 02 00 00 00

	0001A0
	CodingMethod
	403
	Long
	2, MH
	MR
	93 01 04 00 01 00 00 00 04 00 00 00

	0001AC
	VersionYear
	404
	Byte
	n/a
	1998
	94 01 01 00 04 00 00 00 31 39 39 38 

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	Image data

	Offset
	Description
	Comments

	0001bcH - 013f4cH
	IFD 0 Strip 0
	Compressed data

	IFD1  Offset 81740 (0x13f4cH) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	013F4C
	Number of Directory Entries
	–
	–
	–
	19 entries
	13 00

	013F4E
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	FE 00 04 00 01 00 00 00 02 00 00 00

	013F5A
	ImageWidth
	256
	Long
	1728
	A4
	00 01 04 00 01 00 00 00 C0 06 00 00

	013F66
	ImageLength
	257
	Long
	n/a
	4677 lines
	01 01 04 00 01 00 00 00 45 12 00 00 

	013F72
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	02 01 03 00 01 00 00 00 01 00 00 00

	013F7E
	Compression
	259
	Short
	3
	T.4 HM or MR
	03 01 03 00 01 00 00 00 03 00 00 00

	013F8A
	Photometric Interpretation
	262
	Short
	0
	0 is black
	06 01 03 00 01 00 00 00 01 00 00 00

	013F96
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	013FA2
	DocumentName
	269
	ASCII
	n/a
	Test case D06F
	0D 01 02 00 0F 00 00 00 36 40 01 00

	013FAE
	ImageDescription
	270
	ASCII
	n/a
	Page 1: French text
Page 2: kanji
	0E 01 02 00 22 00 00 00 46 40 01 00

	013FBA
	StripOffsets
	273
	Long
	n/a
	82110 bytes
	11 01 04 00 01 00 00 00 BE 40 01 00

	013FC6
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	15 01 03 00 01 00 00 00 01 00 00 00

	013FD2
	RowsPerStrip
	278
	Long
	(ImageLength
	4677 rows
	16 01 04 00 01 00 00 00 45 12 00 00

	013FDE
	StripByteCounts
	279
	Long
	n/a
	153890 bytes
	17 01 04 00 01 00 00 00 22 59 02 00

	013FEA
	Xresolution
	282
	Rational
	204, 200 pixels/inch
	80 pixels/cm
	1A 01 05 00 01 00 00 00 6A 40 01 00

	013FF6
	Yresolution
	283
	Rational
	200,196,100, 98 pixels/inch
	154 pixels/cm
	1B 01 05 00 01 00 00 00 72 40 01 00

	014002
	T4Option
	292
	Long
	0, 4
	2-dimensional MR and not byte-aligned EOL
	24 01 04 00 01 00 00 00 01 00 00 00 

	01400E
	ResolutionUnit
	296
	Short
	2
	cm
	28 01 03 00 01 00 00 00 03 00 00 00

	01401A
	PageNumber
	297
	Short
	n/a
	2 of 2
	29 01 03 00 02 00 00 00 01 00 02 00

	014026
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 7A 40  01 00 

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	014036
	DocumentName
	Test Case D06F
	54 65 73 74 20 63 61 73 65 20 44 30
36 46 00

	014046


014056


014066
	ImageDescription
	Page 1: French text


Page 2: Kanji
	50 61 67 65 20 31 3A 20 46 72 65 6E 63 68 20 74

65 78 74 20 50 61 67 65 20 32 3A 20 6B 61 6E 6A

69 00

	01406A
	Xresolution 
	80 pixels/cm
	50 00 00 00 01 00 00 00

	014072
	Yresolution 
	154 pixels/cm
	9A 00 00 00 01 00 00 00

	01407A


01408A


01409A


0140AA


0140BA
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	Image data

	Offset
	Description
	Comments

	000140beH - 000399e0H
	IFD 1 Strip 0
	Compressed data


IV.1.3
T.6 two dimensional (MMR) compression

A4 page at 400400 pels/25.4 mm, MMR, multi-strip image, FillOrder is MSB, 0 is white and without GlobalParameters IFD, based on test case Profile F F3

Table IV.1-3/T.37 – Coding example of TIFF header, IFD and image data

	Header
	
	
	
	
	File Length 193324 bytes

	Offset
	Description
	
	
	
	Comments
	Coding

	0000
	Octet order
	
	
	
	Big endian order
	4D 4D

	0002
	File identifier
	
	
	
	42, TIFF file
	00 2A

	0004
	1st IFD offset
	
	
	
	Offset of IFD 0 
	00 00 00 08

	IFD0  Offset 8 (0x8H) NextIFDOffset 61434 (0xeffaH)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0008
	Number of Directory Entries
	–
	–
	–
	19 entries
	00 13

	000A
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	00 FE 00 04 00 00 00 01 00 00 00 02 

	0016
	ImageWidth
	256
	Long
	3456
	A4
	01 00 00 04 00 00 00 01 00 00 0D 80

	0022
	ImageLength
	257
	Long
	n/a
	4677 lines
	01 01 00 04 00 00 00 01 00 00 12 45 

	002E
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	01 02 00 03 00 00 00 01 00 01 00 00

	003A
	Compression
	259
	Short
	3
	T.6 MMR
	01 03 00 03 00 00 00 01 00 04 00 00

	0046
	Photometric Interpretation
	262
	Short
	0
	0 is white
	01 06 00 03 00 00 00 01 00 00 00 00

	0052
	FillOrder
	266
	Short
	2
	MSB first
	01 0A 00 03 00 00 00 01 00 01 00 00

	005E
	DocumentName
	269
	ASCII
	n/a
	Test case D03F
	01 0D 00 02 00 00 00 0F 00 00 00 F2

	006A
	ImageDescription
	270
	ASCII
	n/a
	Page 1: French text
Page 2: kanji
	01 0E 00 02 00 00 00 22 00 00 01 02

	0076
	StripOffsets
	273
	Long
	n/a
	458 690 8262 18314 34710 39266 46030 48586 56794 61370 bytes
	01 11 00 04 00 00 00 0A 00 00 01 26

	0082
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	01 15 00 03 00 00 00 01 00 01 00 00

	008E
	RowsPerStrip
	278
	Long
	(ImageLength
	500 rows
	01 16 00 04 00 00 00 01 00 00 01 F4

	009A
	StripByteCounts
	279
	Long
	n/a
	232 7572 10051 16393 4555 6763 2556 8208 4576 62 bytes
	01 17 00 04 00 00 00 0A 00 00 01 4E

	00A6
	Xresolution
	282
	Rational
	204, 200
	400 pixels/inch
	01 1A 00 05 00 00 00 01 00 00 01 76

	00B2
	Yresolution
	283
	Rational
	200,196,100, 98
	400 pixels/inch
	01 1B 00 05 00 00 00 01 00 00 01 7E

	00BE
	T6Option
	293
	Long
	0
	MMR  (G4) compressed
	01 25 00 04 00 00 00 01 00 00 00  00 


	00CA
	ResolutionUnit
	296
	Short
	2
	inch
	01 28 00 03 00 00 00 01 00 02 00 00

	00D6
	PageNumber
	297
	Short
	n/a
	1 of 2
	01 29 00 03 00 00 00 02 00 00 00 02

	00E2
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	01 31 00 02 00 00 00 41 00 00 01 86 

	
	
	
	
	
	
	Next IFD offset 00 00 EF FA

	IFD 0  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	00F2
	DocumentName
	Test Case D03F
	54 65 73 74 20 63 61 73 65 20 44 30
33 46 00

	000102


000112


000122
	ImageDescription
	Page 1: French text


Page 2: Kanji
	50 61 67 65 20 31 3A 20 46 72 65 6E 63 68 20 74

65 78 74 20 50 61 67 65 20 32 3A 20 6B 61 6E 6A

69 00

	000126


000136


000146
	StripOffset
	458 690 8262 18314 34710 39266 46030 48586 56794 61370 bytes
	00 00 01 CA 00 00 02 B2 00 00 20 46 00 00 47 8A

00 00 87 96 00 00 99 62 00 00 B3 CE 00 00 BD CA

00 00 DD DA 00 00 EF BA

	00014E


00015E


00016E
	StipByteCounts
	232 7572 10051 16393 4555 6763 2556 8208 4576 62 bytes
	00 00 00 E8 00 00 1D 94 00 00 27 43 00 00 40 09

00 00 11 CB 00 00 1A 6B 00 00 09 FC 00 00 20 10

00 00 11 E0 00 00 00 3E

	000176
	Xresolution 
	400 pixels/inch
	00 00 01 90 00 00 00 01

	00017E
	Yresolution 
	400 pixels/inch
	00 00 01 90 00 00 00 01

	000186


000196


0001A6


0001B6


0001C6
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 30

38 20 30 38 3A 35 38 3A 30 32 20 50 4D 20 39 39

00

	Image data

	Offset
	Description
	Comments

	0001caH - 0002b2H
	IFD 0 Strip 0
	Compressed data

	0002b2H - 002046H
	IFD 0 Strip 1
	Compressed data

	002046H - 004789H
	IFD 0 Strip 2
	Compressed data

	00478aH - 008793H
	IFD 0 Strip 3
	Compressed data

	008796H - 009961H
	IFD 0 Strip 4
	Compressed data

	009962H - 00b3cdH
	IFD 0 Strip 5
	Compressed data

	00b3ceH - 00bdcaH
	IFD 0 Strip 6
	Compressed data

	00bdcaH - 00dddaH
	IFD 0 Strip 7
	Compressed data

	00dddaH - 00efbaH
	IFD 0 Strip 8
	Compressed data

	00efbaH - 00eff8H
	IFD 0 Strip 9
	Compressed data

	IFD1  Offset 61434 (0xeffaH) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	00EFFA
	Number of Directory Entries
	–
	–
	–
	19 entries
	00 13

	00EFFC
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	00 FE 00 04 00 00 00 01 00 00 00 02

	00F008
	ImageWidth
	256
	Long
	1728
	3456 (A4)
	01 00 00 04 00 00 00 01 00 00 0D 80

	00F014
	ImageLength
	257
	Long
	n/a
	4677 lines
	01 01 00 04 00 00 00 01 00 00 12 45 

	00F020
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	01 02 00 03 00 00 00 01 00 01 00 00

	00F02C
	Compression
	259
	Short
	3
	T.6 MMR
	01 03 00 03 00 00 00 01 00 04 00 00

	00F038
	Photometric Interpretation
	262
	Short
	0
	0 is white
	01 06 00 03 00 00 00 01 00 00 00 00

	00F044
	FillOrder
	266
	Short
	2
	MSB first
	01 0A 00 03 00 00 00 01 00 01 00 00

	00F050
	DocumentName
	269
	ASCII
	n/a
	Test case D03F
	01 0D 00 02 00 00 00 0F 00 00 F0 E4

	00F05C
	ImageDescription
	270
	ASCII
	n/a
	Page 1: French text Page 2: kanji
	01 0E 00 02 00 00 00 22 00 00 F0 F4

	00F068
	StripOffsets
	273
	Long
	n/a
	61884 65460 82600 99624 115020 125960 146916 166016 182812 193276 bytes
	01 11 00 04 00 00 00 0A 00 00 F1 18

	00F074
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	01 15 00 03 00 00 00 01 00 01 00 00

	00F080
	RowsPerStrip
	278
	Long
	(ImageLength
	500 rows
	01 16 00 04 00 00 00 01 00 00 01 F4

	00F08C
	StripByteCounts
	279
	Long
	n/a
	3575 17137 17024 15394 10938 20955 19099 16796 10461 48 bytes
	01 17 00 04 00 00 00 0A 00 00 F1 40

	00F098
	Xresolution
	282
	Rational
	204, 200 pixels/inch
	400 pixels/inch
	01 1A 00 05 00 00 00 01 00 00 F1 68

	00F0A4
	Yresolution
	283
	Rational
	200,196,100, 98 pixels/inch
	400 pixels/inch
	01 1B 00 05 00 00 00 01 00 00 F1 70

	00F0B0
	T6Option
	292
	Long
	0, 4
	MMR  (G4) compressed
	01 25 00 04 00 00 00 01 00 00 00  00 


	00F0BC
	ResolutionUnit
	296
	Short
	2
	inches
	01 28 00 03 00 00 00 01 00 02 00 00

	00F0C8
	PageNumber
	297
	Short
	n/a
	2 of 2
	01 29 00 03 00 00 00 02 00 01 00 02

	00F0D4
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	01 31 00 02 00 00 00 41 00 00 F1

	
	
	
	
	
	
	Next IFD offset 78 00 00 00 00

	IFD 1  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	00F0E4
	DocumentName
	Test Case D03F
	54 65 73 74 20 63 61 73 65 20 44 30
33 46 00

	00F0F4


00F104


00F114
	ImageDescription
	Page 1: French text

Page 2: Kanji
	50 61 67 65 20 31 3A 20 46 72 65 6E 63 68 20 74

65 78 74 20 50 61 67 65 20 32 3A 20 6B 61 6E 6A

69 00

	00F118


00F128


00F138
	StripOffset
	61884 65460 82600 99624 115020 125960 146916 166016 182812 193276 bytes
	00 00 F1 BC 00 00 FF B4 00 01 42 A8 00 01 85 28

00 01 C1 4C 00 01 EC 08 00 02 3D E4 00 02 88 80

00 02 CA 1C 00 02 F2 FC

	00F140


00F150


00F160
	StipByteCounts
	3575 17137 17024 15394 10938 20955 19099 16796 10461 48 bytes
	00 00 0D F7 00 00 42 F1 00 00 42 80 00 00 3C 22

00 00 2A BA 00 00 51 DB 00 00 4A 9B 00 00 41 9C

00 00 28 DD 00 00 00 30

	00F168
	Xresolution 
	400 pixels/inch
	00 00 01 90 00 00 00 01

	00F170
	Yresolution 
	400 pixels/inch
	00 00 01 90 00 00 00 01

	00F178


00F188


00F198


00F1A8


00F1B8
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 30

38 20 30 38 3A 35 38 3A 30 32 20 50 4D 20 39 39

00

	Image data

	Offset
	Description
	Comments

	00f1bcH - 00ffb3H
	IFD 0 Strip 0
	Compressed data

	00ffb4H - 0142a5H
	IFD 0 Strip 1
	Compressed data

	0142a8H - 018528H
	IFD 0 Strip 2
	Compressed data

	:
:
	:
:
	:
:

	02f2fcH - 02f32cH
	IFD 0 Strip 9
	Compressed data


IV.2
Profile J coding examples, T.85 (JBIG) compression

A4 page at 300300 pels/25.4 mm, FillOrder is MSB, 0 is white, big endian byte order and without GlobalParameters IFD, based on test case Profile J J1

Table IV.2/T.37 – Coding example of TIFF header, IFD and image data

	Header
	
	
	
	
	File Length 115140 bytes

	Offset
	Description
	
	
	
	Comments
	Coding

	0000
	Octet order
	
	
	
	Big endian order
	4D 4D

	0002
	File identifier
	
	
	
	42, TIFF file
	00 2A

	0004
	1st IFD offset
	
	
	
	Offset of IFD 0 
	00 00 00 08

	IFD0  Offset 8 (0x8H) NextIFDOffset 35298 (0x89e2H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0008
	Number of Directory Entries
	–
	–
	–
	18 entries
	00 12

	000A
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	00 FE 00 04 00 00 00 01 00 00 00 02 

	0016
	ImageWidth
	256
	Long
	1728
	2592, A4
	01 00 00 04 00 00 00 01 00 00 0A 20

	0022
	ImageLength
	257
	Long
	n/a
	3508 lines
	01 01 00 04 00 00 00 01 00 00 0D B4 

	002E
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	01 02 00 03 00 00 00 01 00 01 00 00

	003A
	Compression
	259
	Short
	9
	9, JBIG T.85
	01 03 00 03 00 00 00 01 00 09 00 00

	0046
	Photometric Interpretation
	262
	Short
	0
	0 is white
	01 06 00 03 00 00 00 01 00 00 00 00

	0052
	FillOrder
	266
	Short
	2
	MSB first
	01 0A 00 03 00 00 00 01 00 01 00 00

	005E
	DocumentName
	269
	ASCII
	n/a
	Test case D01J
	01 0D 00 02 00 00 00 0F 00 00 00 E6

	006A
	ImageDescription
	270
	ASCII
	n/a
	Page 1: French text
Page 2: kanji
	01 0E 00 02 00 00 00 22 00 00 00 F6

	0076
	StripOffsets
	273
	Long
	n/a
	366 bytes
	01 11 00 04 00 00 00 01 00 00 01 6E

	0082
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	01 15 00 03 00 00 00 01 00 01 00 00

	008E
	RowsPerStrip
	278
	Long
	(ImageLength
	3508 rows
	01 16 00 04 00 00 00 01 00 00 0D B4

	009A
	StripByteCounts
	279
	Long
	n/a
	34933 bytes
	01 17 00 04 00 00 00 01 00 00 88 75

	00A6
	Xresolution
	282
	Rational
	204, 200
	300 pixels/inch
	01 1A 00 05 00 00 00 01 00 00 01 1A

	00B2
	Yresolution
	283
	Rational
	200,196,100, 98
	300 pixels/inch
	01 1B 00 05 00 00 00 01 00 00 01 22


	00BE
	ResolutionUnit
	296
	Short
	2
	inch
	01 28 00 03 00 00 00 01 00 02 00 00

	00CA
	PageNumber
	297
	Short
	n/a
	1 of 2
	01 29 00 03 00 00 00 02 00 00 00 02

	00D6
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	01 31 00 02 00 00 00 41 00 00 01 2A

	
	
	
	
	
	
	Next IFD offset 00 00 89 E6

	IFD 0 Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	0000E6
	DocumentName
	Test Case D01J
	54 65 73 74 20 63 61 73 65 20 44 30 31 4A 00

	0000F6


000106


000116
	ImageDescription
	Page 1: French text


Page 2: Kanji
	50 61 67 65 20 31 3A 20 46 72 65 6E 63 68 20 74

65 78 74 20 50 61 67 65 20 32 3A 20 6B 61 6E 6A

69 00

	00011A
	Xresolution 
	300 pixels/inch
	00 00 01 2C 00 00 00 01

	000122
	Yresolution 
	300 pixels/inch
	00 00 01 2C 00 00 00 01

	00012A


00013A


00014A


00015A


00016A
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 30

38 20 30 38 3A 35 38 3A 30 32 20 50 4D 20 39 39

00

	Image data

	Offset
	Description
	Comments

	00016eH - 0089e3H
	IFD 0 Strip 0
	Compressed data

	IFD1  Offset 35302 (0x89e6H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0089E6
	Number of Directory Entries
	–
	–
	–
	18 entries
	00 12

	0089E8
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	00 FE 00 04 00 00 00 01 00 00 00 02 

	0089F4
	ImageWidth
	256
	Long
	1728
	2592, A4
	01 00 00 04 00 00 00 01 00 00 0A 20

	008a00
	ImageLength
	257
	Long
	n/a
	3508 lines
	01 01 00 04 00 00 00 01 00 00 0D B4 

	008A0C
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	01 02 00 03 00 00 00 01 00 01 00 00

	008A18
	Compression
	259
	Short
	9
	9, JBIG T.85
	01 03 00 03 00 00 00 01 00 09 00 00

	008A24
	Photometric Interpretation
	262
	Short
	0
	0 is white
	01 06 00 03 00 00 00 01 00 00 00 00

	008A30
	FillOrder
	266
	Short
	2
	MSB first
	01 0A 00 03 00 00 00 01 00 01 00 00

	008A3C
	DocumentName
	269
	ASCII
	n/a
	Test case D01J
	01 0D 00 02 00 00 00 0F 00 00 8A C4

	008A48
	ImageDescription
	270
	ASCII
	n/a
	Page 1: French text
Page 2: kanji
	01 0E 00 02 00 00 00 22 00 00 8A D4

	008A54
	StripOffsets
	273
	Long
	n/a
	35660 bytes
	01 11 00 04 00 00 00 01 00 00 8B 4C

	008A60
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	01 15 00 03 00 00 00 01 00 01 00 00

	008A6C
	RowsPerStrip
	278
	Long
	(ImageLength
	3508 rows
	01 16 00 04 00 00 00 01 00 00 0D B4

	008A78
	StripByteCounts
	279
	Long
	n/a
	79480 bytes
	01 17 00 04 00 00 00 01 00 01 36 78

	008A84
	Xresolution
	282
	Rational
	204, 200
	300 pixels/inch
	01 1A 00 05 00 00 00 01 00 00 8A F8

	008A90
	Yresolution
	283
	Rational
	200,196,100, 98
	300 pixels/inch
	01 1B 00 05 00 00 00 01 00 00 8B 00

	008A9C
	ResolutionUnit
	296
	Short
	2
	inch
	01 28 00 03 00 00 00 01 00 02 00 00

	008AA8
	PageNumber
	297
	Short
	n/a
	2 of 2
	01 29 00 03 00 00 00 02 00 01 00 02

	008AB4
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	01 31 00 02 00 00 00 41 00 00 8B 08

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	008AC4
	DocumentName
	Test Case D01J
	54 65 73 74 20 63 61 73 65 20 44 30
31 4A 00

	008AD4


008AE4


008AF4
	ImageDescription
	Page 1: French text


Page 2: Kanji
	50 61 67 65 20 31 3A 20 46 72 65 6E 63 68 20 74

65 78 74 20 50 61 67 65 20 32 3A 20 6B 61 6E 6A

69 00

	008AF8
	Xresolution 
	300 pixels/inch
	00 00 01 2C 00 00 00 01

	008B00
	Yresolution 
	300 pixels/inch
	00 00 01 2C 00 00 00 01

	008B08


008B18


008B28


008B38


008B48
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 30

38 20 30 38 3A 35 38 3A 30 32 20 50 4D 20 39 39

00

	Image data

	Offset
	Description
	Comments

	008b4cH - 01c1c4H
	IFD 1 Strip 0
	Compressed data


IV.3
Profile C coding example, JPEG compression and T.4 Annex E (L*a*b*) encoding

A4 page at 200200 pels/25.4 mm, 3 samples per pixel, 8 bits per sample, 4:1:1 subsampling, FillOrder is MSB, little endian byte order, based on test case Profile C C3

Table IV.3/T.37 – Coding example of TIFF header, IFD and image data

	Header
	
	
	
	
	File Length 885062 bytes

	Offset
	Description
	
	
	
	Comments
	Coding

	0000
	Octet order
	
	
	
	Little endian order
	49 49

	0002
	File identifier
	
	
	
	42, TIFF file
	2A 00

	0004
	1st IFD offset
	
	
	
	Offset of IFD 0 
	08 00 00 00

	IFD0  Offset 8 (0x8H) NextIFDOffset 508108 (0x7c0ccH)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0008
	Number of Directory Entries
	–
	–
	–
	20 entries
	14 00

	000A
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	FE 00 04 00 01 00 00 00 02 00 00 00

	0016
	ImageWidth
	256
	Long
	1728
	A4
	00 01 04 00 01 00 00 00 C0 06 00 00

	0022
	ImageLength
	257
	Long
	n/a
	2342 lines
	01 01 04 00 01 00 00 00 26 09 00 00 

	002E
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 B6 C1 07 00

	003A
	Compression
	259
	Short
	7
	JPEG
	03 01 03 00 01 00 00 00 07 00 00 00

	0046
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	0052
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	005E
	DocumentName
	269
	ASCII
	n/a
	Test case D03C
	0D 01 02 00 0F 00 00 00 06 01 00 00

	006A
	ImageDescription
	270
	ASCII
	n/a
	Page 1: Color test chart
Page2: Continuous tone fax test chart
	0E 01 02 00 41 00 00 00 16 01 00 00

	0076
	StripOffsets
	273
	Long
	n/a
	484 bytes
	11 01 04 00 01 00 00 00 E4 01 00 00

	0082
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	008E
	RowsPerStrip
	278
	Long
	(ImageLength
	2342 rows
	16 01 04 00 01 00 00 00 26 09 00 00

	009A
	StripByteCounts
	279
	Long
	n/a
	507622 bytes
	17 01 04 00 01 00 00 00 E6 BE 07 00

	00A6
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 5A 01 00 00

	00B2
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 62 01 00 00


	00BE
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	00CA
	PageNumber
	297
	Short
	n/a
	1 of 2
	29 01 03 00 02 00 00 00 00 00 02 00

	00D6
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	31 01 02 00 41 00 00 00 6A 01 00 00


	00E2
	GlobalParametersIFD
	400
	Long
	n/a
	Offset 430
	90 01 04 00 01 00 00 00 AE 01 00 00

	00EE
	ChromaSubSampling
	530
	Short
	4:1:1
	4:1:1
	12 02 03 00 02 00 00 00 02 00 02 00

	
	
	
	
	
	
	Next IFD offset CC C0 07 00

	IFD 0  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	0000FE
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	000106
	DocumentName
	Test Case D03C
	54 65 73 74 20 63 61 73 65 20 44 30
33 43 00

	000116


000126


000136


000146


000156
	ImageDescription
	Page 1: Color test chart


Page 2: Continuous tone fax test chart
	50 61 67 65 20 31 3A 20 43 6F 6C 6F 72 20 74 65

73 74 20 63 68 61 72 74 20 20 50 61
67 65 20 32

3A 20 43 6F 6E 74 69 6E 75 6F 75 73 20 74 6F 6E

65 20 66 61 78 20 74 65 73 74 20 63
68 61 72 74

00

	00015A
	Xresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	000162
	Yresolution 
	 200 pixels/inch
	C8 00 00 00 01 00 00 00

	00016A


00017A


00018A


00019A


0001AA
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 30

38 20 30 38 3A 35 38 3A 30 32 20 50 4D 20 39 39

00

	IFD 0 ( GP IFD (  Child IFD 0 Offset 430 (0x1aeH) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0001AE
	Number of Directory Entries
	---
	---
	---
	4 entries
	04 00

	0001B0
	ProfileType
	401
	Long
	1
	1 (Group 3 fax)
	91 01 04 00 01 00 00 00 01 00 00 00 

	0001BC
	FaxProfile
	402
	Byte
	4
	Lossy color and grayscale, Profile C
	92 01 01 00 01 00 00 00 04 00 00 00

	0001C8
	CodingMethod
	403
	Long
	32
	T.81 (JPEG)
	93 01 04 00 01 00 00 00 20 00 00 00

	0001D4
	VersionYear
	404
	Byte
	n/a
	1998
	94 01 01 00 04 00 00 00 31 39 39 38

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	Image data

	Offset
	Description
	Comments

	0001e4H - 07c0caH
	IFD 0 Strip 0
	Compressed data

	IFD1  Offset 508108 (0x7c0ccH) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	07C0CC
	Number of Directory Entries
	–
	–
	–
	19 entries
	13 00

	07C0CE
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	FE 00 04 00 01 00 00 00 02 00 00 00

	07C0DA
	ImageWidth
	256
	Long
	1728
	A4
	00 01 04 00 01 00 00 00 C0 06 00 00

	07C0E6
	ImageLength
	257
	Long
	n/a
	1949 lines
	01 01 04 00 01 00 00 00 9D 07 00 00 

	07C0F2
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 BA C1 07 00

	07C1FE
	Compression
	259
	Short
	7
	JPEG
	03 01 03 00 01 00 00 00 07 00 00 00

	07C10A
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	07C116
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	07C122
	DocumentName
	269
	ASCII
	n/a
	Test case D03C
	0D 01 02 00 0F 00 00 00 BE C1 07 00

	07C12E
	ImageDescription
	270
	ASCII
	n/a
	Page 1: Color test chart Page2: Continuous tone fax test chart
	0E 01 02 00 41 00 00 00 CE C1 07 00

	07C13A
	StripOffsets
	273
	Long
	n/a
	508518 bytes
	11 01 04 00 01 00 00 00 66 C2 07 00

	07C146
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	07C152
	RowsPerStrip
	278
	Long
	(ImageLength
	1949 rows
	16 01 04 00 01 00 00 00 9D 07 00 00

	07C15E
	StripByteCounts
	279
	Long
	n/a
	376541 bytes
	17 01 04 00 01 00 00 00 DD BE 05 00

	07C16A
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 12 C2 07 00 

	07C176
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 1A C2 07 00

	07C182
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	07C18E
	PageNumber
	297
	Short
	n/a
	2 of 2
	29 01 03 00 02 00 00 00 01 00 02 00

	07C19A
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	31 01 02 00 41 00 00 00 22 C2 07 00

	07C1A6
	ChromaSubSampling
	530
	Short
	4:1:1
	4:1:1
	12 02 03 00 02 00 00 00 02 00 02 00 

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	07C1B6
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	07C1BE
	DocumentName
	Test Case D03C
	54 65 73 74 20 63 61 73 65 20 44 30
33 43 00

	07C1CE


07C1DE


07C1EE


07C1FE


07C20E
	ImageDescription
	Page 1: Color test chart


Page 2: Continuous tone fax test chart
	50 61 67 65 20 31 3A 20 43 6F 6C 6F 72 20 74 65

73 74 20 63 68 61 72 74 20 20 50 61
67 65 20 32

3A 20 43 6F 6E 74 69 6E 75 6F 75 73 20 74 6F 6E

65 20 66 61 78 20 74 65 73 74 20 63
68 61 72 74

00

	07C212
	Xresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	07C21A
	Yresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	07C222


07C232


07C242


07C252


07C262
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 30

38 20 30 38 3A 35 38 3A 30 32 20 50 4D 20 39 39

00

	Image data

	Offset
	Description
	Comments

	07c266H - 0d8143H
	IFD 1 Strip 0
	Compressed data


IV.4
Profile L coding examples, JBIG compression with T.43 color encoding

IV.4.1
One bit per sample CMYK encoding of color

A4 page at 400400 pels/25.4 mm, 3 samples per pixel, FillOrder is MSB, big endian byte order, based on test case Profile L L2

Table IV.4-1/T.37 – Coding example of TIFF header, IFD and image data

	Header
	
	
	
	
	File Length 1583262 bytes

	Offset
	Description
	
	
	
	Comments
	Coding

	0000
	Octet order
	
	
	
	Big endian order
	4D 4D

	0002
	File identifier
	
	
	
	42, TIFF file
	00 2A

	0004
	1st IFD offset
	
	
	
	Offset of IFD 0 
	00 00 00 08

	IFD0  Offset 8 (0x8H) NextIFDOffset 1053616 (0x1013b0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0008
	Number of Directory Entries
	–
	–
	–
	19 entries
	00 13

	000A
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	00 FE 00 04 00 00 00 01 00 00 00 02

	0016
	ImageWidth
	256
	Long
	3456
	A4
	01 00 00 04 00 00 00 01 00 00 0D 80

	0022
	ImageLength
	257
	Long
	n/a
	4684 lines
	01 01 00 04 00 00 00 01 00 00 12 4C

	002E
	BitsPerSample
	258
	Short
	1
	1, 1, 1 bit/sample
	01 02 00 03 00 00 00 03 00 00 00 F2

	003A
	Compression
	259
	Short
	10
	T.43 JBIG color
	01 03 00 03 00 00 00 01 00 0A 00 00

	0046
	Photometric Interpretation
	262
	Short
	5
	CMYK
	01 06 00 03 00 00 00 01 00 05 00 00

	0052
	FillOrder
	266
	Short
	2
	MSB first
	01 0A 00 03 00 00 00 01 00 01 00 00

	005E
	DocumentName
	269
	ASCII
	n/a
	Test case L2
	01 0D 00 02 00 00 00 0D 00 00 00 FA

	006A
	ImageDescription
	270
	ASCII
	n/a
	Page 1: Color test chart Page2: Bar chart
	01 0E 00 02 00 00 00 2C 00 00 01 0A

	0076
	StripOffsets
	273
	Long
	n/a
	448 bytes
	01 11 00 04 00 00 00 01 00 00 01 C0

	0082
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	01 15 00 03 00 00 00 01 00 03 00 00

	008E
	RowsPerStrip
	278
	Long
	(ImageLength
	4684 rows
	01 16 00 04 00 00 00 01 00 00 12 4C

	009A
	StripByteCounts
	279
	Long
	n/a
	1053166 bytes
	01 17 00 04 00 00 00 01 00 10 11 EE

	00A6
	Xresolution
	282
	Rational
	200
	400 pixels/inch
	01 1A 00 05 00 00 00 01 00 00 01 36

	00B2
	Yresolution
	283
	Rational
	200
	400 pixels/inch
	01 1B 00 05 00 00 00 01 00 00 01 3E


	00BE
	ResolutionUnit
	296
	Short
	2
	inch
	01 28 00 03 00 00 00 01 00 02 00 00

	00CA
	PageNumber
	297
	Short
	n/a
	1 of 2
	01 29 00 03 00 00 00 02 00 00 00 02

	00D6
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	01 31 00 02 00 00 00 41 00 00 01 46


	00E2
	GlobalParametersIFD
	400
	Long
	n/a
	
	01 90 00 04 00 00 00 01 00 00 01 8A

	
	
	
	
	
	
	Next IFD offset 00 10 13 B0

	IFD 0  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	0000F2
	BitsPerSample
	1, 1, 1
	00 01 00 01 00 01

	0000FA
	DocumentName
	Test Case L2
	54 65 73 74 20 63 61 73 65 20 4C 32 00

	00010A


00011A


00012A
	ImageDescription
	Page 1: Color test chart


Page 2: Bar chart
	50 61 67 65 20 31 3A 20 43 6F 6C 6F 72 20 74 65

73 74 20 63 68 61 72 74 20 20 50 61 67 65 20 32

3A 20 42 61 72 20 63 68 61 72 74 00

	000136
	Xresolution 
	400 pixels/inch
	00 00 01 90 00 00 00 01

	00013E
	Yresolution 
	400 pixels/inch
	00 00 01 90 00 00 00 01

	000146


000156


000166


000176


000186
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 30

38 20 30 38 3A 35 38 3A 30 32 20 50 4D 20 39 39

00

	IFD 0 ( GP IFD (  Child IFD 0 Offset 394 (0x18aH) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	00018A
	Number of Directory Entries
	–
	–
	–
	4 entries
	00 04

	00018C
	ProfileType
	401
	Long
	1
	1 (Group 3 fax)
	01 91 00 04 00 00 00 01 00 00 00 01

	000198
	FaxProfile
	402
	Byte
	5
	Lossless color and grayscale, Profile L
	01 92 00 01 00 00 00 01 05 00 00 00

	0001A4
	CodingMethod
	403
	Long
	64
	T.82 (JBIG), using T.43
	01 93 00 04 00 00 00 01 00 00 00 40

	0001B0
	VersionYear
	404
	Byte
	n/a
	1998
	01 94 00 01 00 00 00 04 31 39 39 38

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	Image data

	Offset
	Description
	Comments

	000001c0H - 001013aeH
	IFD 0 Strip 0
	Compressed data

	IFD1  Offset 1053616 (0x1013b0H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	001013B0
	Number of Directory Entries
	–
	–
	–
	18 entries
	00 12

	001013B2
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	00 FE 00 04 00 00 00 01 00 00 00 02

	001013BE
	ImageWidth
	256
	Long
	3456
	A4
	01 00 00 04 00 00 00 01 00 00 0D 80

	001013CA
	ImageLength
	257
	Long
	n/a
	4472 lines
	01 01 00 04 00 00 00 01 00 00 11 78

	001013D6
	BitsPerSample
	258
	Short
	1
	1, 1, 1 bit/sample
	01 02 00 03 00 00 00 03 00 10 14 8E

	001013E2
	Compression
	259
	Short
	10
	T.43 JBIG color
	01 03 00 03 00 00 00 01 00 0A 00 00

	001013EE
	Photometric Interpretation
	262
	Short
	5
	CMYK
	01 06 00 03 00 00 00 01 00 05 00 00

	001013FA
	FillOrder
	266
	Short
	2
	MSB first
	01 0A 00 03 00 00 00 01 00 01 00 00

	00101406
	DocumentName
	269
	ASCII
	n/a
	Test case L2
	01 0D 00 02 00 00 00 0D 00 10 14 96

	00101412
	ImageDescription
	270
	ASCII
	n/a
	Page 1: Color test chart Page2: Bar chart
	01 0E 00 02 00 00 00 2C 00 10 14 A6

	0010141E
	StripOffsets
	273
	Long
	n/a
	1053990 bytes
	01 11 00 04 00 00 00 01 00 10 15 26

	0010142A
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	01 15 00 03 00 00 00 01 00 03 00 00

	00101436
	RowsPerStrip
	278
	Long
	(ImageLength
	4472 rows
	01 16 00 04 00 00 00 01 00 00 11 78

	00101442
	StripByteCounts
	279
	Long
	n/a
	529270 bytes
	01 17 00 04 00 00 00 01 00 08 13 76

	0010144E
	Xresolution
	282
	Rational
	200
	400 pixels/inch
	01 1A 00 05 00 00 00 01 00 10 14 D2

	0010145A
	Yresolution
	283
	Rational
	200
	400 pixels/inch
	01 1B 00 05 00 00 00 01 00 10 14 DA

	00101466
	ResolutionUnit
	296
	Short
	2
	inch
	01 28 00 03 00 00 00 01 00 02 00 00

	00101472
	PageNumber
	297
	Short
	n/a
	2 of 2
	01 29 00 03 00 00 00 02 00 00 00 02

	0010147E
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	01 31 00 02 00 00 00 41 00 10 14 E2

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	0010148E
	BitsPerSample
	1, 1, 1
	00 01 00 01 00 01

	00101496
	DocumentName
	Test Case L2
	54 65 73 74 20 63 61 73 65 20 4C 32 00

	001014A6


001014B6


001014C6
	ImageDescription
	Page 1: Color test chart


Page 2: Bar chart
	50 61 67 65 20 31 3A 20 43 6F 6C 6F 72 20 74 65

73 74 20 63 68 61 72 74 20 20 50 61 67 65 20 32

3A 20 42 61 72 20 63 68 61 72 74 00

	001014D2
	Xresolution 
	400 pixels/inch
	00 00 01 90 00 00 00 01

	001014DA
	Yresolution 
	400 pixels/inch
	00 00 01 90 00 00 00 01

	001014E2


001014F2


00101502


00101512


00101522
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 30

38 20 30 38 3A 35 38 3A 30 32 20 50 4D 20 39 39

00

	Image data

	Offset
	Description
	Comments

	00101526H - 0018289cH
	IFD 1 Strip 0
	Compressed data


IV.4.2
Eight bits per sample ITU L*a*b* palette color encoding

A4 page at 200200 pels/25.4 mm, 3 samples per pixel, 4:1:1 subsampling, FillOrder is MSB, little endian byte order, based on test case Profile L L4. Note that the ColorMap tag (i.e. tag code 320), present in this sample, is no longer used and may be ignored.

Table IV.4-2/T.37 – Coding example of TIFF header, IFD and image data

	Header
	
	
	
	
	File Length 1207546 bytes

	Offset
	Description
	
	
	
	Comments
	Coding

	0000
	Octet order
	
	
	
	Little endian order
	49 49

	0002
	File identifier
	
	
	
	42, TIFF file
	2A 00

	0004
	1st IFD offset
	
	
	
	Offset of IFD 0 
	08 00 00 00

	IFD0  Offset 8 (0x8H) NextIFDOffset 1166376 (0x11cc28H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0008
	Number of Directory Entries
	–
	–
	–
	21 entries
	15 00

	000A
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	FE 00 04 00 01 00 00 00 02 00 00 00

	0016
	ImageWidth
	256
	Long
	1728
	A4
	00 01 04 00 01 00 00 00 C0 06 00 00

	0022
	ImageLength
	257
	Long
	n/a
	2342 lines
	01 01 04 00 01 00 00 00 26 09 00 00 

	002E
	BitsPerSample
	258
	Short
	1
	8 bit/sample
	02 01 03 00 01 00 00 00 08 00 00 00

	003A
	Compression
	259
	Short
	10
	T.43 JBIG color
	03 01 03 00 01 00 00 00 0A 00 00 00

	0046
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	0052
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	005E
	DocumentName
	269
	ASCII
	n/a
	Test case L4
	0D 01 02 00 0D 00 00 00 0A 01 00 00

	006A
	ImageDescription
	270
	ASCII
	n/a
	Page 1: Color test chart Page2: Bar chart
	0E 01 02 00 2C 00 00 00 1A 01 00 00

	0076
	StripOffsets
	273
	Long
	n/a
	2000 bytes
	11 01 04 00 01 00 00 00 D0 07 00 00

	0082
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	15 01 03 00 01 00 00 00 01 00 00 00

	008E
	RowsPerStrip
	278
	Long
	(ImageLength
	2342 rows
	16 01 04 00 01 00 00 00 26 09 00 00

	009A
	StripByteCounts
	279
	Long
	n/a
	1164373 bytes
	17 01 04 00 01 00 00 00 55 C4 11 00

	00A6
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 46 01 00 00

	00B2
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 4E 01 00 00


	00BE
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	00CA
	PageNumber
	297
	Short
	n/a
	1 of 2
	29 01 03 00 02 00 00 00 00 00 02 00

	00D6
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	31 01 02 00 41 00 00 00 56 01 00 00

	00E2
	ColorMap
	320
	Short
	n/a
	410 offset to ITU L*a*b* sample values
	40 01 03 00 00 03 00 00 9A 01 00 00

	00EE
	Indexed
	346
	Short
	n/a
	1, palette color image
	5A 01 03 00 01 00 00 00 01 00 00 00 


	00FA
	GlobalParametersIFD
	400
	Long
	n/a
	offset 1946
	90 01 04 00 01 00 00 00 9A 07 00 00

	
	
	
	
	
	
	Next IFD offset 28 CC 11 00

	IFD 0  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	00010A
	DocumentName
	Test Case L4
	54 65 73 74 20 63 61 73 65 20 4C 34 00

	00011A


00012A


00013A
	ImageDescription
	Page 1: Color test chart


Page 2: Bar chart
	50 61 67 65 20 31 3A 20 43 6F 6C 6F 72 20 74 65

73 74 20 63 68 61 72 74 20 20 50 61 67 65 20 32

3A 20 42 61 72 20 63 68 61 72 74 00

	000146
	Xresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	00014E
	Yresolution 
	 200 pixels/inch
	C8 00 00 00 01 00 00 00

	000156


000166


000176


000186


000196
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 30

38 20 30 38 3A 35 38 3A 30 32 20 50 4D 20 39 39

00

	00019A


0001AA


0001BA


:

:

00078A
	ColorMap
	
	00 00 00 80 00 60 00 C5 00 A5 00 85 00 8E 00 2C

00 9B 00 FD 00 7E 00 66 00 FC 00 7C 00 5F 00 FE

00 80 00 60 00 20 00 81 00 3E 00 9F 00 63 00 25

:

:

00 76 00 52 00 AD 00 80 00 60 00 00 00 80 00 60

	IFD 0 ( GP IFD (  Child IFD 0 Offset 1946 (0x79aH) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	00079A
	Number of Directory Entries
	–
	–
	–
	4 entries
	04 00

	00079C
	ProfileType
	401
	Long
	1
	1 (Group 3 fax)
	91 01 04 00 01 00 00 00 01 00 00 00 

	0007A8
	FaxProfile
	402
	Byte
	5
	Lossless color and grayscale, Profile L
	92 01 01 00 01 00 00 00 05 00 00 00

	0007B4
	CodingMethod
	403
	Long
	64
	T.82 (JBIG), using T.43
	93 01 04 00 01 00 00 00 40 00 00 00

	0007C0
	VersionYear
	404
	Byte
	n/a
	1998
	94 01 01 00 04 00 00 00 31 39 39 38

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	Image data

	Offset
	Description
	Comments

	0007d0H - 0011cc25H
	IFD 0 Strip 0
	Compressed data

	IFD1  Offset 1166376 (0x11cc28H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0011CC28
	Number of Directory Entries
	–
	–
	–
	20 entries
	14 00

	0011CC2A
	NewSubfileType
	254
	Long
	2, Bit1=1
	single page of a multi-page
	FE 00 04 00 01 00 00 00 02 00 00 00

	0011CC36
	ImageWidth
	256
	Long
	1728
	A4
	00 01 04 00 01 00 00 00 C0 06 00 00

	0011CC42
	ImageLength
	257
	Long
	n/a
	2236 lines
	01 01 04 00 01 00 00 00 BC 08 00 00 

	0011CC4E
	BitsPerSample
	258
	Short
	1
	8 bit/sample
	02 01 03 00 01 00 00 00 08 00 00 00

	0011CC5A
	Compression
	259
	Short
	10
	T.43 JBIG color
	03 01 03 00 01 00 00 00 0A 00 00 00

	0011CC66
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	0011CC72
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	0011CC7E
	DocumentName
	269
	ASCII
	n/a
	Test case L4
	0D 01 02 00 0D 00 00 00 1E CD 11 00

	0011CC8A
	ImageDescription
	270
	ASCII
	n/a
	Page 1: Color test chart Page2: Bar chart
	0E 01 02 00 2C 00 00 00 2E CD 11 00

	0011CC96
	StripOffsets
	273
	Long
	n/a
	1168302 bytes
	11 01 04 00 01 00 00 00 AE D3 11 00

	0011CCA2
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	15 01 03 00 01 00 00 00 01 00 00 00

	0011CCAE
	RowsPerStrip
	278
	Long
	(ImageLength
	2236 rows
	16 01 04 00 01 00 00 00 BC 08 00 00

	0011CCBA
	StripByteCounts
	279
	Long
	n/a
	39242 bytes
	17 01 04 00 01 00 00 00 4A 99 00 00

	0011CCC6
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 5A CD 11 00

	0011CCD2
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 62 CD 11 00

	0011CCDE
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	0011CCEA
	PageNumber
	297
	Short
	n/a
	2 of 2
	29 01 03 00 02 00 00 00 01 00 02 00

	0011CCF6
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	31 01 02 00 41 00 00 00 6A CD 11 00

	0011CD02
	ColorMap
	320
	Short
	n/a
	1166766 offset to ITU L*a*b* sample values
	40 01 03 00 00 03 00 00 AE CD 11 00

	0011CD0E
	Indexed
	346
	Short
	n/a
	1, palette color image
	5A 01 03 00 01 00 00 00 01 00 00 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1 Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	0011CD1E
	DocumentName
	Test Case L4
	54 65 73 74 20 63 61 73 65 20 4C 34 00

	0011CD2E


0011CD3E


0011CD4E
	ImageDescription
	Page 1: Color test chart


Page 2: Bar chart
	50 61 67 65 20 31 3A 20 43 6F 6C 6F 72 20 74 65

73 74 20 63 68 61 72 74 20 20 50 61 67 65 20 32

3A 20 42 61 72 20 63 68 61 72 74 00

	0011CD5A
	Xresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	0011CD62
	Yresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	0011CD6A


0011CD7A


0011CD8A


0011CD9A


0011CDAA
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 08 08:58:02 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 30

38 20 30 38 3A 35 38 3A 30 32 20 50 4D 20 39 39

00

	0011CDAE


0011CDBE





0011CDCE



0011D39E
	ColorMap
	
	00 00 00 80 00 60 00 92 00 F3 00 B4 00 F8 00 6A

00 CF 00 FE 00 80 00 60 00 9D 00 80 00 60 00 C2

00 80 00 60 00 BD 00 80 00 60 00 DB 00 06 00 BE

:

:

00 80 00 60 00 00 00 80 00 60 00 2C 00 B5 00 81

	Image data

	Offset
	Description
	Comments

	0011d3aeH - 00126cf8H
	IFD 1 Strip 0
	Compressed data


IV.5
Profile M coding examples

IV.5.1
Three layers & single strip per layer: Mask with T.6 (MMR) compression, Background and Foreground with JPEG compression and T.4 Annex E ITU L*a*b* color encoding

3-layer (BG/M/FG), 100/400/200 resolution, JPEG/MMR/JPEG compression, ChromaSubsampling BG=1:1:1 while FG=4:1:1, single strip image, FG default not black, MSB FillOrder, little-endian, based on test case Profile M M2

Table IV.5-1/T.37 – Coding example of TIFF header, IFD and image data

	Header
	
	
	
	
	File Length 374882 bytes

	Offset
	Description
	
	
	
	Comments
	Coding

	0000
	Octet order
	
	
	
	Little endian order
	49 49

	0002
	File identifier
	
	
	
	42, TIFF file
	2A 00

	0004
	1st IFD offset
	
	
	
	Offset of IFD 0 
	08 00 00 00

	IFD0  (Mask)  Offset 8 (0x8H) NextIFDOffset 196772 (0x300a4H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0008
	Number of Directory Entries
	–
	–
	–
	24 entries
	18 00

	000A
	NewSubfileType
	254
	Long
	18
	primary IFD of a single page of a multi-page 
	FE 00 04 00 01 00 00 00 12 00 00 00

	0016
	ImageWidth
	256
	Long
	3456
	A4
	00 01 04 00 01 00 00 00 80 0D 00 00

	0022
	ImageLength
	257
	Long
	n/a
	4400 lines
	01 01 04 00 01 00 00 00 30 11 00 00 

	002E
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	02 01 03 00 03 00 00 00 01 00 00 00

	003A
	Compression
	259
	Short
	4
	4, T.6 MMR
	03 01 03 00 01 00 00 00 04 00 00 00

	0046
	Photometric Interpretation
	262
	Short
	0
	0 is white
	06 01 03 00 01 00 00 00 00 00 00 00

	0052
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	005E
	DocumentName
	269
	ASCII
	n/a
	Test case M02
	0D 01 02 00 0E 00 00 00 2E 01 00 00 

	006A
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 3E 01 00 00

	0076
	StripOffsets
	273
	Long
	n/a
	1400 bytes
	11 01 04 00 01 00 00 00 78 05 00 00

	0082
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	15 01 03 00 01 00 00 00 01 00 00 00

	008E
	RowsPerStrip
	278
	Long
	(ImageLength
	4400 rows
	16 01 04 00 01 00 00 00 30 11 00 00

	009A
	StripByteCounts
	279
	Long
	n/a
	36943 bytes
	17 01 04 00 01 00 00 00 4F 90 00 00

	00A6
	Xresolution
	282
	Rational
	200
	400 pixels/inch
	1A 01 05 00 01 00 00 00 5E 01 00 00 

	00B2
	Yresolution
	283
	Rational
	200
	400 pixels/inch
	1B 01 05 00 01 00 00 00 66 01 00 00

	00BE
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 6E 01 00 00

	00CA
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 76 01 00 00

	00D6
	T6Option
	293
	Long
	0
	MMR  (G4) compressed
	25 01 04 00 01 00 00 00 00 00 00 00


	00E2
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	00EE
	PageNumber
	297
	Short
	n/a
	1 of 2
	29 01 03 00 02 00 00 00 00 00 02 00

	00FA
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 7E 01 00 00

	0106
	SubIFDs
	330
	Long
	n/a
	Offsets of 466 and 900
	4A 01 04 00 02 00 00 00 C2 01 00 00


	0112
	GlobalParametersIFD
	400
	Long
	n/a
	1334 offset
	90 01 04 00 01 00 00 00 36 05 00 00

	011E
	ImageLayer
	34732
	Long
	2, 1
	2 (mask), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 CA 01 00 00

	
	
	
	
	
	
	Next IFD offset A4 00 03 00

	IFD 0 (Mask)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	00012E
	DocumentName
	Test case M02
	54 65 73 74 20 43 61 73 65 20 4D 30 32 00

	00013E


00014E
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	00015E
	Xresolution 
	400 pixels/inch
	90 01 00 00 01 00 00 00

	000166
	Yresolution 
	400 pixels/inch
	90 01 00 00 01 00 00 00

	00016E
	XPosition
	0 offset
	00 00 00 00 90 01 00 00 

	000176
	YPosition
	0 offset
	00 00 00 00 90 01 00 00

	00017E


00018E


00019E


0001AE


0001BE
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	0001C2
	SubIFDs
	Offsets of 466 and 900
	D2 01 00 00 84 03 00 00

	0001CA
	ImageLayer
	2 (mask), 1 (first image to be imaged)
	02 00 00 00 01 00 00 00

	IFD 0 ( SubIFD 0 ( Child IFD 0 (Bkgnd)  Offset 466 (0x1d2H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0001D2
	Number of Directory Entries
	–
	–
	–
	22 entries
	16 00

	0001D4
	NewSubfileType
	254
	Long
	16
	Child IFD (background) of a single page of a multi-page
	FE 00 04 00 01 00 00 00 10 00 00 00

	0001E0
	ImageWidth
	256
	Long
	864
	A4
	00 01 04 00 01 00 00 00 60 03 00 00

	0001EC
	ImageLength
	257
	Long
	n/a
	1100 lines
	01 01 04 00 01 00 00 00 4C 04 00 00 

	0001F8
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 E0 02 00 00

	000204
	Compression
	259
	Short
	7
	JPEG
	03 01 03 00 01 00 00 00 07 00 00 00

	000210
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	00021C
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	000228
	DocumentName
	269
	ASCII
	n/a
	Test case M02
	0D 01 02 00 0E 00 00 00 E8 02 00 00

	000234
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 F8 02 00 00

	000240
	StripOffsets
	273
	Long
	n/a
	38344 bytes
	11 01 04 00 01 00 00 00 C8 95 00 00

	00024C
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	000258
	RowsPerStrip
	278
	Long
	(ImageLength
	1100 rows
	16 01 04 00 01 00 00 00 4C 04 00 00

	000264
	StripByteCounts
	279
	Long
	n/a
	42833 bytes
	17 01 04 00 01 00 00 00 51 A7 00 00

	000270
	Xresolution
	282
	Rational
	200
	100 pixels/inch
	1A 01 05 00 01 00 00 00 18 03 00 00

	00027C
	Yresolution
	283
	Rational
	200
	100 pixels/inch
	1B 01 05 00 01 00 00 00 20 03 00 00

	000288
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 28 03 00 00

	000294
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 30 03 00 00

	0002A0
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	0002AC
	PageNumber
	297
	Short
	n/a
	1 of 2
	29 01 03 00 02 00 00 00 00 00 02 00

	0002B8
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 38 03 00 00

	0002C4
	ChromaSubSampling
	530
	Short
	4:1:1
	1:1:1
	12 02 03 00 02 00 00 00 01 00 01 00

	0002D0
	ImageLayer
	34732
	Long
	1, 1
	1 (background), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 7C 03 00 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 0 ( SubIFD 0 ( Child IFD 0 (Bkgnd)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	0002E0
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	0002E8
	DocumentName
	Test Case M02
	54 65 73 74 20 43 61 73 65 20 4D 30 32 00

	0002F8


000308
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	000318
	Xresolution 
	100 pixels/inch
	64 00 00 00 01 00 00 00

	000320
	Yresolution 
	 100 pixels/inch
	64 00 00 00 01 00 00 00

	000328
	XPosition
	0 offset
	00 00 00 00 90 01 00 00 

	000330
	YPosition
	0 offset
	00 00 00 00 90 01 00 00

	000338


000348


000358


000368


000378
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	00037C
	ImageLayer
	1 (background), 1 (first image to be imaged)
	01 00 00 00 01 00 00 00

	IFD 0 ( SubIFD 1 ( Child IFD 0 (Frgnd)  Offset 900 (0x384H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	000384
	Number of Directory Entries
	–
	–
	–
	22 entries
	16 00

	000386
	NewSubfileType
	254
	Long
	16
	Child IFD (foreground) of a single page of a multi-page
	FE 00 04 00 01 00 00 00 10 00 00 00

	000392
	ImageWidth
	256
	Long
	1728
	A4
	00 01 04 00 01 00 00 00 C0 06 00 00

	00039E
	ImageLength
	257
	Long
	n/a
	2200 lines
	01 01 04 00 01 00 00 00 98 08 00 00 

	0003AA
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 92 04 00 00

	0003B6
	Compression
	259
	Short
	7
	JPEG
	03 01 03 00 01 00 00 00 07 00 00 00

	0003C2
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	0003CE
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	0003DA
	DocumentName
	269
	ASCII
	n/a
	Test case M02
	0D 01 02 00 0E 00 00 00 9A 04 00 00

	0003E6
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 AA 04 00 00

	0003F2
	StripOffsets
	273
	Long
	n/a
	81180 bytes
	11 01 04 00 01 00 00 00 1C 3D 01 00

	0003FE
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	00040A
	RowsPerStrip
	278
	Long
	(ImageLength
	2200 rows
	16 01 04 00 01 00 00 00 98 08 00 00

	000416
	StripByteCounts
	279
	Long
	n/a
	115591 bytes
	17 01 04 00 01 00 00 00 87 C3 01 00

	000422
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 CA 04 00 00

	00042E
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 D2 04 00 00

	00043A
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 DA 04 00 00

	000446
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 E2 04 00 00

	000452
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	00045E
	PageNumber
	297
	Short
	n/a
	1 of 2
	29 01 03 00 02 00 00 00 00 00 02 00

	00046A
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 EA 04 00 00

	000476
	ChromaSubSampling
	530
	Short
	4:1:1
	4:1:1
	12 02 03 00 02 00 00 00 02 00 02 00

	000482
	ImageLayer
	34732
	Long
	3, 1
	3 (foreground), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 2E 05 00 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 0 ( SubIFD 1 ( Child IFD 0 (Frgnd)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	000492
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	00049A
	DocumentName
	Test Case M02
	54 65 73 74 20 43 61 73 65 20 4D 30 32 00

	0004AA


0004BA
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	0004CA
	Xresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	0004D2
	Yresolution 
	 200 pixels/inch
	C8 00 00 00 01 00 00 00

	0004DA
	XPosition
	0 offset
	00 00 00 00 90 01 00 00 

	0004E2
	YPosition
	0 offset
	00 00 00 00 90 01 00 00

	0004EA


0004FA


00050A


00051A


00052A
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	00052E
	ImageLayer
	1 (foreground), 1 (first image to be imaged)
	03 00 00 00 01 00 00 00

	IFD 0 ( GP IFD (  Child IFD 0 Offset 1334 (0x536H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	000536
	Number of Directory Entries
	–
	–
	–
	5 entries
	05 00

	000538
	ProfileType
	401
	Long
	1
	1 (Group 3 fax)
	91 01 04 00 01 00 00 00 01 00 00 00 

	000544
	FaxProfile
	402
	Byte
	6
	Mixed Raster Content, Profile M
	92 01 01 00 01 00 00 00 06 00 00 00

	000550
	CodingMethod
	403
	Long
	2
	40, T.6 (MMR) and T.81 (JPEG)
	93 01 04 00 01 00 00 00 28 00 00 00

	00055C
	VersionYear
	404
	Byte
	n/a
	1999
	94 01 01 00 04 00 00 00 31 39 39 39

	000568
	ModeNumber
	405
	Byte
	1
	1
	95 01 01 00 01 00 00 00 00 00 00 00 

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	Image data

	Offset
	Description
	Comments

	000578H - 0095c7H
	IFD 0 (Mask) Strip 0
	Compressed data

	0095c8H - 013d19H
	IFD 0 (Bkgnd) Strip 0
	Compressed data

	013d1cH - 0300a3H
	IFD 0 (Frgnd) Strip 0
	Compressed data

	IFD1  (Mask)  Offset 196772 (0x300a4H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0300A4
	Number of Directory Entries
	–
	–
	–
	23 entries
	17 00

	0300A6
	NewSubfileType
	254
	Long
	18
	primary IFD of a single page of a multi-page 
	FE 00 04 00 01 00 00 00 12 00 00 00

	0300B2
	ImageWidth
	256
	Long
	3456
	A4
	00 01 04 00 01 00 00 00 80 0D 00 00

	0300BE
	ImageLength
	257
	Long
	n/a
	4400 lines
	01 01 04 00 01 00 00 00 30 11 00 00 

	0300CA
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	02 01 03 00 01 00 00 00 01 00 00 00

	0300D6
	Compression
	259
	Short
	4
	4, T.6 MMR
	03 01 03 00 01 00 00 00 04 00 00 00

	0300E2
	Photometric Interpretation
	262
	Short
	0
	0 is white
	06 01 03 00 01 00 00 00 00 00 00 00

	0300EE
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	0300FA
	DocumentName
	269
	ASCII
	n/a
	Test case M02
	0D 01 02 00 0E 00 00 00 BE 01 03 00

	030106
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 CE 01 03 00

	030112
	StripOffsets
	273
	Long
	n/a
	198086 bytes
	11 01 04 00 01 00 00 00 C6 05 03 00

	03011E
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	15 01 03 00 01 00 00 00 01 00 00 00

	03012A
	RowsPerStrip
	278
	Long
	(ImageLength
	4400 rows
	16 01 04 00 01 00 00 00 30 11 00 00

	030136
	StripByteCounts
	279
	Long
	n/a
	1678 bytes
	17 01 04 00 01 00 00 00 8E 06 00 00

	030142
	Xresolution
	282
	Rational
	200
	400 pixels/inch
	1A 01 05 00 01 00 00 00 EE 01 03 00

	03014E
	Yresolution
	283
	Rational
	200
	400 pixels/inch
	1B 01 05 00 01 00 00 00 F6 01 03 00

	03015A
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 FE 01 03 00

	030166
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 06 02 03 00

	030172
	T6Option
	293
	Long
	0
	MMR (G4) compressed
	25 01 04 00 01 00 00 00 00 00 00 00

	03017E
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	03018A
	PageNumber
	297
	Short
	n/a
	2 of 2
	29 01 03 00 02 00 00 00 01 00 02 00

	030196
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 0E 02 03 00

	0301A2
	SubIFDs
	330
	Long
	n/a
	Offsets of 197218 and 197652
	4A 01 04 00 02 00 00 00 52 02 03 00

	0301AE
	ImageLayer
	34732
	Long
	2, 1
	2 (mask), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 5A 02 03 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1 (Mask)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	0301BE
	DocumentName
	Test case M02
	54 65 73 74 20 43 61 73 65 20 4D 30 32  00

	0301CE


0301DE
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	0301EE
	Xresolution 
	400 pixels/inch
	90 01 00 00 01 00 00 00

	0301F6
	Yresolution 
	400 pixels/inch
	90 01 00 00 01 00 00 00

	0301FE
	XPosition
	0 offset
	00 00 00 00 90 01 00 00 

	030206
	YPosition
	0 offset
	00 00 00 00 90 01 00 00

	03020E
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

	03021E
	
	
	66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

	
03022E
	
	
	
74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

	
03023E
	

	

	
32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

	03024E
	
	
	00

	030252
	SubIFDs
	Offsets of 197202 and 197632
	62 02 03 00 14 04 03 00

	03025A
	ImageLayer
	2 (mask), 1 (first image to be imaged)
	02 00 00 00 01 00 00 00

	IFD 1 ( SubIFD 0 ( Child IFD 0 (Bkgnd)  Offset 197218 (0x30262H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	030262
	Number of Directory Entries
	–
	–
	–
	22 entries
	16 00

	030264
	NewSubfileType
	254
	Long
	16
	Child IFD (background) of a single page of a multi-page
	FE 00 04 00 01 00 00 00 10 00 00 00

	030270
	ImageWidth
	256
	Long
	864
	A4
	00 01 04 00 01 00 00 00 60 03 00 00

	03027C
	ImageLength
	257
	Long
	n/a
	1100 lines
	01 01 04 00 01 00 00 00 4C 04 00 00 

	030288
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 70 03 03 00

	030294
	Compression
	259
	Short
	7
	JPEG
	03 01 03 00 01 00 00 00 07 00 00 00

	0302A0
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	0302AC
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	0302B8
	DocumentName
	269
	ASCII
	n/a
	Test case M02
	0D 01 02 00 0E 00 00 00 78 03 03 00

	0302C4
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 88 03 03 00

	0302D0
	StripOffsets
	273
	Long
	n/a
	199766 bytes
	11 01 04 00 01 00 00 00 56 0C 03 00

	0302DC
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	0302E8
	RowsPerStrip
	278
	Long
	(ImageLength
	1100 rows
	16 01 04 00 01 00 00 00 4C 04 00 00

	0302F4
	StripByteCounts
	279
	Long
	n/a
	113332 bytes
	17 01 04 00 01 00 00 00 B4 BA 01 00

	030300
	Xresolution
	282
	Rational
	200
	100 pixels/inch
	1A 01 05 00 01 00 00 00 A8 03 03 00

	03030C
	Yresolution
	283
	Rational
	200
	100 pixels/inch
	1B 01 05 00 01 00 00 00 B0 03 03 00

	030318
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 B8 03 03 00

	030324
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 C0 03 03 00

	030330
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	03033C
	PageNumber
	297
	Short
	n/a
	2 of 2
	29 01 03 00 02 00 00 00 01 00 02 00

	030348
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 C8 03 03 00

	030354
	ChromaSubSampling
	530
	Short
	4:1:1
	1:1:1
	12 02 03 00 02 00 00 00 01 00 01 00

	030360
	ImageLayer
	34732
	Long
	1, 1
	1 (background), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 0C 04 03 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1 ( SubIFD 0 ( Child IFD 0 (Bkgnd)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	030370
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	030378
	DocumentName
	Test Case M02
	54 65 73 74 20 43 61 73 65 20 4D 30 32 00

	030388


030398
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	0303A8
	Xresolution 
	100 pixels/inch
	64 00 00 00 01 00 00 00

	0303B0
	Yresolution 
	 100 pixels/inch
	64 00 00 00 01 00 00 00

	0303B8
	XPosition
	0 offset
	00 00 00 00 90 01 00 00 

	0303C0
	YPosition
	0 offset
	00 00 00 00 90 01 00 00

	0303C8


0303D8


0303E8


0303F8


030408
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	03040C
	ImageLayer
	1 (background), 1 (first image to be imaged)
	01 00 00 00 01 00 00 00

	IFD 1 ( SubIFD 1 ( Child IFD 0 (Frgnd)  Offset 197652 (0x30414H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	030414
	Number of Directory Entries
	–
	–
	–
	22 entries
	16 00

	030416
	NewSubfileType
	254
	Long
	16
	Child IFD (foreground) of a single page of a multi-page
	FE 00 04 00 01 00 00 00 10 00 00 00

	030422
	ImageWidth
	256
	Long
	1728
	A4
	00 01 04 00 01 00 00 00 C0 06 00 00

	03042E
	ImageLength
	257
	Long
	n/a
	2200 lines
	01 01 04 00 01 00 00 00 98 08 00 00 

	03043A
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 22 05 03 00

	030446
	Compression
	259
	Short
	7
	JPEG
	03 01 03 00 01 00 00 00 07 00 00 00

	030452
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	03045E
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	03046A
	DocumentName
	269
	ASCII
	n/a
	Test case M02
	0D 01 02 00 0E 00 00 00 2A 05 03 00

	030476
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 3A 05 03 00

	030482
	StripOffsets
	273
	Long
	n/a
	313098 bytes
	11 01 04 00 01 00 00 00 0A C7 04 00

	03048E
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	03049A
	RowsPerStrip
	278
	Long
	(ImageLength
	2200 rows
	16 01 04 00 01 00 00 00 98 08 00 00

	0304A6
	StripByteCounts
	279
	Long
	n/a
	61782 bytes
	17 01 04 00 01 00 00 00 56 F1 00 00

	0304B2
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 5A 05 03 00

	0304BE
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 62 05 03 00

	0304CA
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 6A 05 03 00

	0304D6
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 72 05 03 00

	0304E2
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	0304EE
	PageNumber
	297
	Short
	n/a
	2 of 2
	29 01 03 00 02 00 00 00 01 00 02 00

	0304FA
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 7A 05 03 00

	030506
	ChromaSubSampling
	530
	Short
	4:1:1
	4:1:1
	12 02 03 00 02 00 00 00 02 00 02 00

	030512
	ImageLayer
	34732
	Long
	3, 1
	3 (foreground), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 BE 05 03 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1 ( SubIFD 1 ( Child IFD 0 (Frgnd)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	030522
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	03052A
	DocumentName
	Test Case M02
	54 65 73 74 20 43 61 73 65 20 4D 30 32 00

	03053A


03054A
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	03055A
	Xresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	030562
	Yresolution 
	 200 pixels/inch
	C8 00 00 00 01 00 00 00

	03056A
	XPosition
	0 offset
	00 00 00 00 90 01 00 00 

	030572
	YPosition
	0 offset
	00 00 00 00 90 01 00 00

	03057A


03058A


03059A


0305AA


0305BA
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	0305BE
	ImageLayer
	3 (foreground), 1 (first image to be imaged)
	03 00 00 00 01 00 00 00

	Image data

	Offset
	Description
	Comments

	000305c6H - 00030c54H
	IFD 1 (Mask) Strip 0
	Compressed data

	00030c56H - 0004c70aH
	IFD 1 (Bkgnd) Strip 0
	Compressed data

	0004c70aH - 0005b860H
	IFD 1 (Frgnd) Strip 0
	Compressed data


IV.5.2
Three layers & multi-strip per layer: Mask with T.85 JBIG compression, Background with JPEG compression and Foreground with JBIG compression  and T.43 color encoded ITU L*a*b*

3-layer (BG/M/FG), 200/400/200 resolution with FG having both 400 and 200 resolution, JPEG/JBIG/JBIG compression, multi-strip image with little-endian byte order and StripRowCounts used, BG & FG has strip with X and YPosition >0, ChromaSubsampling BG=4:1:1, FG LayerBaseColor changes between strip, LSB FillOrder, based on test case Profile M M11

Table IV.5-2/T.37 – Coding example of TIFF header, IFD and image data

	Header
	
	
	
	
	File Length 300028 bytes

	Offset
	Description
	
	
	
	Comments
	Coding

	0000
	Octet order
	
	
	
	Little endian order
	49 49

	0002
	File identifier
	
	
	
	42, TIFF file
	2A 00

	0004
	1st IFD offset
	
	
	
	Offset of IFD 0 
	08 00 00 00

	IFD0 (Mask)  Offset 8 (0x8H) NextIFDOffset 88046 (0x4157eeH)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0008
	Number of Directory Entries
	–
	–
	–
	23 entries
	17 00

	000A
	NewSubfileType
	254
	Long
	18
	primary IFD of a single page of a multi-page 
	FE 00 04 00 01 00 00 00 12 00 00 00

	0016
	ImageWidth
	256
	Long
	3456
	A4
	00 01 04 00 01 00 00 00 80 0D 00 00

	0022
	ImageLength
	257
	Long
	n/a
	4400 lines
	01 01 04 00 01 00 00 00 30 11 00 00 

	002E
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	02 01 03 00 01 00 00 00 01 00 00 00

	003A
	Compression
	259
	Short
	4
	9, JBIG T.85
	03 01 03 00 01 00 00 00 09 00 00 00

	0046
	Photometric Interpretation
	262
	Short
	0
	0 is white
	06 01 03 00 01 00 00 00 00 00 00 00

	0052
	FillOrder
	266
	Short
	2
	LSB first
	0A 01 03 00 01 00 00 00 02 00 00 00

	00 5E
	DocumentName
	269
	ASCII
	n/a
	Test case M11
	0D 01 02 00 0E 00 00 00 22 01 00 00 

	00 6A
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 32 01 00 00

	0076
	StripOffsets
	273
	Long
	n/a
	1726 38498 68626 77078 88022  bytes
	11 01 04 00 05 00 00 00 52 01 00 00

	0082
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	15 01 03 00 01 00 00 00 01 00 00 00

	008E
	StripByteCounts
	279
	Long
	n/a
	892 5588 8451 10944 22  bytes
	17 01 04 00 05 00 00 00 66 01 00 00

	009A
	Xresolution
	282
	Rational
	200
	400 pixels/inch
	1A 01 05 00 01 00 00 00 7A 01 00 00 

	00A6
	Yresolution
	283
	Rational
	200
	400 pixels/inch
	1B 01 05 00 01 00 00 00 82 01 00 00

	00B2
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 8A 01 00 00

	00BE
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 92 01 00 00


	00CA
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	00D6
	PageNumber
	297
	Short
	n/a
	1 of 2
	29 01 03 00 02 00 00 00 00 00 02 00

	00E2
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 9A 01 00 00

	00EE
	SubIFDs
	330
	Long
	n/a
	Offsets of 514 and 948
	4A 01 04 00 02 00 00 00 DE 01 00 00


	00FA
	GlobalParametersIFD
	400
	Long
	n/a
	1660 offset
	90 01 04 00 01 00 00 00 7C 06 00 00

	0106
	StripRowCounts
	559
	Long
	n/a
	486 rows
	2F 02 04 00 05 00 00 00 E6 01 00 00

	0112
	ImageLayer
	34732
	Long
	2, 1
	2 (mask), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 FA 01 00 00

	
	
	
	
	
	
	Next IFD offset EE 57 01 00

	IFD 0 (Mask)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	000122
	DocumentName
	Test case M11
	54 65 73 74 20 43 61 73 65 20 4D 31 31 00

	000132


000142
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	000152


000162
	StripOffsets
	1726 38498 68626 77078 88022  bytes
	BE 06 00 00 62 96 00 00 12 0C 01 00 16 2D 01 00

D6 57 01 00

	000166


000176
	StripByteCounts
	892 5588 8451 10944 22  bytes
	7C 03 00 00 D4 15 00 00 03 21 00 00 C0 2A 00 00

16 00 00 00

	00017A
	Xresolution 
	400 pixels/inch
	90 01 00 00 01 00 00 00

	000182
	Yresolution 
	400 pixels/inch
	90 01 00 00 01 00 00 00

	00018A
	XPosition
	0 offset
	00 00 00 00 90 01 00 00 

	000192
	YPosition
	0 offset
	00 00 00 00 90 01 00 00

	00019A


0001AA


0001BA


0001CA


0001DA
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	0001DE
	SubIFDs
	Offsets of 514 and 948
	02 02 00 00 B4 03 00 00

	0001E6


0001F6
	StripRowCounts
	486 rows
	15 02 00 00 3B 01 00 00 A0 03 00 00 00 03 00 00

40 07 00 00

	0001FA
	ImageLayer
	2 (mask), 1 (first image to be imaged)
	02 00 00 00 01 00 00 00

	IFD 0 ( SubIFD 0 ( Child IFD 0 (Bkgnd)  Offset 514 (0x202H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	000202
	Number of Directory Entries
	–
	–
	–
	22 entries
	16 00

	000204
	NewSubfileType
	254
	Long
	16
	Child IFD (background) of a single page of a multi-page
	FE 00 04 00 01 00 00 00 10 00 00 00

	000210
	ImageWidth
	256
	Long
	n/a
	900 pels
	00 01 04 00 01 00 00 00 84 03 00 00

	00021C
	ImageLength
	257
	Long
	n/a
	600 lines
	01 01 04 00 01 00 00 00 58 02 00 00

	000228
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 10 03 00 00

	000234
	Compression
	259
	Short
	7
	JPEG
	03 01 03 00 01 00 00 00 07 00 00 00

	000240
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	00024C
	FillOrder
	266
	Short
	2
	LSB first
	0A 01 03 00 01 00 00 00 02 00 00 00

	000258
	DocumentName
	269
	ASCII
	n/a
	Test case M11
	0D 01 02 00 0E 00 00 00 18 03 00 00

	000264
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 28 03 00 00

	000270
	StripOffsets
	273
	Long
	n/a
	2618 bytes
	11 01 04 00 01 00 00 00 3A 0A 00 00

	00027C
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	000288
	RowsPerStrip
	278
	Long
	(ImageLength
	600 rows
	16 01 04 00 01 00 00 00 58 02 00 00

	000294
	StripByteCounts
	279
	Long
	n/a
	35878 bytes
	17 01 04 00 01 00 00 00 26 8C 00 00

	0002A0
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 48 03 00 00

	0002AC
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 50 03 00 00

	0002B8
	XPosition
	286
	Rational
	n/a
	1.935 X-offset
	1E 01 05 00 01 00 00 00 58 03 00 00

	0002C4
	YPosition
	287
	Rational
	n/a
	6.360 Y-offset
	1F 01 05 00 01 00 00 00 60 03 00 00

	0002D0
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	0002DC
	PageNumber
	297
	Short
	n/a
	1 of 2
	29 01 03 00 02 00 00 00 00 00 02 00

	0002E8
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 68 03 00 00

	0002F4
	ChromaSubSampling
	530
	Short
	4:1:1
	4:1:1
	12 02 03 00 02 00 00 00 02 00 02 00

	000300
	ImageLayer
	34732
	Long
	1, 1
	1 (background), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 AC 03 00 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 0 ( SubIFD 0 ( Child IFD 0 (Bkgnd)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	000310
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	000318
	DocumentName
	Test Case M11
	54 65 73 74 20 43 61 73 65 20 4D 31 31 00

	000328


000338
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	000348
	Xresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	000350
	Yresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	000358
	XPosition
	1.935 offset
	06 03 00 00 90 01 00 00

	000360
	YPosition
	6.360 offset
	F0 09 00 00 90 01 00 00

	000368


000378


000388


000398


0003A8
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	0003AC
	ImageLayer
	1 (background), 1 (first image to be imaged)
	01 00 00 00 01 00 00 00

	IFD 0 ( SubIFD 1 ( Child IFD 0 (Frgnd)  Offset 948 (0x3b4H) NextIFDOffset1238 (0x4d6H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0003B4
	Number of Directory Entries
	–
	–
	–
	19 entries
	13 00

	0003B6
	NewSubfileType
	254
	Long
	16
	Child IFD (foreground) of a single page of a multi-page
	FE 00 04 00 01 00 00 00 10 00 00 00

	0003C2
	ImageWidth
	256
	Long
	3456
	3456 pels, A4
	00 01 04 00 01 00 00 00 80 0D 00 00

	0003CE
	ImageLength
	257
	Long
	n/a
	533 lines
	01 01 04 00 01 00 00 00 15 02 00 00

	0003DA
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 9E 04 00 00

	0003E6
	Compression
	259
	Short
	n/a
	No compression
	03 01 03 00 01 00 00 00 01 00 00 00

	0003F2
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	0003FE
	FillOrder
	266
	Short
	2
	LSB first
	0A 01 03 00 01 00 00 00 02 00 00 00

	00040A
	StripOffsets
	273
	Long
	n/a
	38498 bytes
	11 01 04 00 01 00 00 00 62 96 00 00

	000416
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	000422
	RowsPerStrip
	278
	Long
	(ImageLength
	533 rows
	16 01 04 00 01 00 00 00 15 02 00 00

	00042E
	StripByteCounts
	279
	Long
	n/a
	0 bytes
	17 01 04 00 01 00 00 00 00 00 00 00

	00043A
	Xresolution
	282
	Rational
	200
	400 pixels/inch
	1A 01 05 00 01 00 00 00 A6 04 00 00

	000446
	Yresolution
	283
	Rational
	200
	400 pixels/inch
	1B 01 05 00 01 00 00 00 AE 04 00 00

	000452
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 B6 04 00 00

	00045E
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 BE 04 00 00

	00046A
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	000476
	PageNumber
	297
	Short
	n/a
	1 of 2
	29 01 03 00 02 00 00 00 00 00 02 00

	000482
	DefaultImageColor
	434
	Short
	black
	1222 offset
	B2 01 03 00 03 00 00 00 C6 04 00 00

	00048E
	ImageLayer
	34732
	Long
	3, 1
	3 (foreground), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 CE 04 00 00

	
	
	
	
	
	
	Next IFD offset D6 04 00 00

	IFD 0 ( SubIFD 1 ( Child IFD 0 (Frgnd)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	00049E
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	0004A6
	Xresolution 
	400 pixels/inch
	90 01 00 00 01 00 00 00

	000AE
	Yresolution 
	400 pixels/inch
	90 01 00 00 01 00 00 00

	0004B6
	XPosition
	0 offset
	00 00 00 00 90 01 00 00

	0004BE
	YPosition
	0 offset
	00 00 00 00 90 01 00 00

	0004C6
	DefaultImageColor
	L*=57.25, a*=76.67, b*=65.88
	92 00 F3 00 BE 00

	0004CE
	ImageLayer
	3 (foreground), 1 (first image to be imaged)
	03 00 00 00 01 00 00 00

	IFD 0 ( SubIFD 1 ( Child IFD 1 (Frgnd)  Offset 1238 (0x4d6H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0004D6
	Number of Directory Entries
	–
	–
	–
	21 entries
	15 00

	0004D8
	NewSubfileType
	254
	Long
	16
	Child IFD (foreground) of a single page of a multi-page
	FE 00 04 00 01 00 00 00 10 00 00 00

	0004E4
	ImageWidth
	256
	Long
	
	736
	00 01 04 00 01 00 00 00 E0 02 00 00

	0004F0
	ImageLength
	257
	Long
	n/a
	464 lines
	01 01 04 00 01 00 00 00 D0 01 00 00

	0004FC
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 D8 05 00 00

	000508
	Compression
	259
	Short
	n/a
	JBIG color, T.43
	03 01 03 00 01 00 00 00 0A 00 00 00

	000514
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	000520
	FillOrder
	266
	Short
	2
	LSB first
	0A 01 03 00 01 00 00 00 02 00 00 00

	00052C
	DocumentName
	269
	ASCII
	n/a
	Test case M11
	0D 01 02 00 0E 00 00 00 E0 05 00 00

	000538
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 F0 05 00 00

	000544
	StripOffsets
	273
	Long
	n/a
	44086 bytes
	11 01 04 00 01 00 00 00 36 AC 00 00

	000550
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	00055C
	RowsPerStrip
	278
	Long
	(ImageLength
	464 rows
	16 01 04 00 01 00 00 00 D0 01 00 00

	000568
	StripByteCounts
	279
	Long
	n/a
	24539 bytes
	17 01 04 00 01 00 00 00 DB 5F 00 00

	000574
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 10 06 00 00

	000580
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 18 06 00 00

	00058C
	XPosition
	286
	Rational
	n/a
	3.600 X-offset
	1E 01 05 00 01 00 00 00 20 06 00 00

	000598
	YPosition
	287
	Rational
	n/a
	2.120 Y-offset
	1F 01 05 00 01 00 00 00 28 06 00 00

	0005A4
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	0005B0
	PageNumber
	297
	Short
	n/a
	1 of 2
	29 01 03 00 02 00 00 00 00 00 02 00

	0005BC
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 30 06 00 00

	0005C8
	ImageLayer
	34732
	Long
	3, 2
	3 (foreground), 2 (second image to be imaged)
	AC 87 04 00 02 00 00 00 74 06 00 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 0 ( SubIFD 1 ( Child IFD 1 (Frgnd)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	0005D8
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	0005E0
	DocumentName
	Test Case M11
	54 65 73 74 20 43 61 73 65 20 4D 31 31 00

	0005F0


000600
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	000610
	Xresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	000618
	Yresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	000620
	XPosition
	3.600 offset
	A0 05 00 00 90 01 00 00

	000628
	YPosition
	2.120 offset
	50 03 00 00 90 01 00 00

	000630


000640


000650


000660


000670
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	000674
	ImageLayer
	3 (foreground), 2 (second image to be imaged)
	03 00 00 00 02 00 00 00

	IFD 0 ( GP IFD (  Child IFD 0 Offset 1660 (0x67cH) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	00067C
	Number of Directory Entries
	–
	–
	–
	5 entries
	05 00

	00067E
	ProfileType
	401
	Long
	1
	1 (Group 3 fax)
	91 01 04 00 01 00 00 00 01 00 00 00 

	00068A
	FaxProfile
	402
	Byte
	6
	Mixed Raster Content, Profile M
	92 01 01 00 01 00 00 00 06 00 00 00

	000696
	CodingMethod
	403
	Long
	2
	112, JBIG (T.85), JPEG (T.81) and JBIG color (T.43)
	93 01 04 00 01 00 00 00 70 00 00 00

	0006A2
	VersionYear
	404
	Byte
	n/a
	1999
	94 01 01 00 04 00 00 00 31 39 39 39

	0006AE
	ModeNumber
	405
	Byte
	1
	1
	95 01 01 00 01 00 00 00 00 00 00 00 

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	Image data

	Offset
	Description
	Comments

	000006beH - 00000a3aH
	IFD 0 (Mask) Strip 0
	Compressed data

	00000a3aH - 00009660H
	IFD 0 SubIFD 0 Child IFD 0 (Bkgnd) Strip 0
	Compressed data

	00009662H - 00009662H
	IFD 0 SubIFD 1 Child IFD 0 (Frgnd) Strip 0
	Compressed data

	00009662H - 0000ac36H
	IFD 0 (Mask) Strip 1
	Compressed data

	0000ac36H - 00010c11H
	IFD 0 SubIFD 1 Child IFD 1 (Frgnd) Strip 0
	Compressed data

	00010c12H - 00012d15H
	IFD 0 (Mask) Strip 2
	Compressed data

	00012d16H - 000157d6H
	IFD 0 (Mask) Strip 3
	Compressed data

	000157d6H - 000157ecH
	IFD 0 (Mask) Strip 4
	Compressed data

	IFD1 (Mask)  Offset 88046 (0x157eeH) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0157EE
	Number of Directory Entries
	–
	–
	–
	22 entries
	16 00

	0157F0
	NewSubfileType
	254
	Long
	18
	primary IFD of a single page of a multi-page 
	FE 00 04 00 01 00 00 00 12 00 00 00

	0157FC
	ImageWidth
	256
	Long
	3456
	A4
	00 01 04 00 01 00 00 00 80 0D 00 00

	015808
	ImageLength
	257
	Long
	n/a
	4400 lines
	01 01 04 00 01 00 00 00 30 11 00 00 

	015814
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	02 01 03 00 01 00 00 00 01 00 00 00

	015820
	Compression
	259
	Short
	4
	9, JBIG (T.85)
	03 01 03 00 01 00 00 00 09 00 00 00

	01582C
	Photometric Interpretation
	262
	Short
	0
	0 is white
	06 01 03 00 01 00 00 00 00 00 00 00

	015838
	FillOrder
	266
	Short
	2
	LSB first
	0A 01 03 00 01 00 00 00 02 00 00 00

	015844
	DocumentName
	269
	ASCII
	n/a
	Test case M11
	0D 01 02 00 0E 00 00 00 FC 58 01 00

	015850
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 0C 59 01 00

	01585C
	StripOffsets
	273
	Long
	n/a
	89204 bytes
	11 01 04 00 01 00 00 00 74 5C 01 00

	015868
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	15 01 03 00 01 00 00 00 01 00 00 00

	015874
	RowsPerStrip
	278
	Long
	(ImageLength
	4400 rows
	16 01 04 00 01 00 00 00 30 11 00 00

	015880
	StripByteCounts
	279
	Long
	n/a
	917 bytes
	17 01 04 00 01 00 00 00 95 03 00 00

	01588C
	Xresolution
	282
	Rational
	200
	400 pixels/inch
	1A 01 05 00 01 00 00 00 2C 59 01 00

	015898
	Yresolution
	283
	Rational
	200
	400 pixels/inch
	1B 01 05 00 01 00 00 00 34 59 01 00

	0158A4
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 3C 59 01 00

	0158B0
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 44 59 01 00

	0158BC
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	0158C8
	PageNumber
	297
	Short
	n/a
	2 of 2
	29 01 03 00 02 00 00 00 01 00 02 00

	0158D4
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 4C 59 01 00

	0158E0
	SubIFDs
	330
	Long
	n/a
	Offsets of 88480 and 88914
	4A 01 04 00 02 00 00 00 90 59 01 00

	0158EC
	ImageLayer
	34732
	Long
	2, 1
	2 (mask),1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 98 59 01 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1 (Mask)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	0158FC
	DocumentName
	Test case M11
	54 65 73 74 20 43 61 73 65 20 4D 31 31  00

	01590C


01591C
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	01592C
	Xresolution 
	400 pixels/inch
	90 01 00 00 01 00 00 00

	015934
	Yresolution 
	400 pixels/inch
	90 01 00 00 01 00 00 00

	01593C
	XPosition
	0 offset
	00 00 00 00 90 01 00 00 

	015944
	YPosition
	0 offset
	00 00 00 00 90 01 00 00

	01594C


01595C


01596C


01597C


01598C
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	015990
	SubIFDs
	Offsets of 88480 and 88914
	A0 59 01 00 52 5B 01 00

	015998
	ImageLayer
	2 (mask), 1 (first image to be imaged)
	02 00 00 00 01 00 00 00

	IFD 1 ( SubIFD 0 ( Child IFD 0 (Bkgnd)  Offset 88480 (0x159a0H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0159A0
	Number of Directory Entries
	–
	–
	–
	22 entries
	16 00

	0159A2
	NewSubfileType
	254
	Long
	16
	Child IFD (background) of a single page of a multi-page
	FE 00 04 00 01 00 00 00 10 00 00 00

	0159AE
	ImageWidth
	256
	Long
	
	1198 pels
	00 01 04 00 01 00 00 00 AE 04 00 00

	0159BA
	ImageLength
	257
	Long
	n/a
	1352 lines
	01 01 04 00 01 00 00 00 48 05 00 00

	0159C6
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 AE 5A 01 00

	0159D2
	Compression
	259
	Short
	7
	JPEG
	03 01 03 00 01 00 00 00 07 00 00 00

	0159DE
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	0159EA
	FillOrder
	266
	Short
	2
	LSB first
	0A 01 03 00 01 00 00 00 02 00 00 00

	0159F6
	DocumentName
	269
	ASCII
	n/a
	Test case M11
	0D 01 02 00 0E 00 00 00 B6 5A 01 00

	015A02
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 C6 5A 01 00

	015A0E
	StripOffsets
	273
	Long
	n/a
	90124 bytes
	11 01 04 00 01 00 00 00 0C 60 01 00

	015A1A
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	015A26
	RowsPerStrip
	278
	Long
	(ImageLength
	1352 rows
	16 01 04 00 01 00 00 00 48 05 00 00

	015A32
	StripByteCounts
	279
	Long
	n/a
	209902 bytes
	17 01 04 00 01 00 00 00 EE 33 03 00

	015A3E
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 E6 5A 01 00

	015A4A
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 EE 5A 01 00

	015A56
	XPosition
	286
	Rational
	n/a
	1.185 X-offset
	1E 01 05 00 01 00 00 00 F6 5A 01 00

	015A62
	YPosition
	287
	Rational
	n/a
	1.345 Y-offset
	1F 01 05 00 01 00 00 00 FE 5A 01 00

	015A6E
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	015A7A
	PageNumber
	297
	Short
	n/a
	2 of 2
	29 01 03 00 02 00 00 00 01 00 02 00

	015A86
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 06 5B 01 00

	015A92
	ChromaSubSampling
	530
	Short
	4:1:1
	4:1:1
	12 02 03 00 02 00 00 00 02 00 02 00

	015A9E
	ImageLayer
	34732
	Long
	1, 1
	1 (background), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 4A 5B 01 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1 ( SubIFD 0 ( Child IFD 0 (Bkgnd)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	015AAE
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	015AB6
	DocumentName
	Test Case M11
	54 65 73 74 20 43 61 73 65 20 4D 31 31 00

	015AC6


015AD6
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	015AE6
	Xresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	015AEE
	Yresolution 
	 200 pixels/inch
	C8 00 00 00 01 00 00 00

	015AF6
	XPosition
	1.185 offset
	DA 01 00 00 90 01 00 00

	015AFE
	YPosition
	1.345 offset
	1A 02 00 00 90 01 00 00

	015B06


015B16


015B26


015B36


015B46
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	015B4A
	ImageLayer
	1 (background), 1 (first image to be imaged)
	01 00 00 00 01 00 00 00

	IFD 1 ( SubIFD 1 ( Child IFD 0 (Frgnd)  Offset 88914 (0x15b52H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	015B52
	Number of Directory Entries
	–
	–
	–
	19 entries
	13 00

	015B54
	NewSubfileType
	254
	Long
	16
	Child IFD (foreground) of a single page of a multi-page
	FE 00 04 00 01 00 00 00 10 00 00 00

	015B60
	ImageWidth
	256
	Long
	3456
	A4
	00 01 04 00 01 00 00 00 80 0D 00 00

	015B6C
	ImageLength
	257
	Long
	n/a
	4400 lines
	01 01 04 00 01 00 00 00 30 11 00 00

	015B78
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 3C 5C 01 00

	015B84
	Compression
	259
	Short
	
	uncompressed
	03 01 03 00 01 00 00 00 01 00 00 00

	015B90
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	015B9C
	FillOrder
	266
	Short
	2
	LSB first
	0A 01 03 00 01 00 00 00 02 00 00 00

	015BA8
	StripOffsets
	273
	Long
	n/a
	300028 bytes
	11 01 04 00 01 00 00 00 FC 93 04 00

	015BB4
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	015BC0
	RowsPerStrip
	278
	Long
	(ImageLength
	4400 rows
	16 01 04 00 01 00 00 00 30 11 00 00

	015BCC
	StripByteCounts
	279
	Long
	n/a
	0 bytes
	17 01 04 00 01 00 00 00 00 00 00 00

	015BD8
	Xresolution
	282
	Rational
	200
	400 pixels/inch
	1A 01 05 00 01 00 00 00 44 5C 01 00

	015BE4
	Yresolution
	283
	Rational
	200
	400 pixels/inch
	1B 01 05 00 01 00 00 00 4C 5C 01 00

	015BF0
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 54 5C 01 00

	015BFC
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 5C 5C 01 00

	015C08
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	015C14
	PageNumber
	297
	Short
	n/a
	2 of 2
	29 01 03 00 02 00 00 00 01 00 02 00

	015C20
	DefaultImageColor
	434
	Short
	Black
	89188 offset
	B2 01 03 00 03 00 00 00 64 5C 01 00

	015C2C
	ImageLayer
	34732
	Long
	3, 1
	3 (foreground), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 6C 5C 01 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1 ( SubIFD 1 ( Child IFD 0 (Frgnd)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	015C3C
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	015C44
	Xresolution 
	400 pixels/inch
	90 01 00 00 01 00 00 00

	015C4C
	Yresolution 
	400 pixels/inch
	90 01 00 00 01 00 00 00

	015C54
	XPosition
	0 offset
	00 00 00 00 90 01 00 00 

	015C5C
	YPosition
	0 offset
	00 00 00 00 90 01 00 00

	015C64
	DefaultImageColor
	L*=97.65, a*=-14.67, b*=87.06
	F9 00 6A 00 DA 00

	015C6C
	ImageLayer
	3 (foreground), 1 (first image to be imaged)
	03 00 00 00 01 00 00 00

	Image data

	Offset
	Description
	Comments

	00015c74H - 00016009H
	IFD 1 (Mask) Strip 0
	Compressed data

	0001600cH - 000493faH
	IFD 1 SubIFD 0 Child IFD 0 (Bkgnd) Strip 0
	Compressed data

	000493fcH - 000493fcH
	IFD 1 SubIFD 1 Child IFD 0 (Frgnd) Strip 0
	Compressed data


IV.5.3
Two layers & single strip per layer: Mask with MH (1-dimensional T.4) compression and Foreground with JPEG compression and T.4 Annex E ITU L*a*b* encoding

2-layer (M/FG), -/200/200 resolution, -/MH/JPEG compression, single strip image, ChromaSubsampling FG=4:1:1, FG LayerBaseColor other than black, FG has strip with X and YPosition >0, MSB FillOrder, little-endian byte order, based on test case Profile M M6

Table IV.5-3/T.37 – Coding example of TIFF header, IFD and image data

	Header
	
	
	
	
	File Length 262032 bytes

	Offset
	Description
	
	
	
	Comments
	Coding

	0000
	Octet order
	
	
	
	Little endian order
	49 49

	0002
	File identifier
	
	
	
	42, TIFF file
	2A 00

	0004
	1st IFD offset
	
	
	
	Offset of IFD 0 
	08 00 00 00

	IFD0 (Mask)  Offset 8 (0x8H) NextIFDOffset 83670 (0x146d6H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0008
	Number of Directory Entries
	–
	–
	–
	24 entries
	18 00

	000A
	NewSubfileType
	254
	Long
	18
	primary IFD of a single page of a multi-page 
	FE 00 04 00 01 00 00 00 12 00 00 00

	0016
	ImageWidth
	256
	Long
	1728
	A4
	00 01 04 00 01 00 00 00 C0 06 00 00

	0022
	ImageLength
	257
	Long
	n/a
	2200 lines
	01 01 04 00 01 00 00 00 98 08 00 00

	002E
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	02 01 03 00 01 00 00 00 01 00 00 00

	003A
	Compression
	259
	Short
	4
	T.4 MH or MR
	03 01 03 00 01 00 00 00 03 00 00 00

	0046
	Photometric Interpretation
	262
	Short
	0
	0 is white
	06 01 03 00 01 00 00 00 00 00 00 00

	0052
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	005E
	DocumentName
	269
	ASCII
	n/a
	Test case M06
	0D 01 02 00 0E 00 00 00 2E 01 00 00 

	006A
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 3E 01 00 00

	0076
	StripOffsets
	273
	Long
	n/a
	958 bytes
	11 01 04 00 01 00 00 00 BE 03 00 00

	0082
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	15 01 03 00 01 00 00 00 01 00 00 00

	008E
	RowsPerStrip
	278
	Long
	(ImageLength
	2200 rows
	16 01 04 00 01 00 00 00 98 08 00 00

	009A
	StripByteCounts
	279
	Long
	n/a
	38033 bytes
	17 01 04 00 01 00 00 00 91 94 00 00

	00A6
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 5E 01 00 00

	00B2
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 66 01 00 00

	00BE
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 6E 01 00 00

	00CA
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 76 01 00 00

	00D6
	T4Option
	292
	Long
	0
	1-dimensional T.4 MH, byte aligned
	24 01 04 00 01 00 00 00 04 00 00 00


	00E2
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	00EE
	PageNumber
	297
	Short
	n/a
	1 of 2
	29 01 03 00 02 00 00 00 00 00 02 00

	00FA
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 7E 01 00 00

	0106
	SubIFDs
	330
	Long
	n/a
	Offsets of 458
	4A 01 04 00 01 00 00 00 CA 01 00 00


	0112
	GlobalParametersIFD
	400
	Long
	n/a
	892 offset
	90 01 04 00 01 00 00 00 7C 03 00 00

	011E
	ImageLayer
	34732
	Long
	2, 1
	2 (mask), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 C2 01 00 00

	
	
	
	
	
	
	Next IFD offset D6 46 01 00

	IFD 0 (Mask)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	00012E
	DocumentName
	Test case M06
	54 65 73 74 20 43 61 73 65 20 4D 30 36 00

	00013E


00014E
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	00015E
	Xresolution 
	400 pixels/inch
	C8 00 00 00 01 00 00 00

	000166
	Yresolution 
	400 pixels/inch
	C8 00 00 00 01 00 00 00

	00016E
	XPosition
	0 offset
	00 00 00 00 C8 00 00 00

	000176
	YPosition
	0 offset
	00 00 00 00 C8 00 00 00

	00017E


00018E


00019E


0001AE


0001BE
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	0001C2
	ImageLayer
	2 (mask), 1 (first image to be imaged)
	02 00 00 00 01 00 00 00

	IFD 0 ( SubIFD 0( Child IFD 0 (Frgnd)  Offset 458 (0x1caH) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0001CA
	Number of Directory Entries
	–
	–
	–
	22 entries
	16 00

	0001CC
	NewSubfileType
	254
	Long
	16
	Child IFD (foreground) of a single page of a multi-page
	FE 00 04 00 01 00 00 00 10 00 00 00

	0001D8
	ImageWidth
	256
	Long
	1728
	736 pels
	00 01 04 00 01 00 00 00 E0 02 00 00

	0001E4
	ImageLength
	257
	Long
	n/a
	464 lines
	01 01 04 00 01 00 00 00 D0 01 00 00

	0001F0
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 D8 02 00 00

	0001FC
	Compression
	259
	Short
	7
	JPEG
	03 01 03 00 01 00 00 00 07 00 00 00

	000208
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	000214
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	000220
	DocumentName
	269
	ASCII
	n/a
	Test case M06
	0D 01 02 00 0E 00 00 00 E0 02 00 00

	00022C
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 F0 02 00 00

	000238
	StripOffsets
	273
	Long
	n/a
	38994 bytes
	11 01 04 00 01 00 00 00 52 98 00 00

	000244
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	000250
	RowsPerStrip
	278
	Long
	(ImageLength
	464 rows
	16 01 04 00 01 00 00 00 D0 01 00 00

	00025C
	StripByteCounts
	279
	Long
	n/a
	44673 bytes
	17 01 04 00 01 00 00 00 81 AE 00 00

	000268
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 10 03 00 00

	000274
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 18 03 00 00

	000280
	XPosition
	286
	Rational
	n/a
	3.600 X-offset
	1E 01 05 00 01 00 00 00 20 03 00 00

	00028C
	YPosition
	287
	Rational
	n/a
	2.140 Y-offset
	1F 01 05 00 01 00 00 00 28 03 00 00

	000298
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	0002A4
	PageNumber
	297
	Short
	n/a
	1 of 2
	29 01 03 00 02 00 00 00 00 00 02 00

	0002B0
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 30 03 00 00

	0002BC
	ChromaSubSampling
	530
	Short
	4:1:1
	4:1:1
	12 02 03 00 02 00 00 00 02 00 02 00

	0002C8
	ImageLayer
	34732
	Long
	3, 1
	3 (foreground), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 74 03 00 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 0 ( SubIFD 0 ( Child IFD 0 (Frgnd)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	0002D8
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	0002E0
	DocumentName
	Test Case M06
	54 65 73 74 20 43 61 73 65 20 4D 30 36 00

	0002F0


000300
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	000310
	Xresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	000318
	Yresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	000320
	XPosition
	3.600 offset
	D0 02 00 00 C8 00 00 00

	000328
	YPosition
	2.140 offset
	AC 01 00 00 C8 00 00 00

	000330


000340


000350


000360


000370
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	000374
	ImageLayer
	3 (foreground), 1 (first image to be imaged)
	03 00 00 00 01 00 00 00

	IFD 0 ( GP IFD (  Child IFD 0 Offset 892 (0x37cH) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	00037C
	Number of Directory Entries
	–
	–
	–
	5 entries
	05 00

	00037E
	ProfileType
	401
	Long
	1
	1 (Group 3 fax)
	91 01 04 00 01 00 00 00 01 00 00 00 

	00038A
	FaxProfile
	402
	Byte
	6
	Mixed Raster Content, Profile M
	92 01 01 00 01 00 00 00 06 00 00 00

	000396
	CodingMethod
	403
	Long
	2
	34, T.4 (MH) and T.81 (JPEG)
	93 01 04 00 01 00 00 00 22 00 00 00

	0003A2
	VersionYear
	404
	Byte
	n/a
	1999
	94 01 01 00 04 00 00 00 31 39 39 39

	0003AE
	ModeNumber
	405
	Byte
	1
	1
	95 01 01 00 01 00 00 00 00 00 00 00 

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	Image data

	Offset
	Description
	Comments

	0003beH - 00984fH
	IFD 0 (Mask) Strip 0
	Compressed data

	009852H - 0146d3H
	IFD 0 (Frgnd) Strip 0
	Compressed data

	IFD1 (Mask)  Offset 83670 (0x146d6H) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0146D6
	Number of Directory Entries
	–
	–
	–
	23 entries
	17 00

	0146D8
	NewSubfileType
	254
	Long
	18
	primary IFD of a single page of a multi-page 
	FE 00 04 00 01 00 00 00 12 00 00 00

	0146E4
	ImageWidth
	256
	Long
	1728
	A4
	00 01 04 00 01 00 00 00 C0 06 00 00

	0146F0
	ImageLength
	257
	Long
	n/a
	2200 lines
	01 01 04 00 01 00 00 00 98 08 00 00

	0146FC
	BitsPerSample
	258
	Short
	1
	1 bit/sample
	02 01 03 00 01 00 00 00 01 00 00 00

	014708
	Compression
	259
	Short
	4
	T.4 MH or MR
	03 01 03 00 01 00 00 00 03 00 00 00

	014714
	Photometric Interpretation
	262
	Short
	0
	0 is white
	06 01 03 00 01 00 00 00 00 00 00 00

	014720
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	01472C
	DocumentName
	269
	ASCII
	n/a
	Test case M06
	0D 01 02 00 0E 00 00 00 F0 47 01 00

	01473B
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 00 48 01 00

	014744
	StripOffsets
	273
	Long
	n/a
	84832 bytes
	11 01 04 00 01 00 00 00 60 4B 01 00

	014750
	SamplesPerPixel
	277
	Short
	1
	1 sample/pixel
	15 01 03 00 01 00 00 00 01 00 00 00

	01475C
	RowsPerStrip
	278
	Long
	(ImageLength
	2200 rows
	16 01 04 00 01 00 00 00 98 08 00 00

	014768
	StripByteCounts
	279
	Long
	n/a
	13209 bytes
	17 01 04 00 01 00 00 00 99 33 00 00

	014774
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 20 48 01 00

	014780
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 28 48 01 00

	01478C
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 30 48 01 00

	014798
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 38 48 01 00

	0147A4
	T4Option
	292
	Long
	0
	1-dimensional T.4 MH, byte aligned
	24 01 04 00 01 00 00 00 04 00 00 00

	0147B0
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	0147BC
	PageNumber
	297
	Short
	n/a
	2 of 2
	29 01 03 00 02 00 00 00 01 00 02 00

	0147C8
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 40 48 01 00

	0147D4
	SubIFDs
	330
	Long
	n/a
	Offsets of 84108
	4A 01 04 00 01 00 00 00 8C 48 01 00

	0147E0
	ImageLayer
	34732
	Long
	2, 1
	2 (mask), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 84 48 01 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1 (Mask)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	0147F0
	DocumentName
	Test case M06
	54 65 73 74 20 43 61 73 65 20 4D 30 36 00

	014800


014810
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	014820
	Xresolution 
	400 pixels/inch
	C8 00 00 00 01 00 00 00

	014828
	Yresolution 
	400 pixels/inch
	C8 00 00 00 01 00 00 00

	014830
	XPosition
	0 offset
	00 00 00 00 C8 00 00 00

	014838
	YPosition
	0 offset
	00 00 00 00 C8 00 00 00

	014840


014850


014860


014870


014880
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	014884
	ImageLayer
	2 (mask), 1 (first image to be imaged)
	02 00 00 00 01 00 00 00

	IFD 1 ( SubIFD 0 ( Child IFD 0 (Frgnd)  Offset 84108 (0x1488cH) NextIFDOffset 84398 (0x149aeH)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	01488C
	Number of Directory Entries
	–
	–
	–
	19 entries
	13 00

	01488E
	NewSubfileType
	254
	Long
	16
	Child IFD (foreground) of a single page of a multi-page
	FE 00 04 00 01 00 00 00 10 00 00 00

	01489A
	ImageWidth
	256
	Long
	1728
	1728 pels
	00 01 04 00 01 00 00 00 C0 06 00 00

	0148A6
	ImageLength
	257
	Long
	n/a
	2200 lines
	01 01 04 00 01 00 00 00 98 08 00 00

	0148B2
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 76 49 01 00

	0148BE
	Compression
	259
	Short
	7
	No compression
	03 01 03 00 01 00 00 00 01 00 00 00

	0148CA
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	0148D6
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	0148E2
	StripOffsets
	273
	Long
	n/a
	98044 bytes
	11 01 04 00 01 00 00 00 FC 7E 01 00

	0148EE
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	0148FA
	RowsPerStrip
	278
	Long
	(ImageLength
	2200 rows
	16 01 04 00 01 00 00 00 98 08 00 00

	014906
	StripByteCounts
	279
	Long
	n/a
	0 bytes
	17 01 04 00 01 00 00 00 00 00 00 00

	014912
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 7E 49 01 00

	01491E
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 86 49 01 00

	01492A
	XPosition
	286
	Rational
	n/a
	0 X-offset
	1E 01 05 00 01 00 00 00 8E 49 01 00

	014936
	YPosition
	287
	Rational
	n/a
	0 Y-offset
	1F 01 05 00 01 00 00 00 96 49 01 00

	014942
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	01494E
	PageNumber
	297
	Short
	n/a
	2 of 2
	29 01 03 00 02 00 00 00 01 00 02 00

	01495A
	DefaultImageColor
	434
	Short
	black
	84382 offset
	B2 01 03 00 03 00 00 00 9E 49 01 00

	014966
	ImageLayer
	34732
	Long
	3, 1
	3 (foreground), 1 (first image to be imaged)
	AC 87 04 00 02 00 00 00 A6 49 01 00

	
	
	
	
	
	
	Next IFD offset AE 49 01 00

	IFD 1 ( SubIFD 0 ( Child IFD 0 (Frgnd)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	014976
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	01497E
	Xresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	014986
	Yresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	01498E
	XPosition
	3.600 offset
	00 00 00 00 C8 00 00 00

	014996
	YPosition
	2.140 offset
	00 00 00 00 C8 00 00 00

	01499E
	DefaultImageColor
	L*=100, a*=0, b*=3.92
	FF 00 80 00 65 00

	0149A6
	ImageLayer
	3 (foreground), 1 (first image to be imaged)
	03 00 00 00 01 00 00 00

	IFD 1 ( SubIFD 0 ( Child IFD 1 (Frgnd)  Offset 84398 (0x149aeH) NextIFDOffset 0 (0x0H)

	Offset
	Description
	Tag code
	Type
	Minimum capabilities
	Comments
	Coding sample

	0149AE
	Number of Directory Entries
	–
	–
	–
	22 entries
	16 00

	0149B0
	NewSubfileType
	254
	Long
	16
	Child IFD (foreground) of a single page of a multi-page
	FE 00 04 00 01 00 00 00 10 00 00 00

	0149BC
	ImageWidth
	256
	Long
	1728
	1015 pels
	00 01 04 00 01 00 00 00 F7 03 00 00

	0149C8
	ImageLength
	257
	Long
	n/a
	1174 lines
	01 01 04 00 01 00 00 00 96 04 00 00

	0149D4
	BitsPerSample
	258
	Short
	1
	8, 8, 8 bit/sample
	02 01 03 00 03 00 00 00 BC 4A 01 00

	0149E0
	Compression
	259
	Short
	7
	JPEG
	03 01 03 00 01 00 00 00 07 00 00 00

	0149EC
	Photometric Interpretation
	262
	Short
	10
	ITU L*a*b*
	06 01 03 00 01 00 00 00 0A 00 00 00

	0149F8
	FillOrder
	266
	Short
	2
	MSB first
	0A 01 03 00 01 00 00 00 01 00 00 00

	014A04
	DocumentName
	269
	ASCII
	n/a
	Test case M06
	0D 01 02 00 0E 00 00 00 C4 4A 01 00

	014A10
	ImageDescription
	270
	ASCII
	n/a
	TIFF-FX Profile M Test Sources
	0E 01 02 00 1F 00 00 00 D4 4A 01 00

	014A1C
	StripOffsets
	273
	Long
	n/a
	98044 bytes
	11 01 04 00 01 00 00 00 FC 7E 01 00

	014A28
	SamplesPerPixel
	277
	Short
	1
	3 sample/pixel
	15 01 03 00 01 00 00 00 03 00 00 00

	014A34
	RowsPerStrip
	278
	Long
	(ImageLength
	1174 rows
	16 01 04 00 01 00 00 00 96 04 00 00

	014A40
	StripByteCounts
	279
	Long
	n/a
	163988 bytes
	17 01 04 00 01 00 00 00 94 80 02 00

	014A4C
	Xresolution
	282
	Rational
	200
	200 pixels/inch
	1A 01 05 00 01 00 00 00 F4 4A 01 00

	014A58
	Yresolution
	283
	Rational
	200
	200 pixels/inch
	1B 01 05 00 01 00 00 00 FC 4A 01 00

	014A64
	XPosition
	286
	Rational
	n/a
	1.185 X-offset
	1E 01 05 00 01 00 00 00 04 4B 01 00

	014A70
	YPosition
	287
	Rational
	n/a
	1.345 Y-offset
	1F 01 05 00 01 00 00 00 0C 4B 01 00

	014A7C
	ResolutionUnit
	296
	Short
	2
	inch
	28 01 03 00 01 00 00 00 02 00 00 00

	014A88
	PageNumber
	297
	Short
	n/a
	2 of 2
	29 01 03 00 02 00 00 00 01 00 02 00

	014A94
	Software
	305
	ASCII
	n/a
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	31 01 02 00 41 00 00 00 14 4B 01 00

	014AA0
	ChromaSubSampling
	530
	Short
	4:1:1
	4:1:1
	12 02 03 00 02 00 00 00 02 00 02 00

	014AAC
	ImageLayer
	34732
	Long
	3, 1
	3 (foreground), 2 (second image to be imaged)
	AC 87 04 00 02 00 00 00 58 4B 01 00

	
	
	
	
	
	
	Next IFD offset 00 00 00 00

	IFD 1 ( SubIFD 0 ( Child IFD 1 (Frgnd)  Values longer than 4 bytes

	Offset
	Description
	Comments
	Coding

	014ABC
	BitsPerSample
	8, 8, 8
	08 00 08 00 08 00

	014AC4
	DocumentName
	Test Case M06
	54 65 73 74 20 43 61 73 65 20 4D 30 36 00

	014AD4


014AE4
	ImageDescription
	TIFF-FX Profile M Test Sources
	54 49 46 46 2D 46 58 20 50 72 6F 66 69 6C 65 20

4D 20 54 65 73 74 20 53 6F 75 72 63 65 73 00

	014AF4
	Xresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	014AFC
	Yresolution 
	200 pixels/inch
	C8 00 00 00 01 00 00 00

	014B04
	XPosition
	1.185 offset
	ED 00 00 00 C8 00 00 00

	014B0C
	YPosition
	1.345 offset
	0D 01 00 00 C8 00 00 00

	014B14


014B24


014B34


014B44


014B54
	Software
	Xerox TIFF-FX Reference Implementation Mon Feb 22 07:02:59 PM 99
	58 65 72 6F 78 20 54 49 46 46 2D 46 58 20 52 65

66 65 72 65 6E 63 65 20 49 6D 70 6C 65 6D 65 6E

74 61 74 69 6F 6E 20 4D 6F 6E 20 46 65 62 20 32

32 20 30 37 3A 30 32 3A 35 39 20 50 4D 20 39 39

00

	014B58
	ImageLayer
	3 (foreground), 2 (second image to be imaged)
	03 00 00 00 02 00 00 00

	Image data

	Offset
	Description
	Comments

	014b60H - 017ef9H
	IFD 1 (Mask) Strip 0
	Compressed data

	017efcH - 017efcH
	IFD 1, Child IFD 0 (Frgnd) Strip 0
	Compressed data

	017efcH - 03ff90H
	IFD 1, Child IFD 1 (Frgnd) Strip 0
	Compressed data


	ITU-T  RECOMMENDATIONS  SERIES

	Series A
	Organization of the work of the ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure

	Series Z
	Languages and general software aspects for telecommunication systems



