
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	V.59

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Corrigendum 2

(03/2002)   

	
	SERIES V: DATA COMMUNICATION OVER THE TELEPHONE NETWORK

Transmission quality and maintenance



	
	Managed objects for diagnostic information of public switched telephone network connected V-series modem DCEs

Corrigendum 2



	
	ITU‑T  Recommendation  V.59 (2000)  –  Corrigendum 2




ITU-T V-SERIES  RECOMMENDATIONS

DATA COMMUNICATION OVER THE TELEPHONE NETWORK

	
	

	General
	V.1–V.9

	Interfaces and voiceband modems
	V.10–V.34

	Wideband modems
	V.35–V.39

	Error control
	V.40–V.49

	Transmission quality and maintenance
	V.50–V.59

	Simultaneous transmission of data and other signals
	V.60–V.99

	Interworking with other networks
	V.100–V.199

	Interface layer specifications for data communication
	V.200–V.249

	Control procedures
	V.250–V.299

	Modems on digital circuits
	V.300–V.399

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation V.59

Managed objects for diagnostic information of public 
switched telephone network connected V-series modem DCEs

Corrigendum 2




	


	Source

Corrigendum 2 to ITU‑T Recommendation V.59 was prepared by ITU‑T Study Group 16 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 29 March 2002.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS


Page
1)
Clause 5 – Clarification to resolve conflicts between text of ITU-T Rec. V.59 and ASN.1

1
2)
Clause 6.13 – Conflict between text definition and ASN.1 of some diagnostic objects

1
3)
Table 17 – Clarification of the definition of txCarrier and rxCarrier low-level objects

2
4)
Clause 6.8.2, 6.8.4 and 6.12: Typographical errors

3
ITU-T Recommendation V.59

Managed objects for diagnostic information of public 
switched telephone network connected V-series modem DCEs

Corrigendum 2

1)
Clause 5 – Clarification to resolve conflicts between text of ITU-T Rec. V.59 and ASN.1

Text intended to clarify any conflict between the text of ITU-T Rec. V.59 and the ASN.1 definitions in Annex A was erroneously omitted. The intent has always been to follow the example of other Recommendations that use ASN.1. 

The following sentence is to be added to the end of the final paragraph of clause 5 just prior to 5.1: "In the case of a conflict between the ASN.1 and the text, the ASN.1 governs."
ORIGINAL TEXT


The presentation of the managed objects within this Recommendation complies with the specification of ASN.1 (see ITU-T X.680). The ASN.1 as defined in Annex A should be used in conjunction with the object definitions to provide a complete overview of the diagnostics. The ASN.1 encoding in Annex A should employ the BASIC-ALIGNED version of Packed Encoding Rules (PER) according to ITU-T X.691.

END ORIGINAL

REPLACEMENT TEXT


The presentation of the managed objects within this Recommendation complies with the specification of ASN.1 (see ITU-T Rec. X.680). The ASN.1 as defined in Annex A should be used in conjunction with the object definitions to provide a complete overview of the diagnostics. The ASN.1 encoding in Annex A should employ the BASIC-ALIGNED version of Packed Encoding Rules (PER) according to ITU-T Rec. X.691. In the case of a conflict between the ASN.1 and the text, the ASN.1 governs.

END REPLACEMENT


2)
Clause 6.13 – Conflict between text definition and ASN.1 of some diagnostic objects

The following list of diagnostic objects are defined as not mandatory in 6.13, but are not indicated as being optional in the ASN.1: txThroughput, rxThroughput, txErrors and rxErrors in the DTE DCE high-level object set.

Since these are all derived objects, they are to be defined as optional. Consequently, the ASN.1 in Annex A should be modified accordingly. 

ORIGINAL TEXT


DTEDCE::= SEQUENCE OF SEQUENCE

{


txFlowControl

V59String,


rxFlowControl

V59String,


protocol




V59String,


txSpeed




INTEGER (50..1677215),


rxSpeed




INTEGER (50..1677215),


txThroughput

INTEGER (50..1677215),


rxThroughput

INTEGER (50..1677215),


txErrors




INTEGER (0..65535),


rxErrors




INTEGER (0..65535),


...

}

END ORIGINAL


REPLACEMENT TEXT


DTEDCE::= SEQUENCE OF SEQUENCE

{


txFlowControl

V59String,


rxFlowControl

V59String,


protocol




V59String,


txSpeed




INTEGER (50..1677215),


rxSpeed




INTEGER (50..1677215),


txThroughput

INTEGER (50..1677215) OPTIONAL,


rxThroughput

INTEGER (50..1677215) OPTIONAL,


txErrors




INTEGER (0..65535) OPTIONAL,


rxErrors




INTEGER (0..65535) OPTIONAL,


...

}

END REPLACEMENT


3)
Table 17 – Clarification of the definition of txCarrier and rxCarrier low-level objects

Some definitions for the Call Termination object Result Codes between the text and ASN.1 do not correspond with each other. The definitions for both text and ASN.1 are shown in the following table.

	Object Identifier
	Text
	ASN.1

	CarrierLost
	53
	60

	TrainingFailed
	54
	61

	NoModulationinCommon
	56
	62

	RetrainFailed
	Missing
	63


The correction is to align the text values with the ASN.1 values. Replace the second half of Table 17/V.59 with a new version as shown below.
REPLACEMENT TEXT


Table 17/V.59 ( Call Termination Result Codes (concluded)
	Mnemonic
	Code
	Description

	noDialTone
	47
	Outgoing call failed due there being no dial tone detected

	voiceDetected
	48
	Connection failed to voice being detected

	reorderTone
	49
	Call failed due to the detection of the reorder tone

	sitTone
	50
	Call failed due to the detection of the special information tone

	engagedTone
	51
	Call progress failed due to the detection of the Engaged tone

	longSpaceDisconnect
	52
	Modem disconnected using the long space criteria

	carrierLost
	60
	Modem disconnected due to loss of carrier

	trainingFailed
	61
	Modem disconnected due to failure in modem training

	noModulationinCommon
	62
	Modems failed to connect due to there being no common modulation mode

	retrainFailed
	63
	Connection failed due to number of retrain failures

	retrainAttemptCountExceeded
	64
	Connection terminated due to retrain count exceeded

	gstnCleardownReceived
	65
	Connection terminated with correct CLEARDOWN exchange

	faxDetected
	66
	Connection terminated due to detecting facsimile terminal

	protocolError
	80
	Connection terminated due to excess protocol errors

	userDisconnect
	90
	Connection terminated due to user request

	onHoldTimerExpired
	100
	Connection terminated because on-hold timer expired

	onHoldRemoteDisc
	101
	Connection terminated because remote modem disconnected while on hold


END REPLACEMENT


4)
Clause 6.8.2, 6.8.4 and 6.12: Typographical errors

The following typographical errors need to be corrected:

4.1)
In 6.8.2, for the V.8 bis Diagnostics, in Table 11 the definitions for nAK3 and nAK4 are incorrectly indicated as all being nAK2. Table 11 is replaced as shown below. 

REPLACEMENT TEXT


Table 11/V.59 ( Definition for NAK types

	Mnemonic
	Code
	Description

	none
	0
	No NAK sequence transmitted or received

	nAK1
	1
	NAK1 message transmitted or received

	nAK2
	2
	NAK2 message transmitted or received

	nAK3
	3
	NAK3 message transmitted or received

	nAK4
	4
	NAK4 message transmitted or received


END REPLACEMENT


4.2)
In 6.8.4, for the V.91 Diagnostic Objects summary, the Tag-IDs for the following objects are incorrect: txDataHistory, rxDataHistory, noiseEstimate and rxSignalQuality. The summary text is replaced as shown below.

ORIGINAL TEXT


V.91 Modulation Objects


Object Identifier
Tag-ID


modeV91


0A00


iNFO0_TX†

0A41


iNFO0_RX†

0A42


cP†




0A54


v91TxPowerLevel†
0A45, 0A46, 0A47: This object uses the TxPowerLevel definition.


controlChannel
0A01


transparentMode
0A02


txDataHistory†
0A4D


rxDataHistory†
0A4E


noiseEstimate†
0A50


rxSignalQuality†
0A51


rBSpattern†


0A55


digitalPadLoss†
0A56


localCodecLaw
0A04


remoteCodecLaw
0A05


frameSlipsDetected
0A03

END ORIGINAL


REPLACEMENT TEXT


V.91 Modulation Objects


Object Identifier
Tag-ID


modeV91


0A00


iNFO0_TX†

0A41


iNFO0_RX†

0A42


cP†




0A54


v91TxPowerLevel†
0A45, 0A46, 0A47: This object uses the TxPowerLevel definition.


controlChannel
0A01


transparentMode
0A02


txDataHistory†
0A4C


rxDataHistory†
0A4D


noiseEstimate†
0A4F


rxSignalQuality†
0A50


rBSpattern†


0A55


digitalPadLoss†
0A56


localCodecLaw
0A04


remoteCodecLaw
0A05


frameSlipsDetected
0A03

END REPLACEMENT


4.3)
In 6.12, in the summary of the Error Control Objects, the object 'featureNegotiation' is incorrect. The correct object name  is 'v42featureNegotiation'.

ORIGINAL TEXT


Error Control Objects


Object Identifier

Tag-ID


ModeV42



2F00


protocolNegotiation

2F01


featureNegotiation

2F02


txFrameSize


2F03


rxFrameSize


2F04


txWindow



2F05


rxWindow



2F06


linkTimeout



2F07


framesSentAck

2F08


framesRetransmitted

2F09


framesSentAck

2F0A


framesReceivedDiscard
2F0B


txErrors




2F0C


rxErrors



2F0D


txThroughput


2F0E


rxThroughput


2F0F

END ORIGINAL


REPLACEMENT TEXT


Error Control Objects


Object Identifier

Tag-ID


ModeV42



2F00


protocolNegotiation

2F01


v42featureNegotiation
2F02


txFrameSize


2F03


rxFrameSize


2F04


txWindow



2F05


rxWindow



2F06


linkTimeout



2F07


framesSentAck

2F08


framesRetransmitted

2F09


framesSentAck

2F0A


framesReceivedDiscard
2F0B


txErrors




2F0C


rxErrors



2F0D


txThroughput


2F0E


rxThroughput


2F0F

END REPLACEMENT


	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	


