
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	X.521

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(02/2001)   

	
	SERIES X: DATA NETWORKS AND OPEN SYSTEM COMMUNICATIONS

Directory



	
	Information technology – Open Systems Interconnection – The Directory: Selected
object classes

	
	ITU‑T  Recommendation  X.521

(Formerly  CCITT  Recommendation)


ITU-T X-SERIES  RECOMMENDATIONS

DATA NETWORKS AND OPEN SYSTEM COMMUNICATIONS

	
	

	PUBLIC DATA NETWORKS
	

	Services and facilities
	X.1–X.19

	Interfaces
	X.20–X.49

	Transmission, signalling and switching
	X.50–X.89

	Network aspects
	X.90–X.149

	Maintenance
	X.150–X.179

	Administrative arrangements
	X.180–X.199

	OPEN SYSTEMS INTERCONNECTION
	

	Model and notation
	X.200–X.209

	Service definitions
	X.210–X.219

	Connection-mode protocol specifications
	X.220–X.229

	Connectionless-mode protocol specifications
	X.230–X.239

	PICS proformas
	X.240–X.259

	Protocol Identification
	X.260–X.269

	Security Protocols
	X.270–X.279

	Layer Managed Objects
	X.280–X.289

	Conformance testing
	X.290–X.299

	INTERWORKING BETWEEN NETWORKS
	

	General
	X.300–X.349

	Satellite data transmission systems
	X.350–X.369

	IP-based networks
	X.370–X.399

	MESSAGE HANDLING SYSTEMS
	X.400–X.499

	DIRECTORY
	X.500–X.599

	OSI NETWORKING AND SYSTEM ASPECTS
	

	Networking
	X.600–X.629

	Efficiency
	X.630–X.639

	Quality of service
	X.640–X.649

	Naming, Addressing and Registration
	X.650–X.679

	Abstract Syntax Notation One (ASN.1)
	X.680–X.699

	OSI MANAGEMENT
	

	Systems Management framework and architecture
	X.700–X.709

	Management Communication Service and Protocol
	X.710–X.719

	Structure of Management Information
	X.720–X.729

	Management functions and ODMA functions
	X.730–X.799

	SECURITY
	X.800–X.849

	OSI APPLICATIONS
	

	Commitment, Concurrency and Recovery
	X.850–X.859

	Transaction processing
	X.860–X.879

	Remote operations
	X.880–X.899

	OPEN DISTRIBUTED PROCESSING
	X.900–X.999

	
	


For further details, please refer to the list of ITU-T Recommendations.

	INTERNATIONAL  STANDARD  ISO/IEC  9594-7

ITU-T  RECOMMENDATION  X.521

Information technology – Open Systems Interconnection –
The Directory: Selected object classes




	Summary

This Recommendation | International Standard defines a number of selected object classes and name forms which may be found useful across a range of applications of the Directory. An object class definition specifies the attribute types which are relevant to the objects of that class. A name form definition specifies the attributes to be used in forming names for the objects of a given class.


	Source

ITU‑T Recommendation X.521 was prepared by ITU‑T Study Group 7 (2001‑2004) and approved on 2 February 2001. An identical text is also published as ISO/IEC  9594-7.


	Note

Implementors and users should note that a defect resolution process exists and that corrections may be applied to this Recommendation | International Standard in the form of Technical Corrigenda. Identical corrections may also be applied to this Recommendation in the form of an Implementor's Guide. A list of approved Technical Corrigenda for this International Standard can be obtained from the ISO website and published Technical Corrigenda can be obtained from your national standards organization. Technical Corrigenda and Implementor's Guide for this Recommendation may be obtained from the ITU-T website.


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2002

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page
1SECTION  1  –  GENERAL



11
Scope



12
Normative references



12.1
Identical Recommendations | International Standards



23
Definitions



23.1
OSI Reference Model definitions



23.2
Directory Model definitions



24
Conventions



4SECTION  2  –  SELECTED OBJECT CLASSES



45
Definition of useful attribute sets



45.1
Telecommunication attribute set



45.2
Postal attribute set



45.3
Locale attribute set



45.4
Organizational attribute set



56
Definition of selected object classes



56.1
Country



56.2
Locality



56.3
Organization



56.4
Organizational Unit



56.5
Person



66.6
Organizational Person



66.7
Organizational Role



66.8
Group of Names



76.9
Group of Unique Names



76.10
Residential Person



76.11
Application Process



76.12
Application Entity



86.13
DSA



86.14
Device



86.15
Strong Authentication User



86.16
User Security Information



96.17
Certification Authority



96.18
Certification Authority-V2



96.19
DMD




10SECTION  3  –  SELECTED NAME FORMS



107
Definition of selected name forms



107.1
Country name form



107.2
Locality name form



107.3
State or Province name form



107.4
Organization name form



107.5
Organizational Unit name form



107.6
Person name form



117.7
Organizational Person name form



117.8
Organizational Role name form



117.9
Group of Names name form



117.10
Residential Person name form



117.11
Application Process name form



117.12
Application Entity name form



127.13
DSA name form



127.14
Device name form



127.15
DMD name form



Page
13Annex  A – Selected object classes and name forms in ASN.1



19Annex  B – Suggested name forms and DIT structures



20B.1
Country



20B.2
Organization



20B.3
Locality



21B.4
Organizational Unit



21B.5
Organizational Person



21B.6
Organizational Role



21B.7
Group of Names



22B.8
Residential Person



22B.9
Application Entity



22B.10
Device




22B.11
Application Process



23B.12
Alternative Structure Rule for Locality



24Annex  C – Amendments and corrigenda





Introduction

This Recommendation | International Standard, together with other Recommendations | International Standards, has been produced to facilitate the interconnection of information processing systems to provide directory services. A set of such systems, together with the directory information that they hold, can be viewed as an integrated whole, called the Directory. The information held by the Directory, collectively known as the Directory Information Base (DIB), is typically used to facilitate communication between, with or about objects such as application entities, people, terminals, and distribution lists.

The Directory plays a significant role in Open Systems Interconnection, whose aim is to allow, with a minimum of technical agreement outside of the interconnection standards themselves, the interconnection of information processing systems:

–
from different manufacturers;

–
under different managements;

–
of different levels of complexity; and

–
of different ages.

This Recommendation | International Standard defines a number of attribute sets and object classes which may be found useful across a range of applications of the Directory.

This fourth edition technically revises and enhances, but does not replace, the third edition of this Recommendation | International Standard. Implementations may still claim conformance to the third edition. However, at some point, the third edition will not be supported (i.e. reported defects will no longer be resolved). It is recommended that implementations conform to this fourth edition as soon as possible.

This fourth edition specifies version 1 and version 2 of the Directory protocols.

The first and second editions specified only version 1. Most of the services and protocols specified in this edition are designed to function under version 1. However some enhanced services and protocols, e.g. signed errors, will not function unless all Directory entities involved in the operation have negotiated version 2. Whichever version has been negotiated, differences between the services and between the protocols defined in the four editions, except for those specifically assigned to version 2, are accommodated using the rules of extensibility defined in this edition of ITU-T Rec. X.519 | ISO/IEC 9594-5. 

Annex A, which is an integral part of this Recommendation | International Standard, provides an ASN.1 module containing all of the type and value definitions which appear in this Recommendation | International Standard. 

Annex B, which is not an integral part of this Recommendation | International Standard, provides some common naming and structure rules which may or may not be used by administrative authorities.

Annex C, which is not an integral part of this Recommendation | International Standard, lists the amendments and defect reports that have been incorporated to form this edition of this Recommendation | International Standard.

INTERNATIONAL  STANDARD

ISO/IEC 9594-7:2001 (E)

ITU-T  RECOMMENDATION

Information technology – Open Systems Interconnection –
The Directory: Selected object classes

SECTION  1  –  GENERAL

1
Scope

This Recommendation | International Standard defines a number of object classes and name forms which may be found useful across a range of applications of the Directory. The definition of an object class involves listing a number of attribute types which are relevant to objects of that class. The definition of a name form involves naming the object class to which it applies and listing the attributes to be used in forming names for objects of that class. These definitions are used by the administrative authority which is responsible for the management of the directory information.

Any administrative authority can define its own object classes or subclasses and name forms for any purpose. 

NOTE 1 – Those definitions may or may not use the notation specified in ITU-T Rec. X.501 | ISO/IEC 9594-2.

NOTE 2 – It is recommended that an object class defined in this Recommendation | International Standard, or a subclass derived from one, or a name form defined in this Recommendation | International Standard, be used in preference to the generation of a new one, whenever the semantics is appropriate for the application.

Administrative authorities may support some or all the selected object classes and name forms, and may also add additional ones.

All administrative authorities shall support the object classes which the directory uses for its own purpose (the top, alias and DSA object classes).

2
Normative references

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent editions of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations.

2.1
Identical Recommendations | International Standards

–
ITU-T Recommendation X.200 (1994) | ISO/IEC 7498-1:1994, Information technology – Open Systems Interconnection – Basic Reference Model: The Basic Model.
–
ITU-T Recommendation X.500 (2001) | ISO/IEC 9594-1:2001, Information technology – Open Systems Interconnection – The Directory: Overview of concepts, models and services.

–
ITU-T Recommendation X.501 (2001) | ISO/IEC 9594-2:2001, Information technology – Open Systems Interconnection – The Directory: Models.

–
ITU-T Recommendation X.509 (2000) | ISO/IEC 9594-8:2001, Information technology – Open Systems Interconnection – The Directory: Public-key and attribute certificate frameworks.
–
ITU-T Recommendation X.511 (2001) | ISO/IEC 9594-3:2001, Information technology – Open Systems Interconnection – The Directory: Abstract service definition.
–
ITU-T Recommendation X.518 (2001) | ISO/IEC 9594-4:2001, Information technology – Open Systems Interconnection – The Directory: Procedures for distributed operation.
–
ITU-T Recommendation X.519 (2001) | ISO/IEC 9594-5:2001, Information technology – Open Systems Interconnection – The Directory: Protocol specifications.
–
ITU-T Recommendation X.520 (2001) | ISO/IEC 9594-6:2001, Information technology – Open Systems Interconnection – The Directory: Selected attribute types.
–
ITU-T Recommendation X.525 (2001) | ISO/IEC 9594-9:2001, Information technology – Open Systems Interconnection – The Directory: Replication.
–
ITU-T Recommendation X.530 (2001) | ISO/IEC 9594-10:2001, Information technology – Open Systems Interconnection – The Directory: Use of systems management for administration of the Directory.
–
ITU-T Recommendation X.680 (1997) | ISO/IEC 8824-1:1998, Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation.
–
ITU-T Recommendation X.681 (1997) | ISO/IEC 8824-2:1998, Information technology – Abstract Syntax Notation One (ASN.1): Information object specification.
–
ITU-T Recommendation X.682 (1997) | ISO/IEC 8824-3:1998, Information technology – Abstract Syntax Notation One (ASN.1): Constraint specification.
–
ITU-T Recommendation X.683 (1997) | ISO/IEC 8824-4:1998, Information technology – Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 specifications.
3
Definitions

For the purposes of this Recommendation | International Standard, the following definitions apply.

3.1
OSI Reference Model definitions

The following terms are defined in ITU-T Rec. X.200 | ISO/IEC 7498-1.

a)
application-entity;

b)
application-process.

3.2
Directory Model definitions

The following terms are defined in ITU-T Rec. X.501 | ISO/IEC 9594-2:

a)
attribute;

b)
attribute type;

c)
Directory Information Tree (DIT
);

d)
Directory System Agent (DSA)
;

e)
attribute set;

f)
entry;

g)
name;

h)
object class;

i)
subclass;

j)
name form;

k)
structure rule.

4
Conventions

With minor exceptions, this Directory Specification has been prepared according to the "Presentation of ITU-T | ISO/IEC common text" guidelines in the Guide for ITU-T and ISO/IEC JTC 1 Cooperation.

The term "Directory Specification" (as in "this Directory Specification") shall be taken to mean this Recommendation | International Standard. The term "Directory Specifications" shall be taken to mean the X.500-series Recommendations | parts of ISO/IEC 9594.

This Directory Specification uses the term "1988 edition systems" to refer to systems conforming to the first edition of the Directory Specifications, i.e. the 1988 edition of the series of CCITT X.500 Recommendations and the ISO/IEC 9594:1990 edition. This Directory Specification uses the term "1993 edition systems" to refer to systems conforming to the second (1993) edition of the Directory Specifications, i.e. the 1993 edition of the series of ITU-T X.500 Recommendations and the ISO/IEC 9594:1995 edition. This Directory Specification uses the term "1997 edition systems" to refer to systems conforming to the third edition of the Directory Specifications, i.e. the 1997 edition of the series of ITU-T X.500 Recommendations and the ISO/IEC 9594:1998 edition. This Directory Specification uses the term "4th edition systems" to refer to systems conforming to this fourth edition of the Directory Specifications, i.e. the 2001 editions of ITU-T X.500, X.501, X.511, X.518, X.519, X.520, X.521, X.525, and X.530, the 2000 edition of ITU‑T X.509, and parts 1-10 of the ISO/IEC 9594:2001 edition.

This Directory Specification presents ASN.1 notation in the bold Helvetica typeface. When ASN.1 types and values are referenced in normal text, they are differentiated from normal text by presenting them in the bold Helvetica typeface. The names of procedures, typically referenced when specifying the semantics of processing, are differentiated from normal text by displaying them in bold Times. Access control permissions are presented in italicized Times.

Object classes and name forms are defined in this Directory Specification as values of the OBJECT-CLASS and NAME-FORM information object classes defined in ITU-T Rec. X.501 | ISO/IEC 9594-2.

SECTION  2  –  SELECTED OBJECT CLASSES

5
Definition of useful attribute sets

5.1
Telecommunication attribute set

This set of attributes is used to define those which are commonly used for business communications.

TelecommunicationAttributeSet ATTRIBUTE  ::=  {


facsimileTelephoneNumber |


internationalISDNNumber |


telephoneNumber |

--
teletexTerminalIdentifier |

Attribute type has been deleted


telexNumber |


preferredDeliveryMethod |


destinationIndicator |


registeredAddress |


x121Address }

5.2
Postal attribute set

This set of attributes is used to define those which are directly associated with postal delivery.

PostalAttributeSet  ATTRIBUTE  ::=  {


physicalDeliveryOfficeName |


postalAddress |


postalCode |


postOfficeBox |


streetAddress }

5.3
Locale attribute set

This set of attributes is used to define those which are commonly used for search purposes to indicate the locale of an object.

LocaleAttributeSet  ATTRIBUTE  ::=  {


localityName |


stateOrProvinceName |


streetAddress }

5.4
Organizational attribute set

This set of attributes is used to define the attributes that an organization or organizational unit may typically possess.

OrganizationalAttributeSet  ATTRIBUTE  ::=  {


description |


LocaleAttributeSet |


PostalAttributeSet |


TelecommunicationAttributeSet |


businessCategory |


seeAlso |


searchGuide |


userPassword }

6
Definition of selected object classes

6.1
Country

A Country object class is used to define country entries in the DIT.

country  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ countryName }


MAY CONTAIN
{ description | searchGuide }


ID


id-oc-country }

6.2
Locality

The Locality object class is used to define locality in the DIT.

locality  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MAY CONTAIN
{ description | 





searchGuide |





LocaleAttributeSet | 





seeAlso }


ID


id-oc-locality }

At least one of Locality Name or State or Province Name shall be present.

6.3
Organization

The Organization object class is used to define organization entries in the DIT.

organization  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ organizationName }


MAY CONTAIN
{ OrganizationalAttributeSet }


ID


id-oc-organization }

6.4
Organizational Unit

The Organizational Unit object class is used to define entries representing subdivisions of organizations.

organizationalUnit  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ organizationalUnitName }


MAY CONTAIN
{ OrganizationalAttributeSet }


ID


id-oc-organizationalUnit }

6.5
Person

The Person object class is used to define entries representing people generically.

person  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName | surname }


MAY CONTAIN
{ description |





telephoneNumber |





userPassword |





seeAlso }


ID


id-oc-person }

6.6
Organizational Person 

The Organizational Person object class is used to define entries representing people employed by, or in some other important way associated with, an organization.

organizationalPerson  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ person }


MAY CONTAIN
{ LocaleAttributeSet |





PostalAttributeSet |





TelecommunicationAttributeSet |





organizationalUnitName |





title }





ID
id-oc-organizationalPerson }

6.7
Organizational Role 

The Organizational Role object class is used to define entries representing an organizational role, i.e. a position or role within an organization. An organizational role is normally considered to be filled by a particular organizational person. Over its lifetime, however, an organizational role may be filled by a number of different organizational people in succession. In general, an organizational role may be filled by a person or a non-human entity.

organizationalRole  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName }


MAY CONTAIN
{ description |





LocaleAttributeSet |





organizationalUnitName |





PostalAttributeSet |





preferredDeliveryMethod |





roleOccupant |





seeAlso |





TelecommunicationAttributeSet }


ID


id-oc-organizationalRole }

6.8
Group of Names 

The Group Of Names object class is used to define entries representing an unordered set of names which represent individual objects or other groups of names. The membership of a group is static, i.e. it is explicitly modified by administrative action, rather than dynamically determined each time the group is referred to.

The membership of a group can be reduced to a set of individual object's names by replacing each group with its membership. This process could be carried out recursively until all constituent group names have been eliminated, and only the names of individual objects remain.

groupOfNames  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName | member }


MAY CONTAIN
{ description |





organizationName |





organizationalUnitName |





owner |





seeAlso |





businessCategory }


ID


id-oc-groupOfNames }

6.9
Group of Unique Names 

The Group Of Unique Names object class is used to define entries representing an unordered set of names whose integrity can be assured and which represent individual objects or other groups of names. The membership of a group is static, i.e. it is explicitly modified by administrative action, rather than dynamically determined each time the group is referred to.

groupOfUniqueNames  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName | uniqueMember }


MAY CONTAIN
{ description |





organizationName |





organizationalUnitName |





owner |





seeAlso |





businessCategory }


ID


id-oc-groupOfUniqueNames }

6.10
Residential Person 

The Residential Person object class is used to define entries representing a person in the residential environment.

residentialPerson  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ person }


MUST CONTAIN
{ localityName }


MAY CONTAIN
{ LocaleAttributeSet |





PostalAttributeSet |





preferredDeliveryMethod |





TelecommunicationAttributeSet |





businessCategory }


ID


id-oc-residentialPerson }

6.11
Application Process 

The Application Process object class is used to define entries representing application processes. An application process is an element within a real open-system which performs the information processing for a particular application (see ITU‑T Rec. X.200 | ISO/IEC 7498-1).

applicationProcess  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName }


MAY CONTAIN
{ description |





localityName |





organizationalUnitName |





seeAlso }


ID


id-oc-applicationProcess }

6.12
Application Entity 

The Application Entity object class is used to define entries representing application entities. An application entity consists of those aspects of an application-process pertinent to OSI.

applicationEntity  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName | presentationAddress }


MAY CONTAIN
{ description |





localityName |





organizationName |





organizationalUnitName |





seeAlso |





supportedApplicationContext }


ID


id-oc-applicationEntity }

NOTE – If an application-entity is represented as a Directory object that is distinct from an application-process, the commonName attribute is used to carry the value of the Application Entity Qualifier.

6.13
DSA

The DSA object class is used to define entries representing DSAs. A DSA is as defined in ITU-T Rec. X.501 | ISO/IEC 9594-2.

dSA  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ applicationEntity }


MAY CONTAIN
{ knowledgeInformation }


ID


id-oc-dSA }

6.14
Device

The Device object class is used to define entries representing devices. A device is a physical unit which can communicate, such as a modem, disk drive, etc. 

device  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName }


MAY CONTAIN
{ description |





localityName |





organizationName |





organizationalUnitName |





owner |





seeAlso |





serialNumber }


ID


id-oc-device }

NOTE – At least one of localityName, serialNumber, owner, should be included. The choice is dependent on device type.

6.15
Strong Authentication User

The Strong Authentication User object class is used in defining entries for objects which participate in strong authentication, as defined in ITU-T Rec. X.509 | ISO/IEC 9594-8.

strongAuthenticationUser OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


KIND


auxiliary


MUST CONTAIN
{ userCertificate }


ID


id-oc-strongAuthenticationUser }

NOTE – Use of this object class has been deprecated in favour of the pkiUser and pkiCA object classes defined in ITU-T Rec. X.509 | ISO/IEC 9594-8. Implementations that use strongAuthenticationUser, certificationAuthority and certificationAuthorityv2 object classes are still conformant to the standard, although new implementations are strongly recommended to move to the pkiUser and pkiCA object classes.

6.16
User Security Information

The User Security Information object class is used in defining entries for objects which need to indicate security information associated with them as defined in ITU-T Rec. X.509 | ISO/IEC 9594-8.

userSecurityInformation  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


KIND   

auxiliary


MAY CONTAIN
{ supportedAlgorithms }


ID


id-oc-userSecurityInformation }

6.17
Certification Authority

The Certification Authority object class is used in defining entries for objects which act as certification authorities, as defined in ITU-T Rec. X.509 | ISO/IEC 9594-8.

certificationAuthority  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


KIND   

auxiliary


MUST CONTAIN
{ cACertificate |





certificateRevocationList |





authorityRevocationList }


MAY CONTAIN
{ crossCertificatePair }


ID


id-oc-certificationAuthority }

NOTE – Use of this object class has been deprecated in favour of the pkiUser and pkiCA object classes defined in ITU-T Rec. X.509 | ISO/IEC 9594-8. Implementations that use strongAuthenticationUser, certificationAuthority and certificationAuthorityv2 object classes are still conformant to the standard, although new implementations are strongly recommended to move to the pkiUser and pkiCA object classes.

6.18
Certification Authority-V2

The Certification Authority-V2 object class is used in defining entries for objects which act as certification authorities and can support the delta revocation list as defined in ITU-T Rec. X.509 | ISO/IEC 9594-8.

certificationAuthority-V2  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ certificationAuthority }


KIND   

auxiliary


MAY CONTAIN
{ deltaRevocationList }


ID


id-oc-certificationAuthority-V2 }

NOTE – Use of this object class has been deprecated in favour of the pkiUser and pkiCA object classes defined in ITU-T Rec. X.509 | ISO/IEC 9594-8. Implementations that use strongAuthenticationUser, certificationAuthority and certificationAuthorityv2 object classes are still conformant to the standard, although new implementations are strongly recommended to move to the pkiUser and pkiCA object classes.

6.19
DMD

The DMD object class is used to define DMD entries in the DIT.

dMD  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ dmdName }


MAY CONTAIN
{ OrganizationalAttributeSet }


ID


id-oc-dmd }

SECTION  3  –  SELECTED NAME FORMS

7
Definition of selected name forms

7.1
Country name form

The Country name form specifies how entries of object class country may be named.

countryNameForm  NAME-FORM  ::=  {


NAMES


country


WITH ATTRIBUTES
{ countryName }


ID



id-nf-countryNameForm }

7.2
Locality name form

The Locality name form specifies how entries of object class locality may be named.

locNameForm  NAME-FORM  ::=  {


NAMES


locality


WITH ATTRIBUTES
{ localityName }


ID



id-nf-locNameForm }

7.3
State or Province name form

The State or Province name form specifies how entries of object class locality may be named.

sOPNameForm  NAME-FORM  ::=  {


NAMES


locality


WITH ATTRIBUTES
{ stateOrProvinceName }


ID



id-nf-sOPNameForm }

7.4
Organization name form

The Organization name form specifies how entries of object class organization may be named.

orgNameForm  NAME-FORM  ::=  {


NAMES


organization


WITH ATTRIBUTES
{ organizationName }


ID



id-nf-orgNameForm }

7.5
Organizational Unit name form

The Organizational Unit name form specifies how entries of object class organizationalUnit may be named.

orgUnitNameForm  NAME-FORM  ::=  {


NAMES


organizationalUnit


WITH ATTRIBUTES
{ organizationalUnitName }


ID



id-nf-orgUnitNameForm }

7.6
Person name form

The Person name form specifies how entries of object class person may be named.

personNameForm  NAME-FORM  ::=  {


NAMES


person


WITH ATTRIBUTES
{ commonName }


ID



id-nf-personNameForm }

7.7
Organizational Person name form

The Organizational Person name form specifies how entries of object class organizationalPerson may be named.

orgPersonNameForm  NAME-FORM  ::=  {


NAMES


organizationalPerson


WITH ATTRIBUTES
{ commonName }


AND OPTIONALLY
{ organizationalUnitName }


ID



id-nf-orgPersonNameForm }

7.8
Organizational Role name form

The Organizational Role name form specifies how entries of object class organizationalRole may be named.

orgRoleNameForm  NAME-FORM  ::=  {


NAMES


organizationalRole


WITH ATTRIBUTES
{ commonName }


ID



id-nf-orgRoleNameForm }

7.9
Group of Names name form

The Group of Names name form specifies how entries of object class groupOfNames may be named.

gONNameForm  NAME-FORM  ::=  {


NAMES


groupOfNames


WITH ATTRIBUTES
{ commonName }


ID



id-nf-gONNameForm }

7.10
Residential Person name form

The Residential Person name form specifies how entries of object class residentialPerson may be named.

resPersonNameForm  NAME-FORM  ::=  {


NAMES


residentialPerson


WITH ATTRIBUTES
{ commonName }


AND OPTIONALLY
{ streetAddress }


ID



id-nf-resPersonNameForm }

7.11
Application Process name form

The Application Process name form specifies how entries of object class applicationProcess may be named.

applProcessNameForm  NAME-FORM  ::=  {


NAMES


applicationProcess


WITH ATTRIBUTES
{ commonName }


ID



id-nf-applProcessNameForm }

7.12
Application Entity name form

The Application Entity name form specifies how entries of object class applicationEntity may be named.

applEntityNameForm  NAME-FORM  ::=  {


NAMES


applicationEntity


WITH ATTRIBUTES
{ commonName }


ID



id-nf-applEntityNameForm }

7.13
DSA name form

The DSA name form specifies how entries of object class dSA may be named.

dSANameForm  NAME-FORM  ::=  {


NAMES


dSA


WITH ATTRIBUTES
{ commonName }


ID



id-nf-dSANameForm }

7.14
Device name form

The Device name form specifies how entries of object class device may be named.

deviceNameForm  NAME-FORM  ::=  {


NAMES


device


WITH ATTRIBUTES
{ commonName }


ID



id-nf-deviceNameForm }

7.15
DMD name form

The DMD name form specifies how entries of object class dMD may be named.

dMDNameForm  NAME-FORM  ::=  {


NAMES


dMD


WITH ATTRIBUTES
{ dmdName }


ID



id-nf-dMDNameForm }

Annex  A

Selected object classes and name forms in ASN.1

(This annex forms an integral part of this Recommendation | International Standard)

This annex includes all of the ASN.1 type and value definitions contained in this Directory Specification in the form of the ASN.1 module SelectedObjectClasses.

SelectedObjectClasses {joint-iso-itu-t ds(5) module(1) selectedObjectClasses(6) 4}

DEFINITIONS ::=

BEGIN

--  EXPORTS All  --

--  The types and values defined in this module are exported for use in the other ASN.1 modules contained 

--  within the Directory Specifications, and for the use of other applications which will use them to access 

--  Directory services. Other applications may use them for their own purposes, but this will not constrain

--  extensions and modifications needed to maintain or improve the Directory service.

IMPORTS


authenticationFramework, certificateExtensions, id-nf, id-oc,informationFramework,


objectClass, selectedAttributeTypes



FROM  UsefulDefinitions {joint-iso-itu-t ds(5) module(1) usefulDefinitions(0) 4 }


alias, ATTRIBUTE, NAME-FORM, OBJECT-CLASS, top



FROM  InformationFramework informationFramework


businessCategory, commonName, countryName, description, destinationIndicator, dmdName,


facsimileTelephoneNumber, internationalISDNNumber, knowledgeInformation, localityName,


member, organizationalUnitName, organizationName, owner, physicalDeliveryOfficeName, 


postalAddress, postalCode, postOfficeBox, preferredDeliveryMethod, presentationAddress, 


registeredAddress, roleOccupant, searchGuide, seeAlso, serialNumber, stateOrProvinceName, 


streetAddress, supportedApplicationContext, surname, telephoneNumber,


telexNumber, title, uniqueMember, x121Address



FROM SelectedAttributeTypes selectedAttributeTypes


authorityRevocationList, cACertificate, certificateRevocationList, crossCertificatePair,


deltaRevocationList, supportedAlgorithms, userCertificate, userPassword



FROM AuthenticationFramework authenticationFramework
;

--  Attribute sets  --

TelecommunicationAttributeSet ATTRIBUTE  ::=  {


facsimileTelephoneNumber |


internationalISDNNumber |


telephoneNumber |

--
teletexTerminalIdentifier | 
Attribute type has been deleted


telexNumber |


preferredDeliveryMethod |


destinationIndicator |


registeredAddress |


x121Address }

PostalAttributeSet  ATTRIBUTE  ::=  {


physicalDeliveryOfficeName |


postalAddress |


postalCode |


postOfficeBox |


streetAddress }

LocaleAttributeSet  ATTRIBUTE  ::=  {


localityName |


stateOrProvinceName |


streetAddress }

OrganizationalAttributeSet  ATTRIBUTE  ::=  {


description |


LocaleAttributeSet |


PostalAttributeSet |


TelecommunicationAttributeSet |


businessCategory |


seeAlso |


searchGuide |


userPassword }

--  Object classes  --

country  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ countryName }


MAY CONTAIN
{ description | searchGuide }


ID


id-oc-country }

locality  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MAY CONTAIN
{ description | 





searchGuide |





LocaleAttributeSet | 





seeAlso }


ID


id-oc-locality }

organization  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ organizationName }


MAY CONTAIN
{ OrganizationalAttributeSet }


ID


id-oc-organization }

organizationalUnit  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ organizationalUnitName }


MAY CONTAIN
{ OrganizationalAttributeSet }


ID


id-oc-organizationalUnit }

person  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName | surname }


MAY CONTAIN
{ description |





telephoneNumber |





userPassword |





seeAlso }


ID


id-oc-person }

organizationalPerson  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ person }


MAY CONTAIN
{ LocaleAttributeSet |





PostalAttributeSet |





TelecommunicationAttributeSet |





organizationalUnitName |





title }





ID
id-oc-organizationalPerson }

organizationalRole  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName }


MAY CONTAIN
{ description |





LocaleAttributeSet |





organizationalUnitName |





PostalAttributeSet |





preferredDeliveryMethod |





roleOccupant |





seeAlso |





TelecommunicationAttributeSet }


ID


id-oc-organizationalRole }

groupOfNames  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName | member }


MAY CONTAIN
{ description |





organizationName |





organizationalUnitName |





owner |





seeAlso |





businessCategory }


ID


id-oc-groupOfNames }

groupOfUniqueNames  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName | uniqueMember }


MAY CONTAIN
{ description |





organizationName |





organizationalUnitName |





owner |





seeAlso |





businessCategory }


ID


id-oc-groupOfUniqueNames }

residentialPerson  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ person }


MUST CONTAIN
{ localityName }


MAY CONTAIN
{ LocaleAttributeSet |





PostalAttributeSet |





preferredDeliveryMethod |





TelecommunicationAttributeSet |





businessCategory }


ID


id-oc-residentialPerson }

applicationProcess  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName }


MAY CONTAIN
{ description |





localityName |





organizationalUnitName |





seeAlso }


ID


id-oc-applicationProcess }

applicationEntity  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName | presentationAddress }


MAY CONTAIN
{ description |





localityName |





organizationName |





organizationalUnitName |





seeAlso |





supportedApplicationContext }


ID


id-oc-applicationEntity }

dSA  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ applicationEntity }


MAY CONTAIN
{ knowledgeInformation }


ID


id-oc-dSA }

device  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ commonName }


MAY CONTAIN
{ description |





localityName |





organizationName |





organizationalUnitName |





owner |





seeAlso |





serialNumber }


ID


id-oc-device }

strongAuthenticationUser OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


KIND


auxiliary


MUST CONTAIN
{ userCertificate }


ID


id-oc-strongAuthenticationUser }

userSecurityInformation  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


KIND   

auxiliary


MAY CONTAIN
{ supportedAlgorithms }


ID


id-oc-userSecurityInformation }

certificationAuthority  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


KIND   

auxiliary


MUST CONTAIN
{ cACertificate |





certificateRevocationList |





authorityRevocationList }


MAY CONTAIN
{ crossCertificatePair }


ID


id-oc-certificationAuthority }

certificationAuthority-V2  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ certificationAuthority }


KIND   

auxiliary


MAY CONTAIN
{ deltaRevocationList }


ID


id-oc-certificationAuthority-V2 }

dMD  OBJECT-CLASS  ::=  {


SUBCLASS OF
{ top }


MUST CONTAIN
{ dmdName }


MAY CONTAIN
{ OrganizationalAttributeSet }


ID


id-oc-dmd }

--  Name forms  --

countryNameForm  NAME-FORM  ::=  {


NAMES


country


WITH ATTRIBUTES
{ countryName }


ID



id-nf-countryNameForm }

locNameForm  NAME-FORM  ::=  {


NAMES


locality


WITH ATTRIBUTES
{ localityName }


ID



id-nf-locNameForm }

sOPNameForm  NAME-FORM  ::=  {


NAMES


locality


WITH ATTRIBUTES
{ stateOrProvinceName }


ID



id-nf-sOPNameForm }

orgNameForm  NAME-FORM  ::=  {


NAMES


organization


WITH ATTRIBUTES
{ organizationName }


ID



id-nf-orgNameForm }

orgUnitNameForm  NAME-FORM  ::=  {


NAMES


organizationalUnit


WITH ATTRIBUTES
{ organizationalUnitName }


ID



id-nf-orgUnitNameForm }

personNameForm  NAME-FORM  ::=  {


NAMES


person


WITH ATTRIBUTES
{ commonName }


ID



id-nf-personNameForm }

orgPersonNameForm  NAME-FORM  ::=  {


NAMES


organizationalPerson


WITH ATTRIBUTES
{ commonName }


AND OPTIONALLY
{ organizationalUnitName }


ID



id-nf-orgPersonNameForm }

orgRoleNameForm  NAME-FORM  ::=  {


NAMES


organizationalRole


WITH ATTRIBUTES
{ commonName }


ID



id-nf-orgRoleNameForm }

gONNameForm  NAME-FORM  ::=  {


NAMES


groupOfNames


WITH ATTRIBUTES
{ commonName }


ID



id-nf-gONNameForm }

resPersonNameForm  NAME-FORM  ::=  {


NAMES


residentialPerson


WITH ATTRIBUTES
{ commonName }


AND OPTIONALLY
{ streetAddress }


ID



id-nf-resPersonNameForm }

applProcessNameForm  NAME-FORM  ::=  {


NAMES


applicationProcess


WITH ATTRIBUTES
{ commonName }


ID



id-nf-applProcessNameForm }

applEntityNameForm  NAME-FORM  ::=  {


NAMES


applicationEntity


WITH ATTRIBUTES
{ commonName }


ID



id-nf-applEntityNameForm }

dSANameForm  NAME-FORM  ::=  {


NAMES


dSA


WITH ATTRIBUTES
{ commonName }


ID



id-nf-dSANameForm }

deviceNameForm  NAME-FORM  ::=  {


NAMES


device


WITH ATTRIBUTES
{ commonName }


ID



id-nf-deviceNameForm }

dMDNameForm  NAME-FORM  ::=  {


NAMES


dMD


WITH ATTRIBUTES
{ dmdName }


ID



id-nf-dMDNameForm }

--  Object identifier assignments  --

--  object identifiers assigned in other modules are shown in comments

--  Object classes  --

--  id-oc-top   



OBJECT IDENTIFIER
::=
{id-oc 0}
Defined in ITU-T Rec. X.501 |
--












ISO/IEC 9594-2

--  id-oc-alias



OBJECT IDENTIFIER
::=
{id-oc 1}
Defined in ITU-T Rec. X.501 |
--












ISO/IEC 9594-2

id-oc-country



OBJECT IDENTIFIER
::=
{id-oc 2}

id-oc-locality



OBJECT IDENTIFIER
::=
{id-oc 3}

id-oc-organization


OBJECT IDENTIFIER
::=
{id-oc 4}

id-oc-organizationalUnit

OBJECT IDENTIFIER
::=
{id-oc 5}

id-oc-person



OBJECT IDENTIFIER
::=
{id-oc 6}

id-oc-organizationalPerson

OBJECT IDENTIFIER
::=
{id-oc 7}

id-oc-organizationalRole

OBJECT IDENTIFIER
::=
{id-oc 8}

id-oc-groupOfNames


OBJECT IDENTIFIER
::=
{id-oc 9}

id-oc-residentialPerson  

OBJECT IDENTIFIER
::=
{id-oc 10}

id-oc-applicationProcess

OBJECT IDENTIFIER
::=
{id-oc 11}

id-oc-applicationEntity


OBJECT IDENTIFIER
::=
{id-oc 12}

id-oc-dSA




OBJECT IDENTIFIER
::=
{id-oc 13}

id-oc-device



OBJECT IDENTIFIER
::=
{id-oc 14}

id-oc-strongAuthenticationUser
OBJECT IDENTIFIER
::=
{id-oc 15}
-- Deprecated, see 6.15
id-oc-certificationAuthority

OBJECT IDENTIFIER
::=
{id-oc 16}
-- Deprecated, see 6.17
id-oc-certificationAuthority-V2
OBJECT IDENTIFIER
::=
{id-oc 16 2}
-- Deprecated, see 6.18
id-oc-groupOfUniqueNames

OBJECT IDENTIFIER
::=
{id-oc 17}

id-oc-userSecurityInformation
OBJECT IDENTIFIER
::=
{id-oc 18}

-- id-oc-cRLDistributionPoint

OBJECT IDENTIFIER
::=
{id-oc 19} 
Defined in ITU-T Rec. X.509 |
--












ISO/IEC 9594-8

id-oc-dmd    



OBJECT IDENTIFIER
::=
{id-oc 20}

-- id-oc-pkiUser



OBJECT IDENTIFIER
::=
{id-oc 21}
Defined in ITU-T Rec. X.509 |
--












ISO/IEC 9594-8

-- id-oc-pkiCA



OBJECT IDENTIFIER
::=
{id-oc 22}
Defined in ITU-T Rec. X.509 |
--












ISO/IEC 9594-8

-- id-oc-deltaCRL



OBJECT IDENTIFIER
::=
{id-oc 23}
Defined in ITU-T Rec. X.509 |
--












ISO/IEC 9594-8

-- id-oc-pmiUser



OBJECT IDENTIFIER
::=
{id-oc 24} 
Defined in ITU-T Rec. X.509 |
--












ISO/IEC 9594-8

-- id-oc-pmiAA



OBJECT IDENTIFIER
::=
{id-oc 25} 
Defined in ITU-T Rec. X.509 |
--












ISO/IEC 9594-8

-- id-oc-pmiSOA



OBJECT IDENTIFIER
::=
{id-oc 26}
Defined in ITU-T Rec. X.509 |
--












ISO/IEC 9594-8

-- id-oc-attCertCRLDistributionPts
OBJECT IDENTIFIER
::=
{id-oc 27}
Defined in ITU-T Rec. X.509 |
--












ISO/IEC 9594-8

-- id-oc-parent



OBJECT IDENTIFIER
::=
{id-oc 28}
Defined in ITU-T Rec. X.501 |
--












ISO/IEC 9594-2

-- id-oc-child  



OBJECT IDENTIFIER
::=
{id-oc 29}
Defined in ITU-T Rec. X.501 |
--












ISO/IEC 9594-2

-- id-oc-cpCps



OBJECT IDENTIFIER
::=
{id-oc 30}
Defined in ITU-T Rec. X.509 |
--












ISO/IEC 9594-8

-- id-oc-pkiCertPath


OBJECT IDENTIFIER
::=
{id-oc 31}
Defined in ITU-T Rec. X.509 |
--












ISO/IEC 9594-8

-- id-oc-privilegePolicy


OBJECT IDENTIFIER
::=
{id-oc 32}
Defined in ITU-T Rec. X.509 |
--












ISO/IEC 9594-8

-- id-oc-pmiDelegationPath

OBJECT IDENTIFIER
::=
{id-oc 33}
Defined in ITU-T Rec. X.509 |
--












ISO/IEC 9594-8

--  Name forms --

id-nf-countryNameForm

OBJECT IDENTIFIER
::=
{id-nf 0}

id-nf-locNameForm


OBJECT IDENTIFIER
::=
{id-nf 1}

id-nf-sOPNameForm


OBJECT IDENTIFIER
::=
{id-nf 2}

id-nf-orgNameForm


OBJECT IDENTIFIER
::=
{id-nf 3}

id-nf-orgUnitNameForm  

OBJECT IDENTIFIER
::=
{id-nf 4}

id-nf-personNameForm  

OBJECT IDENTIFIER
::=
{id-nf 5}

id-nf-orgPersonNameForm

OBJECT IDENTIFIER
::=
{id-nf 6}

id-nf-orgRoleNameForm

OBJECT IDENTIFIER
::=
{id-nf 7}

id-nf-gONNameForm


OBJECT IDENTIFIER
::=
{id-nf 8}

id-nf-resPersonNameForm

OBJECT IDENTIFIER
::=
{id-nf 9}

id-nf-applProcessNameForm   
OBJECT IDENTIFIER
::=
{id-nf 10}

id-nf-applEntityNameForm

OBJECT IDENTIFIER
::=
{id-nf 11}

id-nf-dSANameForm


OBJECT IDENTIFIER
::=
{id-nf 12}

id-nf-deviceNameForm   

OBJECT IDENTIFIER
::=
{id-nf 13}

-- id-nf-cRLDistPtNameForm

OBJECT IDENTIFIER 
::=
{id-nf 14}

id-nf-dMDNameForm


OBJECT IDENTIFIER 
::=
{id-nf 15}

-- id-nf-subentryNameForm

OBJECT IDENTIFIER 
::=
{id-nf 16}

END  -- SelectedObjectClasses
Annex  B

Suggested name forms and DIT structures

(This annex does not form an integral part of this Recommendation | International Standard)

This annex suggests a DIT structure shown in Figure B.1 and related DIT structure rules using the name forms defined in clause 3. The rules cover an unconstrained DIT structure.

The integer identifiers assigned in this annex and used in Figure B.1 are arbitrary and have no global (or standardized) significance. A particular structure rule identifier only has significance within the scope of the subschema in which it applied. Each DMD is responsible for creating its own DIT structure and structure rules that may differ from this example.

[image: image2.wmf]TISO4030-94

1

2

4

6

8

7

20

12

17

11

13

15

14

23

16

22

25

24

19

10

5

3

9

18

21

Root

Country

Organization

Residential

person

Group of

names

Device

Organizational

person

Organizational

role

Application

entity

Organizational

unit

Locality

Application

process

Object

Class

From

immediate

superior

to immediate

subordinate


Figure B.1 – Suggested DIT structure

B.1
Country

Attribute countryName is used for naming.

The root is the immediate superior to entries of object class country.

sr1  STRUCTURE-RULE  ::=  {


NAME FORM
countryNameForm


ID


1 }

B.2
Organization

Attribute organizationName is used for naming.

The root, country or locality can be the immediate superior of entries of object class organization.

NOTE – When the organization is directly under the root, this denotes an international organization. The naming values of the organizationName attribute for international organizations must all be distinct.

sr2  STRUCTURE-RULE  ::=  {


NAME FORM

orgNameForm


ID



2 }

sr3  STRUCTURE-RULE  ::=  {


NAME FORM

orgNameForm


SUPERIOR RULES
{ sr1 }


ID



3 }

sr4  STRUCTURE-RULE  ::=  {


NAME FORM

orgNameForm


SUPERIOR RULES
{ sr5 | sr6 | sr7 | sr8 | sr9 }


ID



4 }

B.3
Locality

Attribute localityName or stateOrProvinceName is used for naming.

NOTE – For naming locality using stateOrProvinceName, see B.12.

The root, country, locality, organization or organizationalUnit can be the immediate superior of entries of object class locality.

sr5  STRUCTURE-RULE  ::=  {


NAME FORM

locNameForm


ID



5 }

sr6  STRUCTURE-RULE  ::=  {


NAME FORM

locNameForm


SUPERIOR RULES
{ sr1 }


ID



6 }

sr7  STRUCTURE-RULE  ::=  {


NAME FORM

locNameForm


SUPERIOR RULES
{ sr5 | sr6 | sr7 | sr8 | sr9 }


ID



7 }

sr8  STRUCTURE-RULE  ::=  {


NAME FORM

locNameForm


SUPERIOR RULES
{ sr2 | sr3 | sr4 }


ID



8 }

sr9  STRUCTURE-RULE  ::=  {


NAME FORM

locNameForm


SUPERIOR RULES
{ sr10 | sr11 | sr12 }


ID



9 }

B.4
Organizational Unit

Attribute organizationalUnitName is used for naming.

organization, organizationalUnit or locality can be the immediate superior of entries of object class organizationalUnit.

sr10  STRUCTURE-RULE  ::=  {


NAME FORM

orgUnitNameForm


SUPERIOR RULES
{ sr2 | sr3 | sr4 }


ID



10 }

sr11  STRUCTURE-RULE  ::=  {


NAME FORM

orgUnitNameForm


SUPERIOR RULES
{ sr10 | sr11 | sr12 }


ID



11 }

sr12  STRUCTURE-RULE  ::=  {


NAME FORM

orgUnitNameForm


SUPERIOR RULES
{ sr5 | sr6 | sr7 | sr8 | sr9 }


ID



12 }

B.5
Organizational Person

Attribute commonName and optionally organizationalUnitName is used for naming.

organization or organizationalUnit can be the immediate superior of entries of object class organizationalPerson.

sr13  STRUCTURE-RULE  ::=  {


NAME FORM

orgPersonNameForm


SUPERIOR RULES
{ sr2 | sr3 | sr4 }


ID



13 }

sr14  STRUCTURE-RULE  ::=  {


NAME FORM

orgPersonNameForm


SUPERIOR RULES
{ sr10 | sr11 | sr12 }


ID



14 }

B.6
Organizational Role

Attribute commonName is used for naming.

organization or organizationalUnit can be the immediate superior of entries of object class organizationalRole.

sr15  STRUCTURE-RULE  ::=  {


NAME FORM

orgRoleNameForm


SUPERIOR RULES
{ sr2 | sr3 | sr4 }


ID



15 }

sr16  STRUCTURE-RULE  ::=  {


NAME FORM

orgRoleNameForm


SUPERIOR RULES
{ sr10 | sr11 | sr12 }


ID



16 }

B.7
Group of Names

Attribute commonName is used for naming.

locality, organization or organizationalUnit can be the immediate superior of entries of object class groupOfNames.

sr17  STRUCTURE-RULE  ::=  {


NAME FORM

gonNameForm


SUPERIOR RULES
{ sr5 | sr6 | sr7 | sr8 | sr9 }


ID



17 }

sr18  STRUCTURE-RULE  ::=  {


NAME FORM

gonNameForm


SUPERIOR RULES
{ sr2 | sr3 | sr4 }


ID



18 }

sr19  STRUCTURE-RULE  ::=  {


NAME FORM

gonNameForm


SUPERIOR RULES
{ sr10 | sr11 | sr12 }


ID



19 }

B.8
Residential Person

Attribute commonName and optionally streetAddress is used for naming.

locality is the immediate superior of entries of object class residentialPerson.

sr20  STRUCTURE-RULE  ::=  {


NAME FORM

resPersonNameForm


SUPERIOR RULES
{ sr5 | sr6 | sr7 | sr8 | sr9 }


ID



20 }

B.9
Application Entity

Attribute commonName is used for naming. 

applicationProcess is the immediate superior of entries of object class applicationEntity.

sr21  STRUCTURE-RULE  ::=  {


NAME FORM

applEntityNameForm


SUPERIOR RULES
{ sr24 | sr25 }


ID



21 }

B.10
Device

Attribute commonName is used for naming.

organization or organizationalUnit can be the immediate superior of entries of object class device.

sr22  STRUCTURE-RULE   ::=  {


NAME FORM

deviceNameForm


SUPERIOR RULES
{ sr2 | sr3 | sr4 }


ID



22 }

sr23
STRUCTURE-RULE
::= {


NAME FORM
deviceNameForm


SUPERIOR RULES
{ sr10 | sr11 | sr12 }


ID



23 }

B.11
Application Process

Attribute commonName is used for naming. 

organization or organizationalUnit can be the immediate superior of entries of object class applicationProcess.

sr24  STRUCTURE-RULE  ::=  {


NAME FORM

applProcessNameForm


SUPERIOR RULES
{ sr2 | sr3 | sr4 }


ID



24 }

sr25  STRUCTURE-RULE  ::=  {


NAME FORM
applProcessNameForm


SUPERIOR RULES
{ sr10 | sr11 | sr12 }


ID



25 }

B.12
Alternative Structure Rule for Locality

If the stateOrProvinceName attribute is used for naming locality and locality constrained to existing only as an immediate subordinate of country, then one additional structure rule is required to define this.

sr26  STRUCTURE-RULE  ::=  {


NAME FORM

sOPNameForm


SUPERIOR RULES
{ sr1 }


ID



26 }

In addition the structure rules sr4, sr7, sr12, sr17, and sr20 must be modified to include sr26 within their respective list of superior structure rule as follows.

sr4  STRUCTURE-RULE  ::=  {


NAME FORM

orgNameForm


SUPERIOR RULES
{ sr5 | sr6 | sr7 | sr8 | sr9 | sr26 }


ID



4 }

sr7  STRUCTURE-RULE  ::=  {


NAME FORM
locNameForm


SUPERIOR RULES
{ sr5 | sr6 | sr7 | sr8 | sr9 | sr26 }


ID



7 }

sr12  STRUCTURE-RULE  ::=  {


NAME FORM

orgUnitNameForm


SUPERIOR RULES
{ sr5 | sr6 | sr7 | sr8 | sr9 | sr26 }


ID



12 }

sr17  STRUCTURE-RULE  ::=  {


NAME FORM

gonNameForm


SUPERIOR RULES
{ sr5 | sr6 | sr7 | sr8 | sr9 | sr26 }


ID



17 }

sr20  STRUCTURE-RULE  ::=  {


NAME FORM

resPersonNameForm


SUPERIOR RULES
{ sr5 | sr6 | sr7 | sr8 | sr9 | sr26 }


ID



20 }

Annex  C

Amendments and corrigenda

(This annex does not form an integral part of this Recommendation | International Standard)

This edition of this Directory Specification includes the following amendment:

–
Amendment 1: Enhancements for the support of ITU-T Rec. F.510 and for certificate extensions.

This edition of this Directory Specification includes the technical corrigendum that corrects the following Defect Report: 239.

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	


�PAGE \# "'Page: '#'�'"  �Page: 2���A: Directory Information Tree (DIT


�PAGE \# "'Page: '#'�'"  �Page: 2���A: Directory System Agent (DSA)





