
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	X.770

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(01/2001)   

	
	SERIES X: DATA NETWORKS AND OPEN SYSTEM COMMUNICATIONS

OSI management – Management functions and ODMA functions



	
	ODMA notification dispatch function

	
	ITU‑T  Recommendation  X.770

(Formerly  CCITT  Recommendation)


ITU-T X-SERIES  RECOMMENDATIONS

DATA NETWORKS AND OPEN SYSTEM COMMUNICATIONS

	
	

	PUBLIC DATA NETWORKS
	

	Services and facilities
	X.1–X.19

	Interfaces
	X.20–X.49

	Transmission, signalling and switching
	X.50–X.89

	Network aspects
	X.90–X.149

	Maintenance
	X.150–X.179

	Administrative arrangements
	X.180–X.199

	OPEN SYSTEMS INTERCONNECTION
	

	Model and notation
	X.200–X.209

	Service definitions
	X.210–X.219

	Connection-mode protocol specifications
	X.220–X.229

	Connectionless-mode protocol specifications
	X.230–X.239

	PICS proformas
	X.240–X.259

	Protocol Identification
	X.260–X.269

	Security Protocols
	X.270–X.279

	Layer Managed Objects
	X.280–X.289

	Conformance testing
	X.290–X.299

	INTERWORKING BETWEEN NETWORKS
	

	General
	X.300–X.349

	Satellite data transmission systems
	X.350–X.369

	IP-based networks
	X.370–X.399

	MESSAGE HANDLING SYSTEMS
	X.400–X.499

	DIRECTORY
	X.500–X.599

	OSI NETWORKING AND SYSTEM ASPECTS
	

	Networking
	X.600–X.629

	Efficiency
	X.630–X.639

	Quality of service
	X.640–X.649

	Naming, Addressing and Registration
	X.650–X.679

	Abstract Syntax Notation One (ASN.1)
	X.680–X.699

	OSI MANAGEMENT
	

	Systems Management framework and architecture
	X.700–X.709

	Management Communication Service and Protocol
	X.710–X.719

	Structure of Management Information
	X.720–X.729

	Management functions and ODMA functions
	X.730–X.799

	SECURITY
	X.800–X.849

	OSI APPLICATIONS
	

	Commitment, Concurrency and Recovery
	X.850–X.859

	Transaction processing
	X.860–X.879

	Remote operations
	X.880–X.899

	OPEN DISTRIBUTED PROCESSING
	X.900–X.999

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation X.770

ODMA notification dispatch function




	Summary

This Recommendation specifies a set of UML descriptions of the OMG notification service, thus placing it within the ODMA architecture (specified in ITU‑T X.703) as the ODMA Notification Dispatch Function.


	Source

ITU‑T Recommendation X.770 was prepared by ITU‑T Study Group 4 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 19 January 2001.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS


Page

1
Scope

1
2
References

1
3
Definitions

1
3.1
Definitions from notification service

1
3.2
Definitions from ITU‑T X.703

1
3.3
Definitions from UML

2
4
Abbreviations and conventions

2
5
Enterprise viewpoint UML use-cases for OMG notification service

2
5.1
Use‑case overview

2
5.2
Notification delivery use case diagram

3
5.3
Notification posting use case diagram

4
5.4
Filter administration use case diagram

4
5.5
Consumer administration use case diagram

4
5.6
Supplier administration use case

5
6
Information viewpoint model for OMG notification service

5
7
Computational interfaces for notification service

8
7.1
CosEventComm module

9
7.2
CosEventChannelAdmin module

9
7.3
CosTypedEventComm module

10
7.4
CosTypedEventChannelAdmin module

10
7.5
CosNotifyComm module

12
7.6
CosNotifyChannelAdmin module

14
7.7
CosNotification module

19
7.8
CosNotifyFilter module

20
7.9
CosTypedNotifyComm module

21
7.10
CosTypedNotifyChannelAdmin module

23
8
Engineering interface definitions

27
ITU-T Recommendation X.770

ODMA notification dispatch function

1
Scope

This ODMA Notification Dispatch Function is part of the Open Distributed Management Architecture, as specified in ITU‑T X.703. This Recommendation references the specification of the OMG notification service for the normative specification of the ODP IDL (ITU‑T X.950) engineering viewpoint interface definitions.

This Recommendation provides a UML based set of enterprise, information, and computational viewpoint language description of the OMG notification service, thus placing it within ODMA as the Notification Dispatch Function.

2
References

The following ITU‑T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU‑T Recommendations is regularly published.

(
ITU‑T X.703 (1997) | ISO/IEC 13244:1998, Information technology ( Open Distributed Management Architecture.

(
OMG Document formal/2000‑06‑20, Notification service specification.

(
OMG Document formal/1999‑06‑01, OMG Modeling Book.

3
Definitions

3.1
Definitions from notification service 

The following terms are used as defined in OMG notification service:

(
filter;

(
mapping filter;

(
structured notifications (content having a fixed header and a flexible body with names for each value element);

(
typed notifications (content conforming to a statically defined syntax);

(
untyped notifications (content determined at time of delivery).

3.2
Definitions from ITU‑T X.703

The following terms are used as defined in ITU‑T X.703:

(
enterprise viewpoint;

(
information viewpoint;

(
computational viewpoint;

(
engineering viewpoint;

(
notification.

3.3
Definitions from UML

The following terms are used as defined in UML:

(
association;

(
association class;

(
class diagram;

(
include;

(
use case diagram.

4
Abbreviations and conventions

The following abbreviations are used in this Recommendation:

CEC

Cos Event Comm

CN

Cos Notification

CNC

Cos Notification Comm

CNCA

Cos Notify Channel Admin

CNF

Cos Notify Filter

CTEC

Cos Typed Event Comm

The diagrams in this Recommendation use the Notation specified in the OMG UML version 1.3.

5
Enterprise viewpoint UML use-cases for OMG notification service

The actors are shown only at the first level of the use case diagram. Where the detailed decompositions are described for the use cases, the actors are not explicitly shown for readability. 

5.1
Use‑case overview

See Figure 5‑1.


[image: image2.wmf]T0414440-00

channel

administration

consumer

administration

notification

delivery

administrator

consumer

supplier

administration

notification

posting

filter

administration

supplier


Figure 5-1/X.770 ( High-level use case diagram for notification service

In this figure, all the use cases correspond to the various functions that are part of the notification service. These use cases include both administrative activities such as consumer administration, and the activities for dispatching the notifications.

An administrator is responsible for administering the overall properties of notification channels, which are used to receive notifications from suppliers and deliver the received notifications to consumers that have subscribed to the notifications.

Consumers and administrators are involved in the consumer specific administration for notification channels.

Suppliers and administrators are involved in the supplier specific administration for notification channels.

Administrators, suppliers and consumers are involved in the administration of filters, which control the receipt and delivery of notifications.

The supplier is involved in the posting of notifications. The supplier may not know the ultimate consumers of the notifications it supplies.

The consumer receives the delivered notifications. The consumer may not know the originating supplier of the notifications it consumes.

5.2
Notification delivery use case diagram

See Figure 5‑2.


[image: image3.wmf]T0414450-00

push

delivery

notification

delivery

pull

delivery

<<include>>

<<include>>


Figure 5-2/X.770 – Push and pull as specialization of notification delivery

Notifications may be delivered using either a push model (notification service invokes an operation on consumer in order to deliver a notification to the consumer) or a pull model (consumer invokes an operation on the notification service in order to have its notifications delivered to itself).

5.3
Notification posting use case diagram

See Figure 5‑3.


[image: image4.wmf]T0414460-00

pull

posting

notification

posting

<<include>>

<<include>>

push

posting


Figure 5-3/X.770 – Push and pull as specialization for notification posting

Notifications may be posted using either a push model (supplier invokes an operation on notification service in order to post a notification) or a pull model (notification service invokes an operation on the supplier to receive posted notifications from the supplier).

5.4
Filter administration use case diagram

See Figure 5‑4.


[image: image5.wmf]T0414470-00

create

filter

filter

administration

<<include>>

<<include>>

modify

filter


Figure 5-4/X.770 – Filter administration includes creating and modifying filters

There is a need for mechanisms to both create and modify the logical constructs of filters, which control the delivery of notification instances based on the actual values of the parameters of the notification, (e.g. do not deliver notifications which have a error count of zero).

5.5
Consumer administration use case diagram

Figure 5‑5 shows how to use the proxy administration corresponding to the consumer. The administration of a consumer proxy includes the following functions: create a proxy, define the filter (associating filter) to determine what notifications are to be received, and attach or detach to/from the proxy. The notify subscribe use case supports the function for the consumer to subscribe to notifications resulting from changes to the notification types published by the supplier.


[image: image6.wmf]T0414480-00

consumer proxy

administration

obtain

administrator

consumer

notify

subscribe

attach

consumer

administration

associate

filter

detach

<<include>>

<<include>>

<<include>>

<<include>>

<<include>>

<<include>>


Figure 5-5/X.770 – Consumer administration

5.6
Supplier administration use case

Figure 5‑6 shows how to use the proxy administration corresponding to the supplier. The administration of a supplier proxy includes the following functions: create a proxy, define the filter (associating filter) to determine what notifications are to be forwarded and attach or detach to/from the proxy. The notify publish use case supports the function for the supplier to publish changes (addition or deletion) to notification types it supplies.


[image: image7.wmf]T0414490-00

supplier proxy

administration

obtain

administrator

supplier

notify

publish

attach

supplier

administration

associate

filter

detach

<<include>>

<<include>>

<<include>>

<<include>>

<<include>>

<<include>>


Figure 5-6/X.770 – Supplier administration

6
Information viewpoint model for OMG notification service

This clause shows information object classes and their relationships. Attributes for information object classes are not shown in this clause, for purposes of simplicity of exposition. Not all classes shown in this viewpoint correspond to IDL interfaces for the notification service. See Figure 6‑1.


[image: image8.wmf]T0414500-00

filter

simpleFilter

mappingFilter


Figure 6-1/X.770 – Filter types

A filter has a logical construct, made up from assertions on the values of the various parameters of a notification instance along with logical operators, (i.e. and, or, not). The filter evaluates to either true or false.

A simple filter is used to determine delivery of a notification either into or out of a channel. If the filter construct evaluates to true the notification instance is delivered.

A mapping filter is used to associate a particular quality of service value with a notification instance. If the filter construct is true, the associated QOS value is assigned to that notification instance. See Figure 6‑2.


[image: image9.wmf]T0414510-00

notificationDeliveryType

untypedNotificationType

typedNotificationType

structureNotificationType

notification

deliveredAs

1...3

+instance

+type

0..*


Figure 6-2/X.770 – Notification delivery types

Each notification instance is delivered using a notification delivery type, which may be either an untyped notification type (encoded as a single value of type "any"), a structure notification type (encoded as a structure containing a fixed header followed by elements which are name value pairs), or a typed notification type (encoded as a strongly typed operation). Each notification instance (i.e. the specification of the semantic contents of the notification) may be delivered using one or more of these notification delivery types. 

The notification service provider needs to provide translation services allowing consumers of any notification delivery type to receive notifications posted by providers using any of the three notification delivery types. See Figure 6‑3.


[image: image10.wmf]T0414520-00

0..*

1..1

1..1

0..*

0..*

0..*

0..1

0..1

proxyConsumer

CosEventChannel

notificationChannel

proxySupplier

Consumer

Supplier


Figure 6-3/X.770 – Channel types

A notification channel is a specialization of a COS event channel and, as such, has zero or more proxy consumers and/or proxy suppliers.  Each proxy consumer is associated with at most one supplier. Each proxy supplier is associated with at most one consumer. Each supplier may be associated with zero or more proxy consumers, while each consumer may be associated with zero or more proxy suppliers.

There are three subtypes of proxy consumers, those which consume untyped notifications, those which consume typed notifications, and those which consume structured notifications. See Figure 6‑4.


[image: image11.wmf]T0414530-00

proxyConsumer

typedProxyConsumer

untypedProxyConsumer

structuredProxyConsumer


Figure 6-4/X.770 – Proxy consumers

There are three subtypes of proxy supplier, those which supply untyped notifications, those which supply typed notifications, and those which supply structured notification types. See Figure 6‑5.


[image: image12.wmf]T0414540-00

proxySupplier

typedProxySupplier

untypedProxySupplier

structuredProxySupplier


Figure 6-5/X.770 – Proxy suppliers

7
Computational interfaces for notification service

The detailed specifications of the computational interfaces described in this clause are contained in the OMG Notification service specification (OMG Document telecom/1999-07-01). See Figure 7‑1.


[image: image13.wmf]T0414550-00

Event Service

Notification Service

<<CORBAModule>>

CosTypedEvent

Comm

<<CORBAModule>>

CosEventComm

<<CORBAModule>>

CosTypedEvent

ChannelAdmin

<<CORBAModule>>

CosEventChannel

Admin

<<CORBAModule>>

CosTypedNotify

Comm

<<CORBAModule>>

CosNotifyComm

<<CORBAModule>>

CosNotifyFilter

<<CORBAModule>>

CosNotification

<<CORBAModule>>

CosTypedNotify

ChannelAdmin

<<CORBAModule>>

CosNotifyChannel

Admin


Figure 7-1/X.770 – Module include dependencies for OMG notification service

The dashed lines in the figure show how each module in the notification service includes other modules either defined in notification service or COS event service.

7.1
CosEventComm module

See Figure 7‑2.

	PushConsumer

	

	push(data : in any) : void
disconnect_push_consumer() : void


	PushSupplier

	

	disconnect_push_supplier() : void


	PullConsumer

	

	disconnect_pull_consumer() : void


	PullSupplier

	

	pull() : any
try_pull(has_event : out boolean) : any disconnect_pull_supplier() : void


Figure 7-2/X.770 – CosEventComm module interfaces

7.2
CosEventChannelAdmin module

See Figure 7‑3.

	ProxyPushConsumer

	

	connect_push_supplier(push_supplier : in CosEventComm::PushSupplier) : void

	
	
	

	ProxyPushSupplier

	

	connect_push_consumer(push_consumer : in CosEventComm::PushConsumer) : void

	
	
	

	ProxyPullConsumer

	

	connect_pull_supplier(pull_supplier : in CosEventComm::PullSupplier) : void

	
	
	

	ProxyPullSupplier

	

	connect_pull_consumer(pull_consumer : in CosEventComm::PullConsumer) : void


	ConsumerAdmin
	
	SupplierAdmin

	
	
	

	obtain_push_supplier() : ProxyPushSupplier
obtain_pull_supplier() : ProxyPullSupplier
	
	obtain_push_consumer() : ProxyPushConsumer
obtain_pull_consumer() : ProxyPullConsumer


	EventChannel

	

	for_consumers() : ConsumerAdmin
for_suppliers() : SupplierAdmin
destroy() : void


Figure 7-3/X.770 – CosEventChannelAdmin module interfaces

7.3
CosTypedEventComm module

See Figure 7‑4.


[image: image14.wmf]T0414560-00

TypedPushConsumer

get_typed_consumer() : Object

TypedPullSupplier

get_typed_supplier() : Object

PushConsumer

(from CosEventComm)

PullSupplier

(from CosEventComm)

push (data : in any) : void

disconnect_push_consumer() : void

pull() : any

try_pull(has_event : out boolean) : any

disconnect_pull_supplier() : void


Figure 7-4/X.770 – CosTypedEventComm module interfaces

7.4
CosTypedEventChannelAdmin module

See Figures 7‑5 to 7‑9.


[image: image15.wmf]T0414570-00

(from CosEventComm)

(from CosEventChannelAdmin)

(from CosTypedEventComm)

TypedProxyPushConsumer

ProxyPushConsumer

connect_push_supplier(push_supplier : in CEC::PushSupplier) : void

TypedPushConsumer

get_typed_consumer() : Object

PushConsumer

push (data : in any) : void

disconnect_push_consumer() : void


Figure 7-5/X.770 – TypedProxyPushConsumer interface


[image: image16.wmf]T0414580-00

(from CosEventComm)

(from CosEventChannelAdmin)

(from CosTypedEventComm)

TypedProxyPullSupplier

ProxyPullSupplier

connect_pull_consumer(pull_consumer : in CEC::PullConsumer) : void

TypedPullSupplier

get_typed_supplier() : Object

PullSupplier

pull() : any

try_pull(has_event : out boolean) : any

disconnect_pull_supplier() : void


Figure 7-6/X.770 – TypedProxyPullSupplier interfaces


[image: image17.wmf]T0414590-00

TypedSupplierAdmin

SupplierAdmin

(from CosEventChannelAdmin)

obtain_push_consumer() : ProxyPushConsumer

obtain_pull_consumer() : ProxyPullConsumer

obtain_typed_push_consumer(supported_interface : in Key) : TypedProxyPushConsumer

obtain_typed_pull_consumer(uses_interface : in Key) : ProxyPullConsumer


Figure 7-7/X.770 – TypedSupplierAdmin Interface


[image: image18.wmf]T0414600-00

TypedConsumerAdmin

ConsumerAdmin

(from CosEventChannelAdmin)

obtain_push_supplier() : ProxyPushSupplier

obtain_pull_supplier() : ProxyPullSupplier

obtain_typed_pull_supplier(supported_interface : in Key) : TypedProxyPullSupplier

obtain_typed_push_supplier(uses_interface : in Key) : ProxyPushSupplier


Figure 7-8/X.770 – TypedConsumerAdmin interface

	TypedEventChannel

	

	for_consumers() : TypedConsumerAdmin
for_suppliers() : TypedSupplierAdmin
destroy() : void


Figure 7-9/X.770 – TypedEventChannel interface

7.5
CosNotifyComm module

See Figures 7‑10 and 7‑11.


[image: image19.wmf]T

0414610-00

NotifyPublish

offer_change(added : in CN::EventTypeSeq, removed : in CN::EventTypeSeq) : void

PushConsumer

SequencePushConsumer

StructuredPushConsumer

PullConsumer

StructuredPullConsumer

disconnect_structured_pull_consumer() : void

SequencePullConsumer

disconnect_sequence_pull_consumer() : void

PushConsumer

(from CosEventComm)

PullConsumer

disconnect_pull_consumer() : void

(from CosEventComm)

push(data : in any) : void

disconnect_push_consumer() : void

push_structured_event(notification : in CosNotification::StructuredEvent) : void

disconnect_structured_push_consumer() : void

push_structured_events(notifications : in CosNotification::EventBatch) : void

disconnect_sequence_push_consumer() : void


Figure 7-10/X.770 – Structured and SequenceConsumer interfaces


[image: image20.wmf]T0414620-00

NotifySubscribe

subscription_change(added : in CN::EventTypeSeq, removed : in CN::EventTypeSeq) : void

PullSupplier

StructuredPullSupplier

SequencePullSupplier

PushSupplier

StructuredPushSupplier

disconnect_structured_push_supplier() : void

SequencePushSupplier

disconnect_sequence_push_supplier() : void

PushSupplier

disconnect_push_supplier() : void

(from CosEventComm)

PullSupplier

(from CosEventComm)

pull_structured_event() : CosNotification::StructuredEvent

try_pull_structured_event(has_event : out boolean) : CosNotification::StructuredEvent

disconnect_structured_pull_supplier() : void

pull_structured_events(max_number : in long) : CosNotification::EventBatch

try_pull_structured_events(max_number : in long, has_event : out boolean) : CosNotification::EventBatch

disconnect_sequence_pull_supplier() : void

pull() : any

try_pull(has_event : out boolean) : any

disconnect_pull_supplier() : void


Figure 7-11/X.770 – Structured and Sequence Supplier interfaces

7.6
CosNotifyChannelAdmin module

See Figures 7‑12 to 7‑17.


[image: image21.wmf]T0414630-00

StructuredProxyPushConsumer

connect_structured_push_supplier(push_supplier : in CNC::StructuredPushSupplier) : void

ProxyPushConsumer

connect_any_push_supplier(push_supplier : in CEC::PushSupplier) : void

StructuredProxyPullConsumer

ProxyPullConsumer

ProxyConsumer

QoSAdmin

(from Cos)

FilterAdmin

(from CosNotifyFilter)

StructuredPullConsumer

disconnect_structured_pull_consumer()

(from CosNotifyComm)

PullConsumer

StructuredPushConsumer

(from CosNotifyComm)

PushConsumer

(from CosNotify)

CNC   CosNotificationComm

CN       CosNotification

CEC     CosEventComm

get_qos()

set_qos()

validate_qos()

add_filter()

remove_filter()

get_filter()

get_all_filters()

remove_all_filters()

<<readonly>> MyType : ProxyType

<<readonly>> MyAdmin : SupplierAdmin

obtain_subscription_types(mode : in ObtainInfoMode) : CN::EventTypeSeq

validate_event_qos(required_qos : in CN::QoSProperties, available_qos : out CN::NamedPropertyRangeSeq) : void

push_structured_event()

disconnect_structured_push_consumer()

connect_structured_pull_supplier(pull_supplier : in CNC::StructuredPullSupplier) : void

suspend_connection() : void

resume_connection() : void

connect_any_pull_supplier(pull_supplier : in CEC::PullSupplier) : void

suspend_connection() : void

resume_connection() : void

(from CosNotify)


Figure 7-12/X.770 – StructuredProxy and Proxy Consumer interfaces


[image: image22.wmf]T0414640-00

StructuredProxyPushSupplier

ProxyPushSupplier

ProxyPullSupplier

connect_any_pull_consumer(pull_consumer : in CEC::PullConsumer) : void

StructuredProxyPullSupplier

connect_structured_pull_consumer(pull_consumer : in CNC::StructuredPullConsumer) : void

ProxySupplier

QoSAdmin

PushSupplier

(from CosNotify)

PullSupplier

(from CosNotify)

StructuredPushSupplier

disconnect_structured_push_supplier()

(from CosNotifyComm)

FilterAdmin

(from CosNotifyFilter)

StructuredPullSupplier

get_qos()

set_qos()

validate_qos()

add_filter()

remove_filter()

get_filter()

get_all_filters()

remove_all_filters()

<<readonly>> MyType : ProxyType

<<readonly>> MyAdmin : ConsumerAdmin

priority_filter : CNF::MappingFilter

lifetime_filter : CNF::MappingFilter

obtain_offered_types(mode : in ObtainInfoMode) : CN::EventTypeSeq

validate_event_qos(required_qos : in CN::QoSProperties, available_qos : out CN::NamedPropertyRangeSeq) : void

connect_structured_push_consumer(push_consumer : in CNC::StructuredPushConsumer) : void

suspend_connection() : void

resume_connection() : void

pull_structured_event()

try_pull_structured_event()

disconnect_structured_pull_supplier()

connect_any_push_consumer(push_consumer : in CEC::PushConsumer) : void

suspend_connection() : void

resume_connection() : void

(from Cos)

CNC   CosNotificationComm

CN       CosNotification

CEC     CosEventComm


Figure 7-13/X.770 – StructuredProxy and Proxy Supplier interfaces


[image: image23.wmf]T0414650-00

SupplierAdmin

QoSAdmin

(from Cos)

FilterAdmin

(from CosNotifyFilter)

NotifySubscribe

subscription_change()

(from CosNotifyComm)

ConsumerAdmin

(from CosEventChannelAdmin)

SupplierAdmin

(from CosEventChannelAdmin)

obtain_push_supplier() : ProxyPushSupplier

obtain_pull_supplier() : ProxyPullSupplier

<<readonly>> MyID : AdminID

<<readonly>> MyChannel : EventChannel

<<readonly>> MyOperator : InterFilterGroupOperator

priority_filter : CNF::MappingFilter

lifetime_filter : CNF::MappingFilter

<<readonly>> pull_suppliers : ProxyIDSeq

<<readonly>> push_suppliers : ProxyIDSeq

add_filter()

remove_filter()

get_filter()

get_all_filters()

remove_all_filters()

get_qos()

set_qos()

validate_qos()

<<readonly>> MyID : AdminID

<<readonly>> MyChannel : EventChannel

<<readonly>> MyOperator : InterFilterGroupOperator

<<readonly>> pull_consumers : ProxyIDSeq

<<readonly>> push_consumers : ProxyIDSeq

obtain_push_consumer() : ProxyPushConsumer

obtain_pull_consumer() : ProxyPullConsumer

ConsumerAdmin

get_proxy_supplier(proxy_id : in ProxyID) : ProxySupplier

obtain_notification_pull_supplier(ctype : in ClientType, proxy_id : out ProxyID) : ProxySupplier

obtain_notification_push_supplier(ctype : in ClientType, proxy_id : out ProxyID) : ProxySupplier

destroy() : void

get_proxy_consumer(proxy_id : in ProxyID) : ProxyConsumer

obtain_notification_pull_consumer(ctype : in ClientType, proxy_id : out ProxyID) : ProxyConsumer

obtain_notification_push_consumer(ctype : in ClientType, proxy_id : out ProxyID) : ProxyConsumer

destroy() : void


Figure 7-14/X.770 – Consumer and Supplier Admin interfaces


[image: image24.wmf]T0414660-00

SequenceProxyPushConsumer

connect_sequence_push_supplier(push_supplier : in CNC::SequencePushSupplier) : void

SequenceProxyPullConsumer

SequencePullConsumer

disconnect_sequence_pull_consumer()

(from CosNotifyComm)

SequencePushConsumer

(from CosNotifyComm)

ProxyConsumer

QoSAdmin

(from CosNotification)

FilterAdmin

(from CosNotifyFilter)

get_qos()

set_qos()

validate_qos()

add_filter()

remove_filter()

get_filter()

get_all_filters()

remove_all_filters()

<<readonly>> MyType : ProxyType

<<readonly>> MyAdmin : SupplierAdmin

obtain_subscription_types(mode : in ObtainInfoMode) : CN::EventTypeSeq

validate_event_qos(required_qos : in CN::QoSProperties, available_qos : out CN::NamedPropertyRangeSeq) : void

push_structured_events()

disconnect_sequence_push_consumer()

connect_sequence_pull_supplier(pull_supplier : in CNC::SequencePullSupplier) : void

suspend_connection() : void

resume_connection() : void

CNC   CosNotificationComm

CN       CosNotification

CEC     CosEventComm


Figure 7-15/X.770 – SequenceProxy Consumer interfaces


[image: image25.wmf]T0414670-00

SequenceProxyPushSupplier

SequenceProxyPullSupplier

connect_sequence_pull_consumer(pull_consumer : in CNC::SequencePullConsumer) : void

SequencePullSupplier

(from CosNotifyComm)

SequencePushSupplier

disconnect_sequence_push_supplier()

(from CosNotifyComm)

QoSAdmin

(from CosNotification)

FilterAdmin

(from CosNotifyFilter)

get_qos()

set_qos()

validate_qos()

add_filter()

remove_filter()

get_filter()

get_all_filters()

remove_all_filters()

<<readonly>> MyType : ProxyType

<<readonly>> MyAdmin : ConsumerAdmin

priority_filter : CNF::MappingFilter

lifetime_filter : CNF::MappingFilter

obtain_offered_types(mode : in ObtainInfoMode) : CN::EventTypeSeq

validate_event_qos(required_qos : in CN::QoSProperties, available_qos : out CN::NamedPropertyRangeSeq) : void

pull_structured_events()

try_pull_structured_events()

disconnect_sequence_pull_supplier()

connect_sequence_push_consumer(push_consumer : in CNC::SequencePushConsumer) : void

suspend_connection() : void

resume_connection() : void

ProxySupplier

CNC   CosNotificationComm

CN       CosNotification

CEC     CosEventComm


Figure 7-16/X.770 – SequenceProxy supplier interfaces


[image: image26.wmf]T0414680-00

EventChannelFactory

EventChannel

AdminPropertiesAdmin

(from CosNotification)

EventChannel

(from CosEventChannelAdmin)

QoSAdmin

(from CosNotification)

get_qos() : QoSProperties

set_qos(qos : in QoSProperties) : void

validate_qos(required_qos : in QoSProperties, available_qos : out NamedPropertyRangeSeq) : void

for_consumers() : ConsumerAdmin

for_suppliers() : SupplierAdmin

destroy() : void

<<readonly>> MyFactory : EventChannelFactory

<<readonly>> default_consumer_admin : ConsumerAdmin

<<readonly>> default_supplier_admin : SupplierAdmin

<<readonly>> default_filter_factory : CNF::FilterFactory

new_for_consumers(op : in InterFilterGroupOperator, id : out AdminID) : ConsumerAdmin

new_for_suppliers(op : in InterFilterGroupOperator, id : out AdminID) : SupplierAdmin

get_consumeradmin(id : in AdminID) : ConsumerAdmin

get_supplieradmin(id : in AdminID) : SupplierAdmin

get_all_consumeradmins() : AdminIDSeq

get_all_supplieradmins() : AdminIDSeq

create_channel(initial_qos : in CN::QoSProperties, initial_admin : in CN::AdminProperties, id : out ChannelID) : EventChannel

get_all_channels() : ChannelIDSeq

get_event_channel(id : in ChannelID) : EventChannel

get_admin() : AdminProperties

set_admin(admin : in AdminProperties) : void

CNC   CosNotificationComm

CN       CosNotification

CEC     CosEventComm


Figure 7-17/X.770 – EventChannel and EventChannel Factory interfaces

7.7
CosNotification module

See Figures 7‑18 and 7‑19.

	AdminPropertiesAdmin

	

	get_admin() : AdminProperties
set_admin(admin : in AdminProperties) : void


Figure 7-18/X.770 – AdminProperties Admin interface

	QoSAdmin

	

	get_qos() : QoSProperties
set_qos(qos : in QoSProperties) : void
validate_qos(required_qos : in QoSProperties, available_qos : out NamePropertyRangeSeq) : void


Figure 7-19/X.770 – QoSAdmin interface

7.8
CosNotifyFilter module

See Figures 7‑20 to 7‑23.

	Filter

	<<readonly>> constraint_grammar : string

	add_constraints(constraint_list : in ConstraintExpSeq) : ConstraintInfoSeq
modify_constraints(del_list : in ConstraintIDSeq, modify_list : in ConstraintInfoSeq) : void
get_constraints(id_list : in ConstraintIDSeq) : ConstraintInfoSeq
get_all_constraints() : ConstraintInfoSeq
remove_all_constraints() : void
destroy() : void
match(filterable_data : in any) : boolean
match_structured(filterable_data : in CosNotification::StructuredEvent) : boolean
match_typed(filterable_data : in CosNotification::PropertySeq) : boolean
attach_callback(callback : in CosNotifyComm::NotifySubscribe) : CallbackID
detach_callback(callback : in CallbackID) : void
get_callbacks() : CallbackIDSeq


Figure 7-20/X.770 – Filter interface

	MappingFilter

	<<readonly>> constraint_grammar : string
<<readonly>> value_type : CORBA : TypeCode
<<readonly>> default_value : any

	add_mapping_constraints(pair_list : in MappingConstraintPairSeq) : MappingConstraintInfoSeq
modify_mapping_constraints(del_list : in ConstraintIDSeq, modify_list : in MappingConstraintInfoSeq) : void
get_mapping_constraints(id_list : in ConstraintIDSeq) : MappingConstraintInfoSeq
get_all_mapping_constraints() : MappingConstraintInfoSeq
remove_all_mapping_constraints() : void
destroy() : void
match(filterable_data : in any, result_to_set : out any) : boolean
match_structured(filterable_data : in CosNotification::StructuredEvent, result_to_set : out any) : boolean
match_typed(filterable_data : in CosNotification::PropertySeq, result_to_set : out any) : boolean


Figure 7-21/X.770 – Mapping Filter interface

	FilterAdmin

	

	add_filter(new_filter : in Filter) : FilterID
remove_filter(filter : in FilterID) : void
get_filter(filter : in FilterID) : Filter
get_all_filters() : FilterIDSeq
remove_all_filters() : void


Figure 7-22/X.770 – Filter Admin interface

	FilterFactory

	

	create_filter(constraint_grammar : in string) : Filter
create_mapping_filter(constraint_grammar : in string, default_value : in any) : MappingFilter


Figure 7-23/X.770 – Filter Factory interface

7.9
CosTypedNotifyComm module

See Figures 7‑24 and 7‑25.


[image: image27.wmf]T0414690-00

TypedPushConsumer

offer_change(added : in CN::EventTypeSeq, removed : in CN::EventTypeSeq) : void

(from CosNotifyComm)

TypedPushConsumer

get_typed_consumer() : Object

(from CosTypedEventComm)

PushConsumer

(from CosEventComm)

push(data : in any) : void

disconnect_push_consumer() : void

Notify Publish

CNC   CosNotificationComm

CN       CosNotification

CEC     CosEventComm


Figure 7-24/X.770 – TypedPush Consumer interface


[image: image28.wmf]T0414700-00

TypedPullSupplier

NotifySubscribe

subscription_change(added : in CN::EventTypeSeq, removed : in CN::EventTypeSeq) : void

(from CosNotifyComm)

TypedPullSupplier

get_typed_supplier() : Object

(from CosTypedEventComm)

pull() : any

try_pull(has_event : out boolean) : any

disconnect_pull_supplier() : void

PullSupplier

(from CosEventComm)

CNC   CosNotificationComm

CN       CosNotification

CEC     CosEventComm


Figure 7-25/X.770 – TypedPull Supplier interface

7.10
CosTypedNotifyChannelAdmin module

See Figures 7‑26 to 7‑31.


[image: image29.wmf]T0414710-00

TypedProxyPushConsumer

connect_typed_push_supplier(push_supplier : in CEC::PushSupplier) : void

TypedProxyPushSupplier

ProxyConsumer

TypedPushConsumer

(from CosTypedNotifyComm)

TypedPushConsumer

get_typed_consumer() : Object

(from CosTypedEventComm)

NotifyPublish

offer_change()

(from CosNotify)

PushConsumer

(from CosEventComm)

ProxySupplier

QoSAdmin

(from Cos)

FilterAdmin

(from CosNotifyFilter)

PushSupplier

(from CosNotify)

PushSupplier

disconnect_push_supplier() : void

(from CosEventComm)

NotifySubscribe

subscription_change()

(from CosNotifyComm)

push(data : in any) : void

disconnect_push_consumer() : void

<<readonly>> MyType : ProxyType

<<readonly>> MyAdmin : SupplierAdmin

obtain_subscription_types(mode : in ObtainInfoMode) : CN::EventTypeSeq

validate_event_qos(required_qos : in CN::QoSProperties, available_qos : out CN::NamedPropertyRangeSeq) : void

get_qos()

set_qos()

validate_qos()

add_filter()

remove_filter()

get_filter()

get_all_filters()

remove_all_filters()

<<readonly>> MyType : ProxyType

<<readonly>> MyAdmin : ConsumerAdmin

priority_filter : CNF::MappingFilter

lifetime_filter : CNF::MappingFilter

obtain_offered_types(mode : in ObtainInfoMode) : CN::EventTypeSeq

validate_event_qos(required_qos : in CN::QoSProperties, available_qos : out CN::NamedPropertyRangeSeq) : void

connect_typed_push_consumer(push_consumer : in CTEC::TypedPushConsumer) : void

suspend_connection() : void

resume_connection() : void

CTEC  CosTypedEventComm

CN       CosNotification

CEC     CosEventComm


Figure 7-26/X.770 – TypedProxyPush Consumer and Supplier interfaces


[image: image30.wmf]T0414720-00

TypedProxyPullConsumer

PullConsumer

(from CosNotify)

ProxyConsumer

(from CosNotifyChannelAdmin)

QoSAdmin

(from Cos)

FilterAdmin

(from CosNotifyFilter)

NotifyPublish

offer_change()

(from CosNotify)

disconnect_pull_consumer() : void

PullConsumer

ProxySupplier

(from CosNotifyChannelAdmin)

TypedProxyPullSupplier

connect_typed_pull_consumer(pull_consumer : in CEC::PullConsumer) : void

TypedPullSupplier

(from CosTypedNotify)

NotifySubscribe

subscription_change()

(from CosNotifyComm)

TypedPullSupplier

get_typed_supplier() : Object

(from CosTypedEventComm)

connect_typed_pull_supplier(pull_supplier : in CTEC::TypedPullSupplier) : void

suspend_connection() : void

resume_connection() : void

<<readonly>> MyType : ProxyType

<<readonly>> MyAdmin : SupplierAdmin

obtain_subscription_types(mode : in ObtainInfoMode) : CN::EventTypeSeq

validate_event_qos(required_qos : in CN::QoSProperties, available_qos : out CN::NamedPropertyRangeSeq) : void

get_qos()

set_qos()

validate_qos()

add_filter()

remove_filter()

get_filter()

get_all_filters()

remove_all_filters()

<<readonly>> MyType : ProxyType

<<readonly>> MyAdmin : ConsumerAdmin

priority_filter : CNF::MappingFilter

lifetime_filter : CNF::MappingFilter

obtain_offered_types(mode : in ObtainInfoMode) : CN::EventTypeSeq

validate_event_qos(required_qos : in CN::QoSProperties, available_qos : out CN::NamedPropertyRangeSeq) : void

(from CosEventComm)

CTEC  CosTypedEventComm

CN       CosNotification

CEC     CosEventComm


Figure 7-27/X.770 – TypedProxyPull Consumer and Supplier interfaces


[image: image31.wmf]T0414730-00

TypedConsumerAdmin

TypedConsumerAdmin

(from CosTypedEventChannelAdmin)

ConsumerAdmin

(from CosEventChannelAdmin)

QoSAdmin

(from Cos)

NotifySubscribe

subscription_change()

(from CosNotifyComm)

FilterAdmin

(from CosNotifyFilter)

get_qos()

set_qos()

validate_qos()

add_filter()

remove_filter()

get_filter()

get_all_filters()

remove_all_filters()

get_proxy_supplier(proxy_id : in ProxyID) : ProxySupplier

obtain_notification_pull_supplier(ctype : in ClientType, proxy_id : out ProxyID) : ProxySupplier

obtain_notification_push_supplier(ctype : in ClientType, proxy_id : out ProxyID) : ProxySupplier

destroy() : void

obtain_push_supplier() : ProxyPushSupplier

obtain_pull_supplier() : ProxyPullSupplier

obtain_typed_pull_supplier(supported_interface : in Key) : TypedProxyPullSupplier

obtain_typed_push_supplier(uses_interface : in Key) : ProxyPushSupplier

obtain_typed_notification_pull_supplier(supported_interface : in Key, proxy_id : out CNCA::ProxyID) : TypedProxyPullSupplier

obtain_typed_notification_push_supplier(uses_interface : in Key, proxy_id : out CNCA::ProxyID) : TypedProxyPushSupplier

<<readonly>> MyID : AdminID

<<readonly>> MyChannel : EventChannel

<<readonly>> MyOperator : InterFilterGroupOperator

priority_filter : CNF::MappingFilter

lifetime_filter : CNF::MappingFilter

<<readonly>> pull_suppliers : ProxyIDSeq

<<readonly>> push_suppliers : ProxyIDSeq

ConsumerAdmin

(from CosNotifyChannelAdmin)

CNCA Cos NotifyChannelAdmin

CN       CosNotification

CEC     CosEventComm


Figure 7-28/X.770 – TypedConsumerAdmin interface


[image: image32.wmf]T0414740-00

TypedSupplierAdmin

SupplierAdmin

(from CosNotifyChannelAdmin)

TypedSupplierAdmin

(from CosTypedEventChannelAdmin)

SupplierAdmin

(from CosEventChannelAdmin)

offer_change()

QoSAdmin

(from Cos)

(from CosNotifyFilter)

get_qos()

set_qos()

validate_qos()

add_filter()

remove_filter()

get_filter()

get_all_filters()

remove_all_filters()

<<readonly>> MyID : AdminID

<<readonly>> MyChannel : EventChannel

<<readonly>> MyOperator : InterFilterGroupOperator

<<readonly>> pull_consumers : ProxyIDSeq

<<readonly>> push_consumers : ProxyIDSeq

get_proxy_consumer(proxy_id : in ProxyID) : ProxyConsumer

obtain_notification_pull_consumer(ctype : in ClientType, proxy_id : out ProxyID) : ProxyConsumer

obtain_notification_push_consumer(ctype : in ClientType, proxy_id : out ProxyID) : ProxyConsumer

destroy() : void

obtain_push_consumer() : ProxyPushConsumer

obtain_pull_consumer() : ProxyPullConsumer

obtain_typed_push_consumer(supported_interface : in Key) : TypedProxyPushConsumer

obtain_typed_pull_consumer(uses_interface : in Key) : ProxyPullConsumer

obtain_typed_notification_push_consumer(supported_interface : in Key, proxy_id : out CNCA::ProxyID) : TypedProxyPushConsumer

obtain_typed_notification_pull_consumer(uses_interface : in Key, proxy_id : out CNCA::ProxyID) : TypedProxyPullConsumer

(from CosNotify)

FilterAdmin

NotifyPublish

CNCA Cos NotifyChannelAdmin

CN       CosNotification

CEC     CosEventComm


Figure 7-29/X.770 – TypedSupplierAdmin interface

	CNCA

Cos NotifyChannelAdmin
CN

CosNotification
CEC

CosEventComm

	

	TypedEventChannelFactory

	

	create_typed_channel(initial_QoS::in CN : QoSProperties, initial_admin : in CN::AdminProperties, id : out CNCA::ChannelID) : TypeEventChannel
get_all_typed_channels() : CNCA::ChannelIDSeq
get_typed_event_channel(id : in CNCA::ChannelID) : TypedEventChannel


Figure 7-30/X.770 – TypedEventChannel Factory interface


[image: image33.wmf]T0414750-00

TypedEventChannel

QoSAdmin

(from CosNotification)

AdminPropertiesAdmin

(from CosNotification)

TypedEventChannel

(from CosTypedEventChannelAdmin)

get_qos() : QoSProperties

set_qos(qos : in QoSProperties) : void

validate_qos(required_qos : in QoSProperties, available_qos : out NamedPropertyRangeSeq) : void

get_admin() : AdminProperties

set_admin(admin : in AdminProperties) : void

for_consumers() : TypedConsumerAdmin

for_suppliers() : TypedSupplierAdmin

destroy() : void

<<readonly>> MyFactory : TypedEventChannelFactory

<<readonly>> default_consumer_admin : TypedConsumerAdmin

<<readonly>> default_supplier_admin : TypedSupplierAdmin

<<readonly>> default_filter_factory : CNF::FilterFactory

new_for_typed_notification_consumers(op : in CNCA::InterFilterGroupOperator, id : out CNCA::AdminID) : TypedConsumerAdmin

new_for_typed_notification_suppliers(op : in CNCA::InterFilterGroupOperator, id : out CNCA::AdminID) : TypedSupplierAdmin

get_consumeradmin(id : in CNCA::AdminID) : TypedConsumerAdmin

get_supplieradmin(id : in CNCA::AdminID) : TypedSupplierAdmin

get_all_consumeradmins() : CNCA::AdminIDSeq

get_all_supplieradmins() : CNCA::AdminIDSeq

CNCA Cos NotifyChannelAdmin

CN       CosNotification

CEC     CosEventComm

CNF    CosNotifyFilter


Figure 7-31/X.770 – TypedEventChannel interface

8
Engineering interface definitions

This Recommendation incorporates, by reference, all of the ODP IDL Interface definitions and associated operation description and behaviour text specified in the OMG Notification Service.

The interface definitions, supporting data type definitions, and conformance requirements for this Recommendation are fully specified in OMG Notification Service.

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	











_1061103872.unknown

_1061107014.unknown

_1061107455.unknown

_1061107683.unknown

_1061114625.unknown

_1061277076.unknown

_1061277149.unknown

_1061114798.unknown

_1061115472.unknown

_1061109206.unknown

_1061114546.unknown

_1061107740.unknown

_1061107576.unknown

_1061107632.unknown

_1061107521.unknown

_1061107262.unknown

_1061107341.unknown

_1061107060.unknown

_1061104215.unknown

_1061106757.unknown

_1061106831.unknown

_1061106691.unknown

_1061104074.unknown

_1061104122.unknown

_1061104007.unknown

_1061044398.unknown

_1061044542.unknown

_1061044650.unknown

_1061044466.unknown

_1061043830.unknown

_1061043890.unknown

_1061043731.unknown

