
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	X.780

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	Amendment 1

(05/2002)   

	
	SERIES X: DATA NETWORKS AND OPEN SYSTEM COMMUNICATIONS

OSI management – Management functions and ODMA functions



	
	TMN guidelines for defining CORBA managed objects

Amendment 1: System objects and user guide for bulk attribute retrieval



	
	ITU‑T  Recommendation  X.780 (2001)  –  Amendment 1




ITU-T X-SERIES  RECOMMENDATIONS

DATA NETWORKS AND OPEN SYSTEM COMMUNICATIONS

	
	

	PUBLIC DATA NETWORKS
	

	Services and facilities
	X.1–X.19

	Interfaces
	X.20–X.49

	Transmission, signalling and switching
	X.50–X.89

	Network aspects
	X.90–X.149

	Maintenance
	X.150–X.179

	Administrative arrangements
	X.180–X.199

	OPEN SYSTEMS INTERCONNECTION
	

	Model and notation
	X.200–X.209

	Service definitions
	X.210–X.219

	Connection-mode protocol specifications
	X.220–X.229

	Connectionless-mode protocol specifications
	X.230–X.239

	PICS proformas
	X.240–X.259

	Protocol Identification
	X.260–X.269

	Security Protocols
	X.270–X.279

	Layer Managed Objects
	X.280–X.289

	Conformance testing
	X.290–X.299

	INTERWORKING BETWEEN NETWORKS
	

	General
	X.300–X.349

	Satellite data transmission systems
	X.350–X.369

	IP-based networks
	X.370–X.399

	MESSAGE HANDLING SYSTEMS
	X.400–X.499

	DIRECTORY
	X.500–X.599

	OSI NETWORKING AND SYSTEM ASPECTS
	

	Networking
	X.600–X.629

	Efficiency
	X.630–X.639

	Quality of service
	X.640–X.649

	Naming, Addressing and Registration
	X.650–X.679

	Abstract Syntax Notation One (ASN.1)
	X.680–X.699

	OSI MANAGEMENT
	

	Systems Management framework and architecture
	X.700–X.709

	Management Communication Service and Protocol
	X.710–X.719

	Structure of Management Information
	X.720–X.729

	Management functions and ODMA functions
	X.730–X.799

	SECURITY
	X.800–X.849

	OSI APPLICATIONS
	

	Commitment, Concurrency and Recovery
	X.850–X.859

	Transaction processing
	X.860–X.879

	Remote operations
	X.880–X.899

	OPEN DISTRIBUTED PROCESSING
	X.900–X.999

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation X.780

TMN guidelines for defining CORBA managed objects

Amendment 1
System objects and user guide for bulk attribute retrieval




	Summary

This amendment to ITU-T Rec. X.780 (2001) adds the definition of the System and Subsystem managed objects, and a non-normative appendix containing a user guide.


	Source

Amendment 1 to ITU‑T Recommendation X.780 (2001) was prepared by ITU‑T Study Group 4 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 29 May 2002.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS


Page
1)
Clause 2.1

1
2)
New clauses 5.8 and 5.9

1
5.8
System Managed Object

1
5.8.1
Notifications On The System Managed Object

1
5.9
Subsystem Managed Object

2
3)
Annex A

2
4)
New Appendix II

6
ITU-T Recommendation X.780

TMN guidelines for defining CORBA managed objects

Amendment 1
System Objects and user guide for bulk attribute retrieval

1)
Clause 2.1

Add the following new reference to clause 2.1:

[8]
ITU-T Recommendation X.720 (1992) | ISO/IEC 10165:1993, Information technology – Open Systems Interconnection – Structure of management information: Management infor​mation model.

2)
New clauses 5.8 and 5.9

Add the following new clauses after clause 5.7:

5.8
System managed object

In addition to the top-most ManagedObject class, the IDL also contains a couple of managed object definitions that specialize ManagedObject. The System managed object class is used to represent a set of hardware and software that forms an autonomous whole capable of performing information processing and/or information transfer. The entire specification of the sequence of name bindings to be used in constructing the distinguished name for a System managed object is outside the scope of this Recommendation. Examples of names for systems are specified in ITU-T Rec. X.720 [8].

An instance of this managed object class may be used as the superior in naming managed objects representing either information processing and or information transfer resources contained within this instance.

Note that the managed object defined here does not have the ITU-T Rec. X.721 [6] Supported Features attribute since the CORBA interface does not define negotiable Functional Units.

5.8.1
Notifications on the system managed object

The System managed object has the following notifications:

Table n/X.780 – System notifications

	Notification
	Conditional Package (if Conditional)

	Object Creation
	"itut_x780::createDeleteNotificationsPackage"

	Object Deletion
	"itut_x780::createDeleteNotificationsPackage"

	State Change
	"itut_x780::stateChangeNotificationPackage"


Changes in the following states (when defined) will cause State Change notifications (when supported) to be emitted:

–
Administrative State.

–
Operational State.

–
Usage State.

5.9
Subsystem managed Object

The Subsystem managed object is a subclass of the System managed object class (see 5.8) and is contained by a System or another Subsystem managed object class instance. The Subsystem managed object class may be used as a common containment point for managed objects in a system that relate to the operation of a given layer. The choice of structuring within a system is dependent on what structure the system designer wishes to present externally for management purposes.

This managed object class represents a portion of a system where components are named independently of the components of other subsystems.

3)
Annex A

Add the following to the IDL in Annex A.

After the last exception in the "EXCEPTIONS" portion of the IDL, which is shown in this line:


exception InvalidString {};

Add these lines:


const string administrativeStatePackage =




"itut_x744d1::administrativeStatePackage";


const string createDeleteNotificationsPackage =




"itut_x744d1::createDeleteNotificationsPackage";


const string stateChangeNotificationPackage =




"itut_x744d1::stateChangeNotificationPackage";


exception NOadministrativeStatePackage {};

After the MANAGED OBJECT FACTORY INTERFACE definition, which ends with this line:


}; // end of ManagedObjectFactory interface

Add the following lines:

// SYSTEM INTERFACE


/** This valuetype is used to retrieve multiple attributes.
*/


valuetype SystemValueType : truncatable ManagedObjectValueType {




public OperationalStateType operationalState;






// GET




public UsageStateType usageState;






// GET




public AdministrativeStateType administrativeState;






// GET-REPLACE






// administrativeStatePackage


}; // valuetype SystemValueType


/**  The System managed object class is used to represent a set of hardware and software that forms an autonomous whole capable of performing information processing and/or information transfer.


The entire specification of the sequence of name bindings to be used in 


constructing the distinguished name for a System managed object is outside the scope of this Recommendation. Name bindings to the Recommendation M.3120 Managed Element managed object class is supplied. Examples of names for systems are specified in Recommendation X.720.


NOTE - This definition does not correspond to real open system but corresponds to real systems in Recommendation X.200.


An instance of this managed object class may be used as the superior in naming managed objects representing either information processing and or information transfer resources contained within this instance.


Note that this does not have the Supported Features attribute defined in


Recommendation X.721, since the CORBA interface does not define negotiable


Functional Units.


*/


interface System : ManagedObject


{



/**



Operational State, Usage State and Administrative State are described in 

Recommendation X.731



*/



OperationalStateType operationalStateGet ()






raises (ApplicationError);



UsageStateType usageStateGet ()






raises (ApplicationError);



/**



PRESENT IF an instance supports it.



*/



AdministrativeStateType administrativeStateGet ()






raises (ApplicationError,









NOadministrativeStatePackage);



void administrativeStateSet






in AdministrativeStateType administrativeState)






raises (ApplicationError,









NOadministrativeStatePackage);



CONDITIONAL_NOTIFICATION(






Notifications, objectCreation,






createDeleteNotificationsPackage)



CONDITIONAL_NOTIFICATION(






Notifications, objectDeletion,






createDeleteNotificationsPackage)



CONDITIONAL_NOTIFICATION(






Notifications, stateChange,






stateChangeNotificationPackage)


}; // interface System

// SYSTEM FACTORY INTERFACE


/**


Factory for System


*/


interface SystemFactory : ManagedObjectFactory


{




ManagedObject create






(in NameBindingType nameBinding,






in MONameType superior,






in string reqID,
// auto naming if empty string






out MONameType name,






in AdministrativeStateType administrativeState









// GET-REPLACE









// administrativeStatePackage






)






raises (ApplicationError,









CreateError);


}; // interface SystemFactory

// SUBSYSTEM INTERFACE


/**  This valuetype is used to retrieve multiple attributes. */


valuetype SubsystemValueType : truncatable SystemValueType {


}; // valuetype SubsystemValueType


/**


The Subsystem managed object class may be used as a common containment point for managed objects in a system that relate to the operation of a given layer. The choice of structuring within a system is dependent on what structure the system designer wishes to present externally for management purposes.


This managed object class represents a portion of a system where components are named independently of the components of other subsystems.


*/


interface Subsystem : System


{


}; // interface Subsystem

// SUBSYSTEM FACTORY INTERFACE


/**


Factory for Subsystem


*/


interface SubsystemFactory : ManagedObjectFactory


{




ManagedObject create






(in NameBindingType nameBinding,






in MONameType superior,






in string reqID,
// auto naming if empty string






out MONameType name,






in AdministrativeStateType administrativeState









// GET-REPLACE









// administrativeStatePackage






)






raises (ApplicationError,









CreateError);


}; // interface SubsystemFactory

After the definition of the NOTIFICATIONS INTERFACE, which ends with this line:

  }; // end of Notifications interface

Add the following lines:

// NAME BINDINGS


/**


This name binding is used to name the System object 


relative to the local root. That is, enable it to be


the top-most managed object on a system.


*/


module System


{





const string superiorClass = "";





const boolean superiorSubclassesAllowed = FALSE;





const string subordinateClass = "itut_x780::System";





const boolean subordinateSubclassesAllowed = TRUE;





const boolean managerCreatesAllowed = FALSE;





const DeletePolicyType deletePolicy =







itut_x780::deleteOnlyIfNoContainedObjects;





const string kind = "System";


}; // module System


/**


This name binding is used to name the Subsystem object 


relative to a Subsystem object.


*/


module Subsystem_Subsystem


{





const string superiorClass = "itut_x780::Subsystem";





const boolean superiorSubclassesAllowed = TRUE;





const string subordinateClass = "itut_x780::Subsystem";





const boolean subordinateSubclassesAllowed = TRUE;





const boolean managerCreatesAllowed = TRUE;





const DeletePolicyType deletePolicy =







itut_x780::deleteOnlyIfNoContainedObjects;





const string kind = "Subsystem";


}; // module Subsystem_System


/**


This name binding is used to name the Subsystem object 


relative to a System object.


*/


module Subsystem_System


{





const string superiorClass = "itut_x780::System";





const boolean superiorSubclassesAllowed = TRUE;





const string subordinateClass = "itut_x780::Subsystem";





const boolean subordinateSubclassesAllowed = TRUE;





const boolean managerCreatesAllowed = TRUE;





const DeletePolicyType deletePolicy =







itut_x780::deleteOnlyIfNoContainedObjects;





const string kind = "Subsystem";


}; // module Subsystem_System

4)
New Appendix II

Add the following new non-normative appendix:

Appendix II
User guide for bulk attribute retrieval

This appendix provides additional information about the TMN CORBA framework intended to help those implementing systems that conform to the framework Recommendations.

Bulk retrieval of attributes

The top-most managed object interface, ManagedObject, defines an operation that enables a managing system to retrieve multiple attributes from a managed object in one operation. The signature of this operation, AttributesGet, is shown below:


ManagedObjectValueType attributesGet (





in
NameType name,





inout
StringSetType attributeNames)


raises (ApplicationError);

Note that the names of the attributes requested by the managing system are submitted in the attributeNames parameter, and the names of the attributes actually returned by the managed object are also returned in the attributeNames parameter. Because the list of attributes returned by the managed object may differ from the requested list, the in/out parameter attributeNames may be changed by the managed object. Managing system implementations that wish to repeatedly use the same list of attributes will not want that list modified by the managed object. One solution to this problem is made possible by the way in which CORBA ORBs exchange messages to remotely invoke methods on objects and return results. Operation parameters are passed by order, not by name. Thus, a single inout parameter can be replaced with separate in and out parameters without impacting the inter-operability of systems. This is because the in parameter will take the same place in the invocation message as would the inout parameter, and the out parameter will take the same place in the results message as would the inout parameter. As long as the order of the parameters is not changed, the ORBs will correctly match the parameters in the messages to the parameters on the method invocations. So, the developer of an implementation using the ManagedObject interface may modify the IDL by replacing the attributesGet signature above with the one below.


ManagedObjectValueType attributesGet (





in
NameType name,





in
StringSetType requestedAttributeNames,





out
StringSetType returnedAttributeNames)


raises (ApplicationError);

This will prevent the client system's list of requested attribute names from being overwritten and still be interoperable with managed object implementations using the first operation signature, above.

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Cable networks and transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	


