
	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	Y.1231

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(11/2000)   

	
	SERIES Y: GLOBAL INFORMATION INFRASTRUCTURE AND INTERNET PROTOCOL ASPECTS

Internet protocol aspects – Architecture, access, network capabilities and resource management



	
	IP access network architecture

	
	ITU‑T  Recommendation  Y.1231

(Formerly  CCITT  Recommendation)


ITU-T Y-SERIES  RECOMMENDATIONS

GLOBAL INFORMATION INFRASTRUCTURE AND INTERNET PROTOCOL ASPECTS

	
	

	GLOBAL INFORMATION INFRASTRUCTURE
	

	General
	Y.100–Y.199

	Services, applications and middleware
	Y.200–Y.299

	Network aspects
	Y.300–Y.399

	Interfaces and protocols
	Y.400–Y.499

	Numbering, addressing and naming
	Y.500–Y.599

	Operation, administration and maintenance
	Y.600–Y.699

	Security
	Y.700–Y.799

	Performances
	Y.800–Y.899

	INTERNET PROTOCOL ASPECTS
	

	General
	Y.1000–Y.1099

	Services and applications
	Y.1100–Y.1199

	Architecture, access, network capabilities and resource management
	Y.1200–Y.1299

	Transport
	Y.1300–Y.1399

	Interworking
	Y.1400–Y.1499

	Quality of service and network performance
	Y.1500–Y.1599

	Signalling
	Y.1600–Y.1699

	Operation, administration and maintenance
	Y.1700–Y.1799

	Charging
	Y.1800–Y.1899

	
	


For further details, please refer to the list of ITU-T Recommendations.

	ITU-T Recommendation Y.1231

IP access network architecture




	Summary

This Recommendation provides the definitions of terminology regarding IP access, and the high‑level IP network architecture and models for the IP services. It describes the access types and interfaces to be supported by the IP access network, the IP access network capabilities and requirements, and the IP access network functional models and possible arrangements.


	Source

ITU‑T Recommendation Y.1231 was prepared by ITU‑T Study Group 13 (2001‑2004) and approved under the WTSA Resolution 1 procedure on 24 November 2000.


	


FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of tele​com​mu​ni​ca​tions. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU‑T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS
ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

(  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS


Page

1
Scope

1
2
References

1
2.1
Normative references

1
2.2
Informative references

2
3
Definitions and abbreviations

2
3.1
Definitions

2
3.2
Abbreviations

2
4
Architectural model of IP access networks

4
4.1
General network architecture of IP network

4
4.2
IP access network reference model

5
5
Examples of functional models of IP Access Network

6
5.1
Use of PPP

6
5.1.1
PPP tunnelling aggregation

7
5.1.2
PPP terminated aggregation

8
5.2
Use of Ethernet

9
6
Examples of access types and interfaces in IP access network

10
7
Examples of functional requirements for IP Access

10
Appendix I – Examples of IP mapping mechanisms

10
I.1
IP over PPP over ATM

10
I.2
IP over ATM

11
I.3
IP over Frame Relay

11
I.4
IP over PPP over PHY

11
I.5
IP over Ethernet

12
I.6
IP over PPP over Ethernet

12
Appendix II – An example of functional model of CATV access

13
ITU-T Recommendation Y.1231

IP access network architecture

1
Scope

This Recommendation defines the functional IP Access Network architecture and the functions and requirements above the transport access functions defined in ITU-T Y.1001. The functional requirements for the Access Network Transport are defined for the handling and transport of digital bearer signals defined in ITU-T G.902.

This Recommendation describes:

–
the definitions of terminology regarding IP access;

–
the high-level IP network architecture and models for the IP services;

–
the access types and interfaces to be supported by the IP access network;

–
the IP access network capabilities and requirements; and

–
the IP access network functional models and possible arrangements.

The functional view of IP access network is independent of the access network transport functions described in ITU-T Y.1001.

2
References

The following ITU-T Recommendations and other references contain provisions which, through references in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. 
NOTE – A reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation. 
2.1
Normative references
[1]
ITU-T Q.922 (1992), ISDN data link layer specification for frame mode bearer services.

[2]
ITU-T I.363.5 (1996), B-ISDN ATM Adaptation Layer specification: Type 5 AAL.

[3]
ITU-T I.414 (1997), Overview of Recommendations on Layer 1 for ISDN and B‑ISDN customer accesses.

[4]
ITU-T G.902 (1995), Framework Recommendation on functional Access Networks (AN) ( Architecture and functions, access types, management and service node aspects.
[5]
ITU-T Y.1001 (2000), IP framework – A framework of convergence of telecommunications network and IP network technologies.

[6]
ITU-T Y.1401 (2000), General requirements for interworking with Internet Protocol (IP)-based networks.

[7]
ISO/IEC 8802-2:1998, Information technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements – Part 2: Logical link control.

2.2
Informative references

[8]
IEEE Std 802.2 (1998), Logical link control (802.2).

[9]
IEEE Std 802.3 (1998), CSMA/CD access method (802.3).

[10]
Data-Over-Cable Service Interface Specifications, Cable Modem to Customer Premise Equipment Interface Specification, SP-CMCI-I03-991115, Cable Television Laboratories, Inc.

[11]
IETF RFC 894 (1984), A Standard for the Transmission of IP Datagrams over Ethernet Networks.

[12]
IETF RFC 1042 (1988), A Standard for the Transmission of IP Datagrams over IEEE 802 Networks.

[13]
IETF RFC 1334 (1992), Password Authentication Protocols.

[14]
IETF RFC 2684 (1999), Multiprotocol Encapsulation over ATM Adaptation Layer 5.

[15]
IETF RFC 1661 (1994), The Point-to-Point Protocol (PPP).

[16]
IETF RFC 1662 (1994), PPP in HDLC-like Framing.

[17]
IETF RFC 1994 (1996), PPP Challenge Handshake Authentication Protocol (CHAP).

[18]
IETF RFC 2138 (1997), Remote Authentication Dial In User Services (RADIUS).

[19]
IETF RFC 2225 (1998), Classical IP and ARP over ATM.

[20]
IETF RFC 2364 (1998), PPP over AAL5.

[21]
IETF RFC 2427 (1998), Multiprotocol Interconnect over Frame Relay.

[22]
IETF RFC 2661 (1999), Layer Two Tunnelling Protocol (L2TP).

[23]
IETF RFC 2516 (1999), A Method for Transmitting PPP Over Ethernet (PPPoE).

3
Definitions and abbreviations

3.1
Definitions

This Recommendation defines the following terms:

3.1.1
IP access network: An implementation comprising network entities to provide the required access capabilities between an "IP user" and an "IP service provider" for the provision of IP services. "IP user" and "IP service provider" are logical entities which terminate the IP layer and/or IP related functions, and may also include lower layer functions, and may also include lower layer functions.
3.1.2
IP core network: IP service provider's network, including one or more IP service providers. 

3.2
Abbreviations

This Recommendation uses the following abbreviations:

AAA
Authentication, Authorization and Accounting 

AF
Access Function

ARP
Address Resolution Protocol

ATM
Asynchronous Transfer Mode

CHAP
Challenge Handshake Authentication Protocol

CMCI
Cable Modem to CPE Interface

CMTS
Cable Modem Termination System

CPE
Customer Premises Equipment

CPN
Customer Premises Network

DHCP
Dynamic Host Configuration Protocol

DIX
Digital Intel Xerox

DOCSIS
Data-Over-Cable Service Interface Specifications

EF
Edge Function

FR
Frame Relay

HFC
Hybrid Fibre and Coax

IP
Internet Protocol

IPSEC
Internet Protocol Security

ISDN
Integrated Services Digital Network 

IWF
InterWorking Function

LAN
Local Area Network

LLC
Logical Link Control

L2TP
Layer 2 Tunnelling Protocol

MAC
Medium Access Control

MPLS
Multiprotocol Label Switching

NAT
Network Address Translation

NSI
Network Side Interface

NT 
Network Termination

PAP
Password Authentication Protocol

PC
Personal Computer

PCI
Peripheral Component Interface

PHY
Physical Layer

PON
Passive Optical Network

PPP
Point-to-Point Protocol

PSTN
Public Switched Telephone Network

RADIUS
Remote Access Dial In User Services

RFI
Radio Frequency Interface

RP
Reference Point

SNMP
Simple Network Management Protocol

TE
Terminal Equipment

TEL
Telephone

TFTP
Trivial File Transfer Protocol

UDP
User Datagram Protocol

VCI
Virtual Channel Identifier

VPI
Virtual Path Identifier

WAN
Wide Area Network

4
Architectural model of IP access networks

4.1
General network architecture of IP network

Figure 1a shows a general network architecture of IP network. In Figure 1a, the lines between various rectangles and ellipses represent connections that are bidirectional, that may be asymmetrical in bit rate, and that may be of differing media in the two directions. The reference points (RP) illustrated are logical separation between the functions and may not correspond to physical interfaces in certain network implementations. In certain network implementations, access and core networks may not be separable.


[image: image2.wmf]T1317460-00

TE

TE

PC

TE

TE

PC

TEL

RP

RP

RP

RP

CPN

CPN

TEL

IP Access

Network

IP Core

Network

IP Access

Network


Figure 1a/Y.1231 ( General network architecture of IP network

Figure 1b shows an example of relationship between IP network and PSTN/ISDN. In Figure 1b, some connections and InterWorking Function (IWF) between IP Access/Core network and PSTN/ISDN are shown as an example, but not all PSTN/ISDN connections may be necessary. A definition of IWF is described in ITU-T Y.1401 [6].


[image: image3.wmf]T1317470-00

TE

TE

PC

TE

TE

PC

TEL

RP

RP

RP

RP

CPN

CPN

TEL

IWF

TE

TE

PC

TEL

CPN

IWF

IWF

NT

IP Access

Network

IP Core

Network

IP Access

Network

PSTN/ISDN


Figure 1b/Y.1231 ( An example of relationship between IP network and PSTN/ISDN

4.2
IP access network reference model

Figure 2 shows an example of IP access network reference model. In some cases, the IP access Function (IP-AF) can be distributed within the IP Access Network. Examples of the IP-AF are described in clause 7. 

The functional requirements for the Access Network Transport are defined for the handling and transport of digital bearer signals defined in ITU-T G.902 [4].

The functional view of IP access network is independent of the access network transport functions described in ITU-T Y.1001 [5].


[image: image4.wmf]T1317480-00

RP

RP

CPN

RP

Access Network Transport Function

IP Access Function

IP Access Network System Management

for management interface

IP Access Network

IP Core Network

IP Service 

Provider 3

IP Service 

Provider 2

IP Service 

Provider 1


Figure 2/Y.1231 ( IP Access Network architecture example

5
Examples of functional models of IP Access Network

5.1
Use of PPP

In this clause, PPP is shown as an example for protocol used to carry IP over access network. PPP is a widely used protocol for dial-up IP access and ADSL access. PPP termination point in the network side interacts with AAA (Authentication, Authorization and Accounting) server (e.g. RADIUS server) and provides AAA functions. 

Figures 3a and 3b provide the following two options regarding PPP termination point in the network side: 

1)
PPP tunnelling aggregation (PPP is terminated by IP service provider); and 

2)
PPP terminated aggregation (PPP is terminated by the IP access function).


[image: image5.wmf]T1317490-00

IP User

IP Access Function

IP Service Provider

PPP

Termination

Function

PPP

Termination

Function

AAA

Client

Function

AAA

Server

Function


Figure 3a/Y.1231 ( PPP tunnelling aggregation


[image: image6.wmf]T1317500-00

IP User

IP Access Function

IP Service Provider

PPP

PPP

 AAA

Client

 AAA

Server


Figure 3b/Y.1231 ( PPP terminated aggregation

5.1.1
PPP tunnelling aggregation

This clause gives some examples of protocol stacks for the PPP tunnelling aggregation. In this case, "IP user" directly accesses the "IP service provider". Several transport systems can be cascaded without IP and PPP processing in the access network.

Case 1)

Layer 2 multiplexing in IP access network (e.g. direct IP access)

This is a case that only Layer 2 multiplexing transport function such as ATM or FR are supported in IP access network. This functional model is shown in Figure 4. 


[image: image7.wmf]T1317510-00

 PHY

PHY

PHY

PHY

IP Access Function

IP  Service Provider

IP

IP

PPP

PPP

IP 

User

 

ATM,

FR 

etc.

 

ATM, 

FR

etc.


Figure 4/Y.1231 ( Functional model of Layer 2 multiplexing

Case 2)

PPP tunnelling (e.g. L2TP [22])

This is a case that L2TP function is used in IP access network. This functional model is shown in Figure 5.


[image: image8.wmf]T1317520-00

PHY

PHY

                L2TP

PHY

L2TP

PHY

IP Access Function

IP

IP

PPP

PPP

IP 

User

IP  Service Provider

IP

(Note)

ATM

, FR

etc.

ATM

, FR

etc.

IP

(Note)

NOTE – This IP layer provides a transport function between IP access function and IP service provider.


Figure 5/Y.1231 ( Functional model of PPP tunnelling

5.1.2
PPP terminated aggregation

This clause gives some examples of protocol stacks for the PPP terminated aggregation. In this case, PPP is terminated in IP access network.

Case 1)

IP tunnelling (e.g. IPSEC)

This is a case that IP tunnelling is used in IP access network. This functional model is shown in Figure 6.


[image: image9.wmf]T1317530-00

PHY

IPSEC

PHY

IP Access Function

IP

IP

IP 

User

IP  Service Provider

IP

(Note)

ATM

, FR

etc.

ATM

, FR

etc.

IP

(Note)

IPSEC

PPP

PPP

PHY

PHY

NOTE – This IP layer provides a transport function between IP access function and IP service provider.


Figure 6/Y.1231 ( Functional model of IP tunnelling

Case 2-1)
Layer 3 routing (e.g. IP router)

This is a case that IP handling function such as IP router exists in IP access network. This functional model is shown in Figure 7.


[image: image10.wmf]T1317540-00

PHY

PHY

IP Access Function

IP

IP

IP 

User

IP  Service Provider

ATM

, FR

etc.

ATM

, FR

etc.

PPP

PPP

PHY

PHY

IP

IP


Figure 7/Y.1231 ( Functional model of L3 routing

Case 2-2)
Virtual router 

This is also a case that IP handling function exists in IP access network. This functional model is shown in Figure 8.


[image: image11.wmf]T1317550-00

PHY

PHY

IP Access Function

IP

IP

IP 

User

IP  Service Provider

ATM

, FR

etc.

ATM

, FR

etc.

PPP

PPP

PHY

PHY

IP

IP

 

 

 

Multiple instances of IP routing tables for each

virtual router


Figure 8/Y.1231 ( Functional model of virtual router

Case 3)

MPLS (multiprotocol label switching)

This is a case that IP transport function is translated by the lower layer such as ATM in IP access network. This functional model is shown in Figure 9. There is a case that "Label" stack is null in the user-plane, when the ATM VPI/VCIs are used as MPLS labels.


[image: image12.wmf]T1317560-00

PHY

Label

PHY

IP Access Function

IP

IP

IP 

User

IP  Service Provider

ATM

, SDH

etc.

ATM

, SDH

etc.

PPP

PPP

PHY

PHY

Label


Figure 9/Y.1231 ( Functional model of MPLS

5.2
Use of Ethernet

In this clause, Ethernet is an example of protocol used to carry IP over access network. Figure 10 shows examples of protocol stacks of Ethernet access.


[image: image13.wmf]T1317570-00

PHY

PHY

IP Access Function

IP

IP

IP 

User

IP  Service Provider

ATM

, FR

etc.

ATM

, FR

etc.

Ethernet

PHY

PHY

IP

IP

 

 

 

Ethernet


Figure 10/Y.1231 ( Functional model of Ethernet use

6
Examples of access types and interfaces in IP access network

Possible transmission mechanisms for user access are shown as follows:
(
ISDN:

•
Basic rate access (B/2B/D channel);

•
Primary rate (1544 kbit/s, 2048 kbit/s).
–
B-ISDN access (1544 kbit/s-622 080 kbit/s).

–
xDSL.

–
Wireless and Satellite.

–
PON, SDV, HFC and other optical systems.

–
CATV access.

–
LAN/WAN.

Examples of IP mapping mechanisms on each transmission system are shown in Appendix I.

An example of functional architecture of CATV access is shown in Appendix II.

Further examples and overview are described in ITU-T I.414 [3].

7
Examples of functional requirements for IP Access

Possible IP access functions are as follows:

–
Dynamic selection of multiple IP service providers.

–
Dynamic allocation of IP address using PPP.

–
NAT.

–
Authentication (PAP [13], CHAP [17]).

–
Encryption.

–
Billing usage metering and interaction with RADIUS [18] server.

APPENDIX I

Examples of IP mapping mechanisms

The following diagrams show protocol stacks for IP on various transmission systems. The detailed mapping mechanisms are described in the referenced documents. 

I.1
IP over PPP over ATM

Figure I.1 shows IP over PPP over ATM mapping mechanism which is defined in IETF RFC 2364, "PPP over AAL5" [20].

	IP

	PPP
(IETF RFC 1661/IETF RFC 2364)

	Encapsulation Header
(IETF RFC 2684)

	AAL5
(ITU-T I.363.5)

	ATM

	PHY


Figure I.1/Y.1231 ( Mapping mechanism for IP
over PPP over ATM 

I.2
IP over ATM

Figure I.2 shows IP over ATM mapping mechanism which is defined in IETF RFC 2225, "Classical IP and ARP over ATM" [19].

	IP
(IETF RFC 2225)

	Encapsulation Header
(IETF RFC 2684)

	AAL5
(ITU-T I.363.5)

	ATM

	PHY


Figure I.2/Y.1231 ( Mapping mechanism
for IP over ATM

I.3
IP over Frame Relay

Figure I.3 shows IP over FR mapping mechanism which is defined in IETF RFC 2427, "Multiprotocol Interconnect over Frame Relay" [21].

	IP

	Encapsulation Header
(IETF RFC 2427)

	LAPF
(ITU-T Q.922)

	PHY


Figure I.3/Y.1231 ( Mapping mechanism
for IP over FR 

I.4
IP over PPP over PHY

Figure I.4 shows PPP directly mapping to physical layer (e.g. ISDN, SDH).

	IP

	PPP
(IETF RFC 1661/IETF RFC 1662)

	PHY


Figure I.4/Y.1231 ( Mapping mechanism
for IP over PPP
NOTE – Other mapping mechanisms may be used, for example to support the requirements of radio systems such as satellite systems.

I.5
IP over Ethernet

Two types of Ethernet are commonly used. Two types of IP over Ethernet mapping mechanism are defined in separate IETF documents (IETF RFC 894 [11] and IETF RFC 1042 [12]).  

Figure I.5 shows IP mapping mechanism using Ethernet (IEEE 802.3 [9]/ISO|IEC 8802-2 [7]) and Figure I.6 shows IP mapping mechanism using Ethernet (IEEE 802.3 [9]/ISO|IEC 8802-2 [7]) with LLC.
	IP
(IETF RFC 894)

	MAC
(IEEE 802.3/ISO|IEC 8802-2)

	PHY


Figure I.5/Y.1231 ( Mapping mechanism for IP
over Ethernet (IEEE 802.3/ISO|IEC 8802-2)
	IP
(IETF RFC 1042)

	LLL
(IEEE 802.2)

	MAC
(IEEE 802.3/ISO|IEC 8802-2)

	PHY


Figure I.6/Y.1231 ( Mapping mechanism for IP over
Ethernet (IEEE 802.3/ISO|IEC 8802-2) with LLC

I.6
IP over PPP over Ethernet

Figure I.7 shows IP over PPP over Ethernet mapping mechanism which is defined in IETF document (IETF RFC 2516 [23]).

	IP

	PPP 
(IETF RFC 1661/IETF RFC 1662)

	Encapsulation Head
(IETF RFC 2516)

	MAC
(IEEE 802.3/ISO|IEC 8802-2)

	PHY


Figure I.7/Y.1231 ( Mapping mechanism for
IP over PPP over Ethernet

APPENDIX II

An example of functional model of CATV access

See Figure II.1.


[image: image14.wmf]T1318100-01

SNMP

TFTP

DHCP

UDP

IP, ARP

SNMP

TFTP

DHCP

UDP

UDP

CMTS

PCI CMCI

RFI

IP, ARP

IP, ARP

PCI

PHY

DOCSIS

PHY

DOCSIS

PHY

DOCSIS

MAC

DOCSIS

MAC

DIX/

802.3

MAC

DIX/

802.3

MAC

NSI

PHY

DIX/

802.2

LLC

DIX/

802.2

LLC

DIX/

802.3

MAC

DIX/

802.2

LLC

PCI

PHY

NSI

Data

Link

Forwarder

Internet

PCI Cable Modem

Host CPE

Optional Router

Security 

Magmt

Security 

Magmt

Internet Applications, 

services, 

 and agents 

DOCSIS

con-

vergence

DOCSIS

con-

vergence

DOCSIS

Security

DOCSIS

Security

DIX/802.3 MAC

filter

NOTE – This figure is extracted from Cable Television Laboratores, Inc.'s specification – Data-Over-Cable

Service Interface Specifications (SP-CMCI-I03-991115 [10]) in order to give an example of Ethernet

use to carry IP over the access network.


Figure II.1/Y.1231 ( Functional model of CATV access

	SERIES OF ITU-T RECOMMENDATIONS

	Series A
	Organization of the work of ITU-T

	Series B
	Means of expression: definitions, symbols, classification

	Series C
	General telecommunication statistics

	Series D
	General tariff principles

	Series E
	Overall network operation, telephone service, service operation and human factors

	Series F
	Non-telephone telecommunication services

	Series G
	Transmission systems and media, digital systems and networks

	Series H
	Audiovisual and multimedia systems

	Series I
	Integrated services digital network

	Series J
	Transmission of television, sound programme and other multimedia signals

	Series K
	Protection against interference

	Series L
	Construction, installation and protection of cables and other elements of outside plant

	Series M
	TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

	Series N
	Maintenance: international sound programme and television transmission circuits

	Series O
	Specifications of measuring equipment

	Series P
	Telephone transmission quality, telephone installations, local line networks

	Series Q
	Switching and signalling

	Series R
	Telegraph transmission

	Series S
	Telegraph services terminal equipment

	Series T
	Terminals for telematic services

	Series U
	Telegraph switching

	Series V
	Data communication over the telephone network

	Series X
	Data networks and open system communications

	Series Y
	Global information infrastructure and Internet protocol aspects

	Series Z
	Languages and general software aspects for telecommunication systems

	
	











_1054370960.unknown

_1054371295.unknown

_1054385075.unknown

_1054385101.unknown

_1054387226.unknown

_1054371365.unknown

_1054371994.unknown

_1054371076.unknown

_1054371234.unknown

_1054371014.unknown

_1054370799.unknown

_1054370878.unknown

_1054370735.unknown

